Perface

Thanks for using our product. we'll do our best to provide the best service for you. This handbook maybe contains technology mistakes or text errors. The content will be updated regularly without prior notice and the new updated part will be added in the new version.

Client SDK Instructions

Summary

Client SDK is a matching product of embedded hard disk video recorder(DVR), network hard disk video recorder(NVR), video server(DVS) and IPCamera, mostly used in remote device access and remote control software development.All functions below is mainly for DVR and NVR, when you integrate IPC with SDK ,just take IPC as one channel DVR device, and IPC SDK instruction page lists which functions are supported for IPC integration. The version of this product is 1.0,and its main function as the list below:

Function Module

Device basic info/Parameters config

Living preview

Function Details

device serial number,firmware version number,firmware version compiling date,core version number,hardware version number.

video formats,device ID,device name

device interface language,start password protection,screen protection,VGA parameters setting

local preview,remote preview,channel hidden,picture segmentation,picture polling

channel name,record state,alarm state(sensor alarm,network address conflict,harddisk error,disk full,system time)

electronic zoom in,picture in picture

living sound,volume adjust,mute control

Preview video/Parameterscolor adjust,adapt color to different setting	time slot

Video & Audio record/Parameters settings

Capture/Parameters setting

Alarm handle/Parameters setting

character overlay:channel name,timestamp,user-defined information

area keep out

video&audio record/parameters settings

record of alarming:record time before alarming,record time after alarming,different encoding parameters setting

record switch:video switch,audio switch,video & audio binding relationship

data expiration,redundancy record,group record,whether circulation cover

record plan:record on time, record when sensor alarming,record when motion detection,record when video kept out alarming

manual record,remote control of manual record

picture measurement,picture quality,capture interval, capture numbers

manual capture,picture retrieval,picture diaplay

alarm type:sensor alarm,motion detection alarm,video kept out alarm,video lost alarm,smart analytics alarm

detection plant, detection time-lag,sensor device type,sensor device name, motion detection area & sensitivity

Past video

types

Alarm output/Parameters setting

alarm output:relay alarm,buzzer alarm,large picture caution,send emails,send up to center

alarm record:trigger record(specify channel),record log(alarm starts information,alarm ends information)

action with alarm:PTZ preset positions,PTZ cruise line, PTZ track

alarm from email:channel information,alarm type,attached picture(specify channel,picture numbers & time interval)

alarm from relay:switch,response schedule,response time-lag,alarm name

alarm from buzzer:switch,response schedule,response time-lag

manual alarm,remotely manual alarm

search by time:data distribution retrieval/Playback/Backupcharter, distinguish different record

search by event:event list, filter event type

search by file :file list(no partitioning record segment),lock/delete file

search by picture:picture diaplaydisplay, locked/delete pictures, picture backup(save as)

start playback:specify the start time(specify a group of channels) specify event,specify file

playback control:stop ,speed,fast backward,single frame play,reposition,exist playback,store

Network/Parameters setting

Mail functionary/Config

capture(harddisk),select area for backup

manual backup:backup by start time & stop time,backup by the specified file,montage backup,DVR & AVI formats,combination backup(multi-channels backup in one file)

automatically backup:specify time and condition,backup in external memory,backup in network service

network address setting:static address,dynamic address,PPPoE

ports-settings:HTTP port,data port,alarm port,etc.

multicast address setting

DDNS setting:customization demand,send up the period setting,etc.

parameters setting of network substream encoding:resolution,frame rate,encoded mode,picture quality,code stream limit,whether self-adaption(picture quality & fluency)

network linking setting:register user number limit,video channel number limit,whether release mainstream,black and white list

check network state,prompt network state live(normal connection,no connection,conflict),check users online,push-off users online

send emails when alarm,combine emails in a while,emails whether

PTZ control/Parameters setting

Configuration management

Disc management/Health check

System maintain

Other function

with attachment,specify to send emails, send emails manually

parameters setting of serial port ,preset position setting,cruise line setting,track setting

PTZ control:eight directions,stop,aperture,focus,zoom in,rate(128),lamplight,windshield wiper,automatically scan line

control mode:mouse 3D control,mouse control through dialog box,front panel,remote-controller,professional keyboard,remote control

protocol

local configure,remote configuration,configuration import & export,recover default configuration

format disc,delete data,specify disc group,set disc attribute(read only, read-write,redundancy,backup)

firmware upgrade,device health checked, remote upgrade,FTP upgrade

user management,permission setting

log record/retrieval/export

FTP setting

Client SDK Instructions

IPC SDK instruction

All supported functions for IPC:live preview, capture, sensor alarm, motion alarm, config import and export, getting or setting config, getting device time.

supported config for IPC:

DD_CONFIG_ITEM_DEVICE_INFO DD_CONFIG_ITEM_SYSTEM_BASIC DD_CONFIG_ITEM_DATE_TIME DD_CONFIG_ITEM_DAYLIGHT_INFO DD_CONFIG_ITEM_NETWORK_IP DD_CONFIG_ITEM_NETWORK_ADVANCE DD_CONFIG_ITEM_DDNS_SERVER_INFO DD_CONFIG_ITEM_ACCOUNT DD_CONFIG_ITEM_SENSOR_SETUP DD_CONFIG_ITEM_SENSOR_SCHEDULE DD_CONFIG_ITEM_SENSOR_ALARM_OUT DD_CONFIG_ITEM_SENSOR_TO_RECORD DD_CONFIG_ITEM_MOTION_SETUP DD_CONFIG_ITEM_MOTION_SCHEDULE DD_CONFIG_ITEM_MOTION_ALARM_OUT DD_CONFIG_ITEM_RELAY_SETUP DD_CONFIG_ITEM_NETWORK_SMTP DD_CONFIG_ITEM_PTZ_PRESET DD_CONFIG_ITEM_PTZ_SETUP DD_CONFIG_ITEM_VIDEO_COLOR DD_CONFIG_ITEM_ENCODE_MASK_MAJOR DD_CONFIG_ITEM_ENCODE_MASK_MINOR DD_CONFIG_ITEM_ENCODE_SCHEDULE DD_CONFIG_ITEM_ENCODE_NETWORK

Programming Guide

This section mainly introduces functions in SDK with flow charts and text brief description.

Client SDK Instructions

main flow of calling SDK interface

It has seven function modules,each function module has four necessary the same flow:initialize SDK,user to login,user to logout and free SDK resource.

Initialization of SDK interface:initialize whole network SDK system,preassign internal memory etc.

Interface of setting connection timeout:this part is optional,user can set SDK network connection timeout time according to their need.If not call this interface,adopt the default in SDK.

Set callback function of receiving exception information:most module function in SDK is realized with asynchronous mode,so we provide this interface to receive exception in preview,alarming, playback and talkback module.User can call this callback function after initialization of SDK to receive and dispose exception from all module on application layer.

Get device IP address from analysis server:this interface provides the function of getting device IP address from analysis server in condition of only knowing device name and serial number. Interface of user register:realize function of

register,under successful register,returned ID is the unique identification for other functions.SDK permits maximum of register user count is 512.For device,this version permits 32 register username,meanwhile permits 128 users to register at the same time. Preview module:get real time code stream from front server,decode to display and play control function etc. Play and download module:playback or download from front server by time remotely ,later encode and store,meanwhile support resuming from break point. Parameter configuration module:set and obtain parameter of front server,including device parameter,network parameter,channel compression parameter,port parameter,alarm parameter,exception parameter,exchange information and user configuration parameter etc.

Remote device maintenance module:close device,reboot device,recover default setting, format harddisk remotely,remote upgrade,import&export of configuration file etc.

Audio talkback module:audio data talkback and obtain from front server,audio encoding method can be appointed.

Alarm module:dispose alarm signal uploaded from front server.

PTZ control module:including basic operation on PTZ,preset point,cruise and track control.SDK divides PTZ control into two methods:control by the returned handle from image preview;preview without limit,user to control PTZ through register ID.

Client SDK Instructions

flow of preview module

The part marked with dotted line is relative to preview module,call it in condition of open preview,the two modules are collocated and they both have their own functions.

Volume control function mainly means monopolization and volume control.

Real time data capture and record module mainly means data callback and local record for later dispose. Capture function mainly means capturing current image in decoding and store them into .BMP file.

PTZ module means PTZ control and operation function in condition of open preview,including PTZ preset point,cruise and track etc.

Client SDK Instructions

flow of talkback and forward module

Talkback function sends and receives audio between PC and device.Call NET_SDK_StartVoiceCom after device register,meanwhile user can get data which is sent by

device or collected by PC through setting callback function.

Firstly call NET_SDK_StartVoiceCom_MR to start talkback(connection with device has been done and wait for sending data). Second,prepare which data to send(need to encode),flow of encoding is the part in purple,if data has been compressed in appointed method omit the encoding part.Data resource can be from PC sound card or read from file,but must be compressed with our compression algorithm. After encoding operation we can get encoded data in fixed size,then call NET_SDK_VoiceComSendData to send the data to device. After sending all data,call NET_SDK_StopVoiceCom to stop forward connection with device.

Client SDK Instructions

flow of alarm module

Alarm method in this version is arming:SDK connects device actively, sends command of uploading alarm,then device alarms and sends to SDK. From the flow chart we can know that arming method needs user

register first.The part in purple is a necessary conditon for realizing alarm information upload, mainly finishes relative alarm condition and process configuration,the interface of parameter configuration is NET_SDK_GetDVRConfig and NET_SDK_SetDVRConfig.Configurated struct for single quantity alarm is NET_SDK_AlarmInfo,if these parameters configuration has been done, next step is setting alarm callback function NET_SDK_SetDVRMessageCallBack,after above steps,set arming alarm NET_SDK_SetupAlarmChan. Cancel arming interface should be called after the whole alarm uploading progress.

Client SDK Instructions

flow of playback and download by time module

When playback and downloading by time,user needn't call relative find file interface but just appoint the start time and stop time in interface. The start play command of control interface should be called after calling playback and downloading interface,then the nearest video will playback or download by appinted time range. Also user can call relative interface of finding record file to get the start time and stop time,and appoint the time parameter according to the returned time range,at last the start play command of control interface should be called too.

Client SDK Instructions

flow of remote device maintainance module

Remote device maintenance module includes getting device work status,remote upgrade,log search,recover device default parameter and import&export configuration file etc.

Get device work status:get device current harddisk state,channel state,alarm import and export state,local display state and audio channel state,the part marked with dotted line is reserved temporarily.

Remote upgrade:upgrade device and get the upgrade progress and state.

Search log:search current device log information,including alarm,exception,operation and log with S.M.A.R.T information.

Recover default device parameter:call NET_SDK_RestoreConfig to recover all default parameter setting.

Import and export configuration file :export current all configuration information and store them or import the appointed configuration information.

Client SDK Instructions

flow of intelligent alarm

Client SDK Instructions

flow of exception

Client SDK Instructions

flow of config N9000's face album target

Client SDK Instructions

flow of config IPC's face album target

Client SDK Instructions

flow of accept device's register

Interface Definition

This section mainly introduces all the interface definitions involved in SDK , one single page corresponding to one interface definition,

and has a brief description for parameters and return values.

Further more the structures' definitions that involved in interface definitions are added,also one single page corresponding to one structure definition.

Client SDK Instructions

Macro Definition

value of

macro definition	 macro	meaning definition

DD_MAX_CAMERA_NUM	128

DD_MAX_CAMERA_NUM_BYTE_LEN	16

DD_MAX_SERIAL_NUMBER_LEN	64

DD_MAX_VERSION_BUF_LEN	64

DD_MAX_NAME_LEN	64

DD_MAX_NAME_BUF_LEN	132

DD_MAX_CAMERA_NAME_LEN	64

the maximum number of inserted camera shooting device

the maximum byte length of inserted camera shooting device

length of serial number

length of version buffer

length of user name

buffer length of user name

the maximum

DD_MAX_CAMERA_NAME_BUF_LEN	132

DD_MAX_URL_LEN	256

DD_MAX_URL_BUF_LEN	260

DD_MAX_COLOR_CFG_NUM	3

DD_MAX_TEXT_LEN	64

DD_MAX_TEXT_BUF_LEN	132

name length of inserted camera shooting device

the maximum name buffer length of inserted camera device

the maximum length of input URL

the maximum buffer length of input URL

number of stream for controlling color

the maximum length of input text

the maximum buffer length of input text

DD_MAX_VIDEO_COVER_NUM	3

DD_MAX_USER_NAME_LEN	64

DD_MAX_USER_NAME_BUF_LEN	132

DD_MAX_PASSWORD_LEN	128

DD_MAX_PASSWORD_BUF_LEN	132

DD_MAX_PPPOE_ACCOUNT_LEN	128

DD_MAX_PPPOE_ACCOUNT_BUF_LEN 132

DD_MAX_DDNS_ACCOUNT_LEN	128

the maximum number of video override

the maximum length of user name

buffer length of user name

the maximum length of password

the maximum buffer length of password

the maximum length of PPPOE dialling number

buffer length of PPPOE dialling number

the maximum length of

+ 3) / 4	of motion

DDNS number

the maximum

DD_MAX_DDNS_ACCOUNT_BUF_LEN 132	length of DDNS number

DD_MAX_EMAIL_RECEIVE_ADDR_NUM3

DD_MAX_MOTION_AREA_WIDTH_NUM1920/16

number of address for receiving emails

value of the width of motion area

value of DD_MAX_MOTION_AREA_HIGHT_NUM ((1080/16)the height

area

DD_MAX_PRESET_NUM

DD_MAX_CRUISE_NUM

DD_MAX_TRACK_NUM

DD_MAX_ACCOUNT_NUM

DD_MAX_BUF_SIZE

128

32

1

64

512*1024

number of PTZ preset points

number of PTZ cruise

number of PTZ track

the maximum number of user

size of buffer

Client SDK Instructions

NET_SDK_Init

initialize SDK,before calling other functions in SDK

BOOL NET_SDK_Init( );

Return Values

TRUE means success; FALSE means failure.To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_Cleanup

Client SDK Instructions

NET_SDK_SetConnectTime

set network connection timeout time and connection times

BOOL NET_SDK_SetConnectTime( DWORD	dwWaitTime

DWORD	dwTryTimes );

Parameters

dwWaitTime

[in] timeout time,in millisecond,its value is greater than 300. actual maximum timeout value is connect timeout value(connect timeout value depends on different system)the excess part is invalid,default value is 5 seconds.

dwTryTimes

[in] reconnection times(kept),default value is 3

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

SDK default timeout value is 5 seconds.

Client SDK Instructions

NET_SDK_SetReconnect

set reconnection function

BOOL NET_SDK_SetReconnect( DWORD	dwInterval, BOOL	bEnableRecon

);

Parameters

dwInterval

[in] reconnection interval,in millisecond,default value is 30 seconds

bEnableRecon

[in] whether to reconnect,0- no reconnect,1-reconnect,default value is 1

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

This interface can control preview, transparent channel and reconnection functions in arming at the same time.When it isn't called SDK enables the three functions acquiescently, and the reconnection time interval is 5 seconds.

Client SDK Instructions

NET_SDK_Cleanup

before finish, the last step is to free SDK resource.

BOOL NET_SDK_Cleanup( );

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_Init

Client SDK Instructions

NET_SDK_DiscoverDevice

discover device automatically on LAN

long NET_SDK_DiscoverDevice( NET_SDK_DEVICE_DISCOVERY_INFO

*pDeviceInfo, long bufNum;

long waitSeconds; );

Parameters

*pDeviceInfo

[in] an array witch is needed to asign values,its size is bufNum ,if descovered device num is more than,the returned size is just bufNUm

bufNum

[in] size of the array waitSeconds

[in] time to discover devices, unit is second,this interface will be returned after waitSeconds

Return Values

Returned value is the number of discovered devices, if no device is found or discovering device gets error,the value is 0. To get error information, please refer to NET_SDK_GetLastError

See Also

NET_SDK_GetDeviceInfo

Client SDK Instructions

NET_SDK_DiscoverDeviceStart

start discover device on LAN(asynchronous, only for windows)

unsigned int NET_SDK_DiscoverDeviceStart( IPTool_SearchDataCallBack

SearchCallBack, IPTool_SearchDataCallBackEx

SearchCallBackEx, void     *pParam,

unsigned int SearchTypeMask, int nMaxRecordCount

);

Parameters

SearchCallBack

search result callback during searching, result format is xml

SearchCallBackEx

[in] search result callback during searching, result format is SEARCHED_DEVICE_INFO struct

*pParam

pointer to user data SearchTypeMask

search type，refer to the table：

type	value

_SEARCH_STANDARD 0x001

_SEARCH_ONVIF	0x002

_SEARCH_UPNP	0x004

meanning

standard device

onvif device

upnp device

_SEARCH_AIPSTAR	0x008

_SEARCH_DAHUA	0x010

_SEARCH_HIK	0x020

_SEARCH_UNIVIEW	0x040

_SEARCH_YCX	0x080

_SEARCH_SPECO	0x100

_SEARCH_ALL	0xffff

AIPSTAR device

DAHUA device

HIK device

UNIVIEW device

YCX device

SPECO device

all type

nMaxRecordCount

max number of searched devices

Return Values

Returned value is the handle of searching. To get error information, please refer to NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_DiscoverDeviceStop

stop asynchronous searching（only for windows）

void NET_SDK_DiscoverDeviceStop( unsigned int hSearch,

);

Parameters

hSearch

the handle of searching, the return value of NET_SDK_DiscoverDeviceStart

Client SDK Instructions

IPTool_SearchDataCallBack

call back of asynchronous searching

void *IPTool_SearchDataCallBack( char* hwaddr ,

char* szDevIP , int	opt ,

const char*	szXmlData, void *	pParam,

const char * szRecvFromNIC );

Parameters

hwaddr

hard ware address szDevIP

IP opt

reserve szXmlData

the searched device's information with xml format pParam

reserve szRecvFromNIC

reserve

Return Values

No return value. To get error information, please refer to NET_SDK_GetLastError

Client SDK Instructions

IPTool_SearchDataCallBackEx

call back of asynchronous searching

void *IPTool_SearchDataCallBackEx( char* hwaddr ,

char* szDevIP , int	opt ,

const SEARCHED_DEVICE_INFO *	pData , void *	pParam,

const char * szRecvFromNIC );

Parameters

hwaddr

hard ware address szDevIP

IP opt

reserve pData

the searched device's information pParam

reserve szRecvFromNIC

reserve

Return Values

No return value. To get error information, please refer to NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_SetRegisterCallback

callback function when device receives DVR registered local port

BOOL NET_SDK_SetRegisterCallback( ACCEPT_REGISTER_CALLBACK fRegisterCBFun

void	*pUser );

Parameters

fRegisterCBFun

[in] callback information when device receives DVR registered local port

*pUser

[in] custom parameter passed by user

Return Values

TRUE means sccess; FALSE means failure. To get error code, please call NET_SDK_GetLastError

See Also

NET_SDK_SetRegisterPort      ACCEPT_REGISTER_CALLBAC

K

Client SDK Instructions

NET_SDK_SetUnRegisterCallback

the function is a callback for the unregistered device,calls NET_SDK_AddRegisterDeviceInfo to notify SDK after receiving callback

BOOL NET_SDK_SetUnRegisterCallback(

ACCEPT_UNREGISTER_CALLBACK

fUnRegisterCBFun

void	*pUser );

Parameters

fUnRegisterCBFun

[in] callback information when device receives DVR registered local port

*pUser

[in] custom parameter passed by user

Return Values

TRUE means sccess; FALSE means failure. To get error code, please call NET_SDK_GetLastError

See Also

NET_SDK_SetRegisterPort        ACCEPT_REGISTER_CALLB

ACK

Client SDK Instructions

NET_SDK_SetRegisterPort

local port when device receives DVR's register

BOOL NET_SDK_SetRegisterPort( WORD wRegisterPort

);

Parameters

wRegisterPort

[in] local port when device receives DVR's register

Return Values

TRUE means sccess; FALSE means failure. To get error code, please call NET_SDK_GetLastError

See Also

NET_SDK_SetRegisterCallback      ACCEPT_REGISTER_CALL

BACK

Client SDK Instructions

NET_SDK_AddRegisterDeviceInfo

add the informations of the devices which need to auto register to sdk

BOOL NET_SDK_AddRegisterDeviceInfo( REG_LOGIN_INFO * pLoginInfo, unsigned int deviceNum

);

Parameters

REG_LOGIN_INFO

[in] the pointer of the devices' information deviceNum

[in] number of the devices

Return Values

TRUE means sccess; FALSE means failure. To get error code, please call NET_SDK_GetLastError

See Also

NET_SDK_SetRegisterCallback      ACCEPT_REGISTER_CALL

BACK

Client SDK Instructions

ACCEPT_REGISTER_CALLBACK

callback information when device receives DVR's register

void ACCEPT_REGISTER_CALLBACK( LONG	lUserID,

LONG	lRegisterID, LPNET_SDK_DEVICEINFO           pDeviceInfo, void	*pUser

);

Parameters

lUserID

[in] connection ID,other interface accesses device through this ID

lRegisterID

[in] device received DVR initiative register ID pDeviceInfo

[in] information of initiative register device *pUser

[in] custom parameter passed by user

Return Values

None. To get error code, please call NET_SDK_GetLastError

See Also

NET_SDK_SetRegisterCallback      NET_SDK_SetRegisterPort

Client SDK Instructions

NET_SDK_GetDeviceIPByName

Get the device Ip by the device name

BOOL NET_SDK_GetDeviceIPByName( char	*sSerIP,

DWORD	wSerPort; char             *sDvrName; char             *sDvrIP

);

Parameters

*sSerIP

[in] the IP address of IPSever wSerPort

[in] the port of IPSever *sDvrName

[in] the device name automatically reported to IP Server

*sDvrIP

[in] the device port automatically reported to IP Server

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_DiscoverDevice

Client SDK Instructions

NET_SDK_SetSDKMessageCallBack

callback function of SDK operation exception

BOOL NET_SDK_SetSDKMessageCallBack( UINT	nMessage,

HWND	hWnd, EXCEPTION_CALLBACK fExceptionCallBack, void	*pUser

);

Parameters

nMessage

[in] message hWnd

[in] window handle of receiving exception message fExceptionCallBack

[in] callback function of receiving exception message,callback current relevant information of exception

*pUser

[in] user data

Callback Function

void (CALLBACK fExceptionCallBack)( DWORD	dwType,

LONG	lUserID, LONG	lHandle, void	*pUser );

Callback Function Parameters

dwType

message type of exception or reconnection lUserID

login ID lHandle

relevant type handle of exception pUser

user data

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Client SDK Instructions

EXCEPTION_CALLBACK

data callback during SDK exception

void *EXCEPTION_CALLBACK( DWORD	dwType, LONG	lUserID, LONG	lHandle, void	*pUser

);

Parameters

dwType

[in] type of exception or reconnection message,refer to NET_SDK_EXCEPTION_TYPE:

Type	Value Description

NETWORK_DISCONNECT	0

NETWORK_RECONNECT	1

Disconnection

Reconnection

NETWORK_CH_DISCONNECT2

Channel Disconnection

NETWORK_CH_RECONNECT 3

Channel Reconnection

lUserID

[in] login ID lHandle

[in] corresponding type handle when exception * pUser

[in] pointer to user data

Return Values

None. To get error code, please call NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_SetVideoEffect

Set the display parameter of the video.

BOOL NET_SDK_SetVideoEffect( LONG	lUserID,

LONG	lChannel, DWORD	dwBrightValue,

DWORD	dwContrastValue, DWORD	dwSaturationValue, DWORD	dwHueValue

);

Parameters

lUserID

[in] return value of NET_SDK_Login lChannel

[in] return value of NET_SDK_CLIENTINFO,channel number starts from 0

dwBrightValue

[out] brightness,range[0,255] dwContrastValue

[out] contrast,range[0,255] dwSaturationValue

[out] saturation,range[0,255] dwHueValue

[out] gray scale,range[0,255]

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_LivePlay

Client SDK Instructions

NET_SDK_GetVideoEffect

Set the display parameter of the video.

BOOL NET_SDK_GetVideoEffect( LONG	lUserID,

LONG	lChannel, DWORD	*pBrightValue,

DWORD	*pContrastValue, DWORD	*pSaturationValue, DWORD	*pHueValue

);

Parameters

lUserID

[in] return value of NET_SDK_Login lChannel

[in] return value of NET_SDK_CLIENTINFO,channel number starts from 0

pBrightValue

[out] pointer to brightness,range[0,255] pContrastValue

[out] pointer to contrast,range[0,255] pSaturationValue

[out] pointer to saturation,range[0,255] pHueValue

[out] pointer to gray scale,range[0,255]

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_LivePlay

Client SDK Instructions

NET_SDK_SetVideoEffect_Ex

Set the display parameter of the video.

BOOL NET_SDK_SetVideoEffect_Ex( LONG	lUserID,

LONG	lChannel, DWORD      dwBrightValue,

DWORD	dwContrastValue, DWORD	dwSaturationValue, DWORD	dwHueValue

);

Parameters

lUserID

[in] the return value of NET_SDK_Login() lChannel

[in] the return value of NET_SDK_CLIENTINFO. The channel number starts from 0

dwBrightValue

[out] Brightness: the return value range of NET_SDK_SetVideoEffect_Ex : [minValue,maxValue]

dwContrastValue

[out] Contrast: the return value range of NET_SDK_SetVideoEffect_Ex: [minValue,maxValue]

dwSaturationValue

[out] Saturation: the return value range of NET_SDK_SetVideoEffect_Ex : [minValue,maxValue]

dwHueValue

[out] Hue: the return value range of NET_SDK_SetVideoEffect_Ex :[minValue,maxValue]

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_LivePlay

Android Interface

The corresponding Android interface：NO

Client SDK Instructions

typedef struct {

unsigned int	minValue; // the minimum value

unsigned int

// the maximum value unsigned int

// the current value unsigned int

maxValue;

curValue;

defaultValue;

// the default value }NET_SDK_IMAGE_EFFECT_T;

NET_SDK_GetVideoEffect_Ex

Get the display parameter of video.

BOOL NET_SDK_GetVideoEffect_Ex( LONG	lUserID,

LONG	lChannel, NET_SDK_IMAGE_EFFECT_T	*pBrightValue,

NET_SDK_IMAGE_EFFECT_T	*pContrastValue, NET_SDK_IMAGE_EFFECT_T	*pSaturationValue, NET_SDK_IMAGE_EFFECT_T	*pHueValue

);

Parameters

lUserID

[in] the return value of NET_SDK_Login(). lChannel

[in] the return value of NET_SDK_CLIENTINFO. The channel number starts from 0.

pBrightValue

[out] a pointer to brightness pContrastValue

[out] a pointer to contrast pSaturationValue

[out] a pointer to saturation pHueValue

[out] a pointer to hue

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_LivePlay

Android Interface

The corresponding Android interface：NO

Client SDK Instructions

NET_SDK_GetSDKBuildVersion

get version and build information of SDK

DWORD NET_SDK_GetSDKBuildVersion( );

Return Values

Version and build information of SDK.Two high bytes are the version:25~32 bits are main version,17~24 bits are minor version;two low bytes are build information. Take 0x01000101 for example:version is 1.0,build number is 0101.

Client SDK Instructions

NET_SDK_GetSDKVersion

get information of SDK version

DWORD NET_SDK_GetSDKVersion( );

Return Values

Version of SDK.Two high bytes are the version:25~32 bits are main version,17~24 bits are minor version;two low bytes are build information. Take 0x01000101 for example:version is 1.0,build number is 0101.

Client SDK Instructions

NET_SDK_SetLogToFile

enable writing log file

BOOL NET_SDK_SetLogToFile( BOOL	bLogEnable,

char	*strLogDir, BOOL	bAutoDel

);

Parameters

bLogEnable

[in] whether enable the function of writing log,default value is FALSE

strLogDir

[in] directory of log file,default directory is "C:\\SdkLog\\"

bAutoDel

[in] whether delete excess file count,default value is TRUE

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

Directory of log file must be absolute path,and ends with "\\",for example:"C:\\SdkLog\\".Advise user to create file manually,if no specified path,adopt default path "C:\\SdkLog\\". This interface can be called many times to create log files, and support creating 10 files at most.When assign bAutoDel TRUE,system will delete excess file

automatically.New directory will be valid in writing file when changing directory or next time to write file.

Client SDK Instructions

NET_SDK_GetErrorMsg

return the last error code message

char* NET_SDK_GetErrorMsg( LONG *pErrorNo

);

Parameters

pErrorNo

[out] pointer to the value of error code

Return Values

return value is pointer to error code information. error message has two main types,error message of network communication library and error message of soft and hard decoding library,list the first type as follows:

error message of network communication library

type of errors

NET_SDK_SUCCESS

NET_SDK_PASSWORD_ERROR

NET_SDK_NOENOUGH_AUTH

NET_SDK_NOINIT

NET_SDK_CHANNEL_ERROR

NET_SDK_OVER_MAXLINK

NET_SDK_LOGIN_REFUSED

NET_SDK_VERSION_NOMATCH

NET_SDK_NETWORK_FAIL_CONNECT

NET_SDK_NETWORK_NOT_CONNECT

NET_SDK_NETWORK_SEND_ERROR

NET_SDK_NETWORK_RECV_ERROR

NET_SDK_NETWORK_RECV_TIMEOUT

NET_SDK_NETWORK_ERRORDATA

NET_SDK_ORDER_ERROR

NET_SDK_OPER_BY_OTHER

NET_SDK_OPER_NOPERMIT

NET_SDK_COMMAND_TIMEOUT

NET_SDK_ERROR_SERIALPORT

NET_SDK_ERROR_ALARMPORT

NET_SDK_PARAMETER_ERROR

NET_SDK_CHAN_EXCEPTION

NET_SDK_NODISK

error value

0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

i

no error

user's name or passw

no right for this ope

SDK is not initialize

error of channel num

the client connected

SDK login is refuse

version doesn't matc

failed to connect to

network isn't conne

failed to send data t

failed to receive the

timeout when receiv

send illegal data to

the called order erro

operation method is

the privileged user i

DVR command tim

error of serial port n

error of alarm port n

parameter error

server's channel is i

no hard disk

NET_SDK_ERROR_DISKNUM

NET_SDK_DISK_FULL

NET_SDK_DISK_ERROR

NET_SDK_NOSUPPORT

NET_SDK_BUSY

NET_SDK_MODIFY_FAIL

NET_SDK_PASSWORD_FORMAT_ERROR

NET_SDK_DISK_FORMATING

NET_SDK_DVR_NORESOURCE

NET_SDK_DVR_OPRATE_FAILED

NET_SDK_OPEN_HOSTSOUND_FAIL

NET_SDK_DVR_VOICEOPENED

NET_SDK_TIME_INPUTERROR

NET_SDK_NOSPECFILE

NET_SDK_CREATEFILE_ERROR

NET_SDK_FILEOPENFAIL

NET_SDK_OPERNOTFINISH

NET_SDK_GETPLAYTIMEFAIL

NET_SDK_PLAYFAIL

NET_SDK_FILEFORMAT_ERROR

NET_SDK_DIR_ERROR

NET_SDK_ALLOC_RESOURCE_ERROR

NET_SDK_AUDIO_MODE_ERROR

NET_SDK_NOENOUGH_BUF

NET_SDK_CREATESOCKET_ERROR

23	hard disk no. error

24	server hark disk is f

25	server hard disk erro

26	server does not supp

27	server is busy

28	failed to modify in t

29	the password inputt

30	hard disk is formatt

31	DVR no resources

32	DVR failed to opea

33	failed open PC voic

34	server voice dialogu

35	time input is not cor

36	there is no appointe

37	failed to create a fil

38	faile to open a file

39	the last operation is

40	faile to get the curre

41	failed to play

42	the file input format

43	path error

44	resources allotting e

45	display card mode e

46	buffer is not enough

47	establish SOCKET

NET_SDK_SETSOCKET_ERROR

NET_SDK_MAX_NUM

NET_SDK_USERNOTEXIST

NET_SDK_WRITEFLASHERROR

NET_SDK_UPGRADEFAIL

NET_SDK_CARDHAVEINIT

NET_SDK_PLAYERFAILED

NET_SDK_MAX_USERNUM

NET_SDK_GETLOCALIPANDMACFAIL

NET_SDK_NOENCODEING

NET_SDK_IPMISMATCH

NET_SDK_MACMISMATCH

NET_SDK_UPGRADELANGMISMATCH

NET_SDK_MAX_PLAYERPORT

NET_SDK_NOSPACEBACKUP

NET_SDK_NODEVICEBACKUP

NET_SDK_PICTURE_BITS_ERROR

NET_SDK_PICTURE_DIMENSION_ERROR

NET_SDK_PICTURE_SIZ_ERROR

NET_SDK_LOADPLAYERSDKFAILED

NET_SDK_LOADPLAYERSDKPROC_ERROR

NET_SDK_LOADDSSDKFAILED

NET_SDK_LOADDSSDKPROC_ERROR

NET_SDK_DSSDK_ERROR

NET_SDK_VOICEMONOPOLIZE

48	set SOCKET error

49	the max number

50	user doest not exit

51	wirte FLASH error

52	failed to upgrade DV

53	the decode card is in

54	player failed

55	the max user no.

failed to get the IP a 56	end or physical add

57	the channel is not co

58	IP address not matc

59	MAC address not m

60	the language of upg

61	reach to the max pla

62	no enough space to

63	no backup device

64	the bits of picture n

65	the dimension is ov

66	the size of picture is

67	failed to load player

68	not find some funct

69	failed to load DsSD

70	not find some funct

71	failed to call functio

72	voice card is monop

NET_SDK_JOINMULTICASTFAILED	73

NET_SDK_CREATEDIR_ERROR	74

NET_SDK_BINDSOCKET_ERROR	75

NET_SDK_SOCKETCLOSE_ERROR	76

NET_SDK_USERID_ISUSING	77

NET_SDK_PROGRAM_EXCEPTION	78

NET_SDK_WRITEFILE_FAILED	79

NET_SDK_FORMAT_READONLY	80

NET_SDK_WITHSAMEUSERNAME	81

NET_SDK_DEVICETYPE_ERROR	82

NET_SDK_LANGUAGE_ERROR	83

NET_SDK_PARAVERSION_ERROR	84

NET_SDK_FILE_SUCCESS	85

NET_SDK_FILE_NOFIND	86

NET_SDK_NOMOREFILE	87

NET_SDK_FILE_EXCEPTION	88

NET_SDK_TRY_LATER	89

NET_SDK_DEVICE_OFFLINE	90

NET_SDK_CREATEJPEGSTREAM_FAIL	91

NET_SDK_USER_ERROR_NO_USER	92

NET_SDK_USER_ERROR_USER_OR_PASSWORD_IS_NULL 93

NET_SDK_USER_ERROR_ALREDAY_LOGIN	94

NET_SDK_USER_ERROR_SYSTEM_BUSY	95

NET_SDK_DEVICE_NOT_SUPPROT	96

NET_SDK_USER_ERROR_SYSTEM_NO_READY	97

failed join to multic

failed to create log f

failed to bind socke

socket is closed

the user ID is opera

sdk program except

write file failed

failed to format read

there is same userna

device type no matc

language no match

soft version no matc

file has been created

file isn't found

there is no more file

file exception

Try again later

Device offline

Failed to create JPE

No such user!

No username or pas

The user has been lo

The device is busy.

The device don not

Do not complete ge

NET_SDK_CHANNEL_OFFLINE

NET_SDK_GETREADYINFO_FAIL

NET_SDK_NORESOURCE

NET_SDK_DEVICE_QUERYSYSTEMCAPS_FAIL

NET_SDK_INBUFFER_TOSMALL

NET_SDK_NO_PASSWORD_STRENGTH

98	Camera is offline.

99	It fails to get device

100	SDK resources is no

101	The device fails to g

102	The input buffer are

103	The password stren

Remarks

Get error number through function NET_SDK_GetErrorMsg

See Also

NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_Login

user to register device

LONG NET_SDK_Login( char

WORD char char

NET_SDK_DEVICEINFO );

*sDVRIP, wDVRPort, *sUserName, *sPassword,

lpDeviceInfo

Parameters

sDVRIP

[in] device IP address wDVRPort

[in] number of device port sUserName

[in] user name for login sPassword

[in] user password lpDeviceInfo

[out] device information

Return Values

-1 means failure and other value is the returned ID from user.The ID is unique. Later operations on device realize through this ID. To get error information,please call NET_SDK_GetLastError

Remarks

Device permits 32 registered names,and supports 128 users to register at the same time.

See Also

NET_SDK_Logout

Client SDK Instructions

NET_SDK_Logout

user to logout

BOOL NET_SDK_Logout( LONG	lUserID

);

Parameters

lUserID

[in] user ID,return value of NET_SDK_Login

Return Values

TRUE means success; FALSE means failure.To get error information, please call NET_SDK_GetLastError

See Also

NET_SDK_Login

Client SDK Instructions

NET_SDK_LoginEx

user to register device

NET_SDK_API LONG CALL_METHOD NET_SDK_LoginEx( char	*sDVRIP,

WORD	wDVRPort, char	*sUserName, char	*sPassword,

NET_SDK_DEVICEINFO           lpDeviceInfo, NET_SDK_CONNECT_TYPE	eConnectType, const char              *sDevSN

);

Parameters

sDVRIP

[in] device IP address or P2P Server address wDVRPort

[in] number of device port or P2P Server port sUserName

[in] user name for login sPassword

[in] user password lpDeviceInfo

[out] device informantion eConnectType

[in] Connect type,refer to NET_SDK_CONNECT_TYPE:

Type	Value NET_SDK_CONNECT_TCP	0 NET_SDK_CONNECT_NAT	1 NET_SDK_CONNECT_NAT20 2

Description TCP connection NAT1.0 connection NAT2.0 connection

sDevSN

[in] Serial number of P2P. If TCP is connected, ignore this parameter

Return Values

-1 means failure and other value is the returned ID from user.The ID is unique. Later operations on device realize through this ID. To get error information,please callNET_SDK_GetLastError

Remarks

Device permits 32 registered names,and supports 128 users to register at the same time.

See Also

NET_SDK_Logout

Client SDK Instructions

NET_SDK_SetNat2Addr

set the p2p2.0 address

LONG NET_SDK_SetNat2Addr( char	*sServerAddr, WORD                    wDVRPort, );

Parameters

sServerAddr

[in] device IP address （it is the p2p server address （c2020.autonat.com））

wDVRPort

[in] p2p port

Return Values

TRUE means success; FALSE means failure.To get error information, please call NET_SDK_GetLastError

See Also

NET_SDK_LoginEx

Client SDK Instructions

NET_SDK_MakeKeyFrame

one key frame from main code stream dynamically

BOOL NET_SDK_MakeKeyFrame( LONG	lUserID,

LONG	lChannel );

Parameters

lUserID

[in] return value of NET_SDK_Login lChannel

[in] channel number,start from 0

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

This interface is set to reset I frame.According to the preview parameter(NET_SDK_CLIENTINFO) type main code stream or sub code stream calls NET_SDK_MakeKeyFrame or NET_SDK_MakeKeyFrameSub to realize resetting I frame.

See Also

NET_SDK_MakeKeyFrameSub

Client SDK Instructions

NET_SDK_MakeKeyFrameEx

one key frame from main code stream dynamically

BOOL NET_SDK_MakeKeyFrameEx( LONG	lUserID,

LONG unsigned );

lChannel，

int	streamType

Parameters

lUserID

[in] return value of NET_SDK_Login lChannel

[in] channel number,start from 0

streamType

[in] stream type

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

This interface is set to reset I frame.According to the preview parameter(NET_SDK_CLIENTINFO) type main code stream or sub code stream calls NET_SDK_MakeKeyFrame or NET_SDK_MakeKeyFrameSub to realize resetting I frame.

See Also

NET_SDK_MakeKeyFrameSub

Client SDK Instructions

NET_SDK_MakeKeyFrameSub

one key frame from sub code stream dynamically

BOOL NET_SDK_MakeKeyFrameSub( LONG	lUserID,

LONG	lChannel );

Parameters

lUserID

[in] return value of NET_SDK_Login lChannel

[in] channel number starts from 0

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Remarks

This interface is set to reset I frame.According to the preview parameter(NET_SDK_CLIENTINFO) type main code stream or sub code stream calls NET_SDK_MakeKeyFrame or NET_SDK_MakeKeyFrameSub to realize resetting I frame.

See Also

NET_SDK_MakeKeyFrame

Client SDK Instructions

NET_SDK_LivePlay

real time preview

LONG NET_SDK_LivePlay( LONG LPNET_SDK_CLIENTINFO LIVE_DATA_CALLBACK void

);

lUserID, lpClientInfo, fLiveDataCallBack, * pUser

Parameters

lUserID

[in] return value of NET_SDK_Login() lpClientInfo

[in] preview parameter fLiveDataCallBack

[in] preview data callback parameter,default value is NULL

* pUser

[in] pointer to user,default value is NULL

Return Values

-1 means failure and other value is parameter of handle of function NET_SDK_StopLivePlay. To get error information,please call NET_SDK_GetLastError

Remarks

After calling this interface successfully,if need to capture real time code stream data,call NET_SDK_SetLiveDataCallBack to register the callback

function for capturing code stream data,and access the code stream data in the callback function.

See Also

NET_SDK_StopLivePlay

Client SDK Instructions

NET_SDK_StopLivePlay

stop preview

BOOL NET_SDK_StopLivePlay( LONG	lLiveHandle

);

Parameters

lLiveHandle

[in] handle for preview,return value of NET_SDK_LivePlay

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_LivePlay

Client SDK Instructions

DRAW_FUN_CALLBACK

data callback when preview

void *DRAW_FUN_CALLBACK( LONG	lLiveHandle, HDC	hDC,

void	* pUser );

Parameters

lLiveHandle

[in] preview interface handle hDC

[in] device context handle *pUser

[in] pointer to user information

Return Values

None. To get error code,please call NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_LivePlay_Ex

Real-time Preview

LONG NET_SDK_LivePlayEx( LONG                      lUserID, LPNET_SDK_CLIENTINFO

LIVE_DATA_CALLBACK_EX void	* pUser );

lpClientInfo, fLiveDataCallBack,

Parameters

lUserID

[in] the return value of NET_SDK_Login() lpClientInfo

[in] preview parameters fLiveDataCallBack

[in] preview data callback parameters, the default value is NULL .

* pUser

[in] user pointer, the default value is NULL.

Return Values

-1 means failure; other values is the handles of the functions, like NET_SDK_StopLivePlay. To get error information, please call NET_SDK_GetLastError

Remarks

Having sucessfully called up this interface, call up the callback function of stream data captured through the registeration of the interface

(NET_SDK_SetLiveDataCallBackEx) to get the real-time stream data.

See Also

NET_SDK_StopLivePlay      NET_SDK_GetLivePlayerIndex

Client SDK Instructions

NET_SDK_SupportStreamNum

get the number of streams which support live

unsigned int LONG

LONG );

NET_SDK_SupportStreamNum( lUserID, lChannel

Parameters

lUserID

[in] the return value of NET_SDK_Login() lChannel

[in] the index of channel，from 0

Return Values

return the number of streams which support live。To get error information, please callNET_SDK_GetLastError

Client SDK Instructions

NET_SDK_RegisterDrawFun

overlapping picture of character and image when preview or playback

BOOL NET_SDK_RigisterDrawFun( LONG	lLiveHandle, DRAW_FUN_CALLBACK fDrawFun, void	*pUser

);

Parameters

lLiveHandle

[in] return value of NET_SDK_LivePlay fDrawFun

[in] callback function for image pUser

[in] pointer to user data

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

This interface is mainly used to register callback function,and get current surface device context. User can draw or write on this DC just like on the window's client DC,but this DC is Off-Screen surface DC in player DirectDraw,not window's client DC .

See Also

NET_SDK_LivePlay

Client SDK Instructions

NET_SDK_SetPlayerBufNumber

set frame buffer area count of broadcast library

BOOL NET_SDK_SetPlayerBufNumber( LONG	lLiveHandle,

DWORD	dwBufNum );

Parameters

lLiveHandle

[in] return value of NET_SDK_LivePlay dwBufNum

[in] the maximum frame count in buffer area when playing one frame by one frame,range[1,50],default value is 15 in SDK

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

This interface is called to adjust network delay and play fluency.value of dwBufNum more large,fluency more well,relative delay more large and vice versa. But when network isn't enough well,lost frame makes effect on fluency of broadcast.If current stream is complex,assign buffer frame count equal to or greater than 6 to ensure audio and video effect. This function follows NET_SDK_LivePlay closely,because it's invalid after playing image .

See Also

NET_SDK_LivePlay

Client SDK Instructions

NET_SDK_OpenSound

open sound in monopolistic sound card mode

BOOL NET_SDK_OpenSound( LONG	lLiveHandle

);

Parameters

lLiveHandle

[in] return value of NET_SDK_LivePlay

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

If current mode is sharing,calling this interface returns failure.The monopolistic mode just opens one channel to playback, when in turn to open multichannel,just opens the last one.

See Also

NET_SDK_LivePlay NET_SDK_CloseSound

Client SDK Instructions

NET_SDK_Volume

adjust play volume

BOOL NET_SDK_Volume( LONG	lLiveHandle, WORD	wVolume

);

Parameters

lLiveHandle

[in] return value of NET_SDK_LivePlay wVolume

[in] volumeless, range[0,0xffff]

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

See Also

NET_SDK_LivePlay

Client SDK Instructions

NET_SDK_CloseSound

close sound under monopolistic sound card mode

BOOL NET_SDK_CloseSound( );

Return Values

TRUE means success; FALSE means failure. to get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_OpenSound

Client SDK Instructions

NET_SDK_SetLiveDataCallBack

set preview data callback

BOOL NET_SDK_SetLiveDataCallBack( LONG	lLiveHandle, LIVE_DATA_CALLBACK fLiveDataCallBack, void	*pUser

);

Parameters

lLiveHandle

[in] return value of NET_SDK_LivePlay fLiveDataCallBack

[in] callback function of code stream pUser

[in] user data

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

This function includes start and stop operation on user dispose captured data by SDK,when assigning callback function fLiveDataCallBack other value except NULL, start callback and disposing data,when assigning NULL,stop callback and disposing data.The first package in callback is a file head with 40 bytes,for the later usage in decoding,the next package which to call is compressed code stream.

See Also

NET_SDK_LivePlay

Client SDK Instructions

NET_SDK_SaveLiveData

save real time preview data

BOOL NET_SDK_SaveLiveData( LONG	lLiveHandle,

char	*sFileName );

Parameters

lLiveHandle

[in] return value of NET_SDK_LivePlay *sFileName

[in] pointer to file directory

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_LivePlay NET_SDK_StopSaveLiveData

Client SDK Instructions

NET_SDK_SetLiveDataCallBackEx

Set the callback of preview data.

BOOL NET_SDK_SetLiveDataCallBackEx( LONG	lLiveHandle,

LIVE_DATA_CALLBACK_EX fLiveDataCallBack, void	*pUser

);

Parameters

lLiveHandle

[in] the return value of NET_SDK_LivePlay() fLiveDataCallBack

[in] callback function of stream data pUser

[in] user data

Return Values

TRUE means success; FALSE means failure. To get error code, please call NET_SDK_GetLastError

Remarks

This function includes start and stop operation on user dispose captured data by SDK,when assigning callback function fLiveDataCallBack other value except NULL, start callback and disposing data,when assigning NULL,stop callback and disposing data.The first package in callback is a file head with 40 bytes,for the later usage in decoding,the next package which to call is compressed code stream.

See Also

NET_SDK_LivePlay

Client SDK Instructions

NET_SDK_StopSaveLiveData

stop data capture

BOOL NET_SDK_StopSaveLiveData( LONG	lLiveHandle

);

Parameters

lLiveHandle

[in] return value of NET_SDK_LivePlay

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_LivePlay NET_SDK_SaveLiveData

Client SDK Instructions

YUV_DATA_CALLBACK

YUV data cabllback after decoding the captured real-time data

void YUV_DATA_CALLBACK( POINTERHANDLE

DECODE_FRAME_INFO void	* pUser );

lLiveHandle, frameInfo,

Parameters

lLiveHandle

[in] the handle of real-time preview frameInfo

[in] YUV data after decoding the frame of real-time preview data

*pUser

[in] the pointer of user information

Return Values

No return value. To get error information, please call NET_SDK_GetLastError

Android Interface

Client SDK Instructions

NET_SDK_SetYUVCallBack

Configure the YUV data cabllback after decoding video.

BOOL NET_SDK_SetYUVCallBack( LONG	lLiveHandle,

YUV_DATA_CALLBACK fYuvCallBack, void	*pUser

);

Parameters

lLiveHandle

[in] the return value of NET_SDK_LivePlay() fYuvCallBack

[in] YUV stream data callback function pUser

[in] user data

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

This function includes starting and stopping processing data captured by SDK . When the callback function (fYuvCallback) set to any vaule except null, it means callback and processing data; when it set to null, it means stopping callback and processing data. The callback data are the YUV data after decoding.

See Also

NET_SDK_LivePlay

Android Interface

Client SDK Instructions

LIVE_DATA_CALLBACK

data callback when capture data in living preview

void LIVE_DATA_CALLBACK( LONG

NET_SDK_FRAME_INFO BYTE

void );

lLiveHandle, frameInfo, *pBuffer,

* pUser

Parameters

lLiveHandle

[in] real time preview handle frameInfo

[in] preview data frame information in real time *pBuffer

[in] pointer to buffer area *pUser

[in] pointer to user information

Return Values

None. To get error code, please call NET_SDK_GetLastError

Client SDK Instructions

LIVE_DATA_CALLBACK_Ex

Data callback when capturing the real-time preview data.

void LIVE_DATA_CALLBACK_Ex( LONG	lLiveHandle, UINT                         dataType, BYTE                         *pBuffer, UINT                         dataLen, void                         * pUser

);

Parameters

lLiveHandle

[in] real-time preview handle dataType

[out] data frame type, see DD_FRAME_TYPE *pBuffer

[out] a pointer to the buffer, the contents is NET_SDK_FRAME_INFO + FrameData dataLen

[out] the data length of the buffer *pUser

[out] a pointer to a user

Return Values

No return value. To get error information, please call NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_CapturePicture

capture data one frame by one frame and store to be BMP file.

BOOL NET_SDK_CapturePicture( LONG	lLiveHandle,

char	*sPicFileName );

Parameters

lLiveHandle

[in] return value of NET_SDK_LivePlay() sPicFileName

[in] directory of storing .BMP image,the length of directory is less than or equals to 256 bytes

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

This interface is used to capture present frame of decoding to be .BMP image.

See Also

NET_SDK_LivePlay     NET_SDK_CapturePicture_Other

Client SDK Instructions

NET_SDK_CapturePicture_Other

capture data one frame by one frame and store to be BMP file.

BOOL NET_SDK_CapturePicture_Other( LONG	lUserID,

LONG	lChannel, char	*sPicFileName );

Parameters

lUserID

[in] user ID lChannel

[in] channel number sPicFileName

[in] directory of storing .BMP image,the length of directory is less than or equals to 256 bytes

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

This interface is used to capture certain user ID and channel frame of decoding to be .BMP image.

See Also

NET_SDK_CapturePicture

Client SDK Instructions

NET_SDK_CaptureJPEGData_V2

Capture data in JPEG format.

BOOL NET_SDK_CaptureJPEGData_V2( LONG	lUserID,

LONG	lChannel,

char	*sJpegPicBuffer, DWORD        dwPicSize,

LPDWORD	lpSizeReturned );

Parameters

lUserID

[in] user ID lChannel

[in] channel number sJpegPicBuffer

[out] JPEG data buffer dwPicSize

[in] sJpegPicBuffer buffer size lpSizeReturned

[out] JPEG data size

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

This interface is used to capture jpeg data,do not need to open the stream. Only applicable to specific firmware.

See Also

NET_SDK_CaptureJPEGFile_V2

Client SDK Instructions

NET_SDK_CaptureJPEGFile_V2

Capture data in JPEG format and store to be JPEG file.

BOOL NET_SDK_CaptureJPEGFile_V2( LONG	lUserID,

LONG	lChannel, char	*sPicFileName );

Parameters

lUserID

[in] user ID lChannel

[in] channel number *sPicFileName

[in] save the file path

Return Values

TRUE means success; FALSE means failure. To get error information,please callNET_SDK_GetLastError

Remarks

This interface is used to capture jpeg data and store,do not need to open the stream. Only applicable to specific firmware

See Also

NET_SDK_CaptureJPEGData_V2

Client SDK Instructions

NET_SDK_CaptureJPEGPict ure

Capture data in JPEG format and store to be JPEG file.

BOOL NET_SDK_CaptureJPEGFile_V2( LONG	lUserID,

LONG	lChannel, LPNET_SDK_JPEGPARA lpJpegPara, char *sJpegPicBuffer ,

DWORD dwPicSize, LPDWORD lpSizeReturned );

Parameters

lUserID

[in] user ID lChannel

[in] channel number *LPNET_SDK_JPEGPARA

[in] a command type sJpegPicBuffer

[in]JPEG data buffer dwPicSize

[in]sJpegPicBuffer buffer size lpSizeReturned

[in]JPEG data size

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

This interface is used to capture jpeg data and store,do not need to open the stream. Only applicable to specific firmware

See Also

NET_SDK_CaptureJPEGData_V2

Android Interface

The corresponding Android interface：no

Client SDK Instructions

NET_SDK_RemoteSnap

Caputre images by controlling devices remotely. The images are saved in the device end.(only N9000 availabe）

BOOL NET_SDK_RemoteSnap( LONG	lUserID,

int lChannel, );

Parameters

lUserID

[in] the return value of NET_SDK_Login() lChannel

[in] Channel number（starting with 0）

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Client SDK Instructions

typedef struct _net_sdk_image_sreach

{

DWORD	dwChannel;//Snapshot channel

（starting with 0）

DD_TIME	StartTime; //time DD_TIME	StopTime; //time

DWORD	pageIndex;//Page number DWORD	pageSize;//Page IMAGE_SORT_TYPE sort; //return

unsigned char	resv[8]; }NET_SDK_IMAGE_SREACH;

typedef struct _net_sdk_image_ {

DWORD	dwChannel; //Snapshot cahnnel DWORD	dwImageType; //Snapshot type

IMAGE_EVENT_TYPE

DD_TIME	captureTime;//Snapshot time DWORD	totalNum;//Totol number unsigned char        resv[8];

}NET_SDK_IMAGE;

NET_SDK_SearchPictures

Get the image list of the remote device.( only N9000 device availabe)

BOOL NET_SDK_SearchPictures( LONG lUserID,

NET_SDK_IMAGE_SREACH sInSreachImage, LONG lInImageBufferSize,

NET_SDK_IMAGE *pOutImageInfo, LONG *pOutImageNum

);

Parameters

lUserID

[in] return value of NET_SDK_Login() sInSreachImage

[in] Search condition lInImageBufferSize

[in] the space of pOutImageInfo applied for pOutImageInfo

[out] Return image information pOutImageNum

[out] the number of returning to image information

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Remarks

This interface can realize JPEG snapshot and save the files without enabling stream. It is only available for specified

devices.

See Also

NET_SDK_CaptureJPEGData_V2

Client SDK Instructions

//Image Type

enum IMAGE_MODE {

IMAGE_MODE_JPG, IMAGE_MODE_PNG, IMAGE_MODE_BMP,

};

typedef struct _net_sdk_image_info {

unsigned int	imageSize;

IMAGE_MODE	imageMode; //Image format unsigned char	resv[8];

}NET_SDK_IMAGE_INFO;

NET_SDK_DownLoadPicture

Uploading remote images（only N9000 devices availabe）

BOOL NET_SDK_DownLoadPicture( LONG lUserID,

NET_SDK_IMAGE captureImage, NET_SDK_IMAGE_INFO *pOutImageInfo, char* pOutBuffer,

int outBufferSize );

Parameters

lUserID

[in] return value of NET_SDK_Login() captureImage

[in] NET_SDK_SearchPictures returns a data of the searched image list

pOutImageInfo

[out] return the uploaded image information pOutBuffer

[out] return the image data（when the current image exists and the space requested is larger than or equal to the image size, it takes effect)

outBufferSize

[int] buffer area size of pOutBuffer requesting

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Two conditions to return successfully：

1、When the image exsits and the size of outBufferSize is less than the image size，the return will be successful and the upper application can decode the image size from pOutImageInfo parameters.

2、When the image exsits and the size of outBufferSize is larger than or equal to the downloaded image size, the return value of pOutBuffer is image data.

Client SDK Instructions

NET_SDK_FindFile

find record file by time

LONG NET_SDK_FindFile(

LONG	lUserID, LONG	lChannel, DD_TIME             lpStartTime, DD_TIME             lpStopTime );

Parameters

lUserID

[in] return value of NET_SDK_Login() lChannel

[in] channel number,start from 0 lpStartTime

[in] the start time of file lpStopTime

[in] the stop time of file

Return Values

-1 means failure and other value is a parameter of function NET_SDK_FindClose. To get error information,please call NET_SDK_GetLastError

Remarks

This interface specifies the type and time range of finding record file,after calling the interface,call NET_SDK_FindNextFile to get file information.

See Also

NET_SDK_FindNextFile NET_SDK_FindClose

Client SDK Instructions

NET_SDK_FindNextFile

get file information one by one

LONG NET_SDK_FindNextFile(

LONG	lFindHandle, NET_SDK_REC_FILE       lpFindData );

Parameters

lFindHandle

[in] handle of finding file,return value of NET_SDK_FindFile()

lpFindData

[out] pointer to store file information

Return Values

-1 means failure and other value is current state information. To get error information,please call NET_SDK_GetLastError

Remarks

Before calling this interface,call NET_SDK_FindFile to get current handle for finding

See Also

NET_SDK_FindFile

Client SDK Instructions

NET_SDK_FindClose

close finding filename, free resource

BOOL NET_SDK_FindClose( LONG lFindHandle

);

Parameters

lFindHandle

[in] return handle of finding file in function NET_SDK_FindFile

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_FindFile

Client SDK Instructions

NET_SDK_FindRecDate

find record file by data

LONG NET_SDK_FindRecDate( LONG	lUserID

);

Parameters

lUserID

[in] user ID

Return Values

-1 means failure and other value is the return information of finding. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_FindRecDateClose NET_SDK_FindNextRecDate

Client SDK Instructions

NET_SDK_FindNextRecDate

get record file one by one

LONG NET_SDK_FindNextRecDate( LONG	lFindHandle, DD_DATE     *lpRecDate

);

Parameters

lFindHandle

[in] handle of finding *lpRecDate

[in] date of record

Return Values

-1 means failure and other value is the return value of finding. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_FindRecDate NET_SDK_FindRecDateClose

Client SDK Instructions

NET_SDK_FindRecDateClose

close finding record file by date,free resource

BOOL NET_SDK_FindRecDateClose( LONG	lFindHandle

);

Parameters

lFindHandle

[in] handle of finding

Return Values

TURE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_FindRecDate NET_SDK_FindNextRecDate

Client SDK Instructions

NET_SDK_FindEvent

find record file by event

LONG NET_SDK_FindEvent( LONG	lUserID,

LONG	lChannel, DWORD	dwRecType, DD_TIME             lpStartTime, DD_TIME             lpStopTime );

Parameters

lUserID

[in] return value of NET_SDK_Login() lChannel

[in] channel number,start from 0 dwRecType

[in] type of event lpStartTime

[in] pointer to the start time of file lpStopTime

[in] pointer to the stop time of file

Return Values

-1 means failure and other value is parameter of function NET_SDK_FindNextEvent. To get error information,please call NET_SDK_GetLastError

Remarks

This interface specifies information of finding record file(by event),after calling the interface,call

NET_SDK_FindNextEvent to get file information. The record file which is found by event aims at the start time and stop time,so it only supports playback by time.

See Also

NET_SDK_FindNextEvent NET_SDK_FindEventClose

Client SDK Instructions

NET_SDK_FindNextEvent

get information of found file one by one

LONG NET_SDK_FindNextEvent(

LONG	lFindHandle, NET_SDK_REC_EVENT         *lpRecEvent );

Parameters

lFindHandle

[in] handle of finding file,return value of NET_SDK_FindEvent()

*lpRecEvent

[out] pointer to store file information

Return Values

-1 means failure and other value is current state information. To get error information,please call NET_SDK_GetLastError

Remarks

Before calling this interface,call NET_SDK_FindEvent to get current handle for finding.The record file found by event aims at the start time and stop time,so only supports playback by time.

See Also

NET_SDK_FindEvent

Client SDK Instructions

NET_SDK_FindEventClose

close finding file by event,free resource

BOOL NET_SDK_FindEventClose( LONG	lFindHandle

);

Parameters

lFindHandle

[in] handle of query

Return Values

TURE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_FindEvent NET_SDK_FindNextEvent

Client SDK Instructions

NET_SDK_FindTime

find record file by time

LONG NET_SDK_FindTime( LONG	lUserID, LONG	lChannel,

DD_TIME     * lpStartTime, DD_TIME     * lpStopTime );

Parameters

lUserID

[in] user ID lChannel

[in] channel number,start from 0 * lpStartTime

[in] pointer to the start time * lpStopTime

[in] pointer to the stop time

Return Values

-1 means failure and other value is the result of finding. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_FindNextTime NET_SDK_FindTimeClose

Client SDK Instructions

NET_SDK_FindNextTime

get record file one by one

LONG NET_SDK_FindNextTime(

LONG	lFindHandle, NET_SDK_REC_TIME *lpRecTime );

Parameters

lFindHandle

[in] handle of finding *lpRecTime

[in] time of record

Return Values

-1 means failure and other value is found information. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_FindTime NET_SDK_FindTimeClose

Client SDK Instructions

NET_SDK_FindTimeClose

close finding record file by time,free resource

BOOL NET_SDK_FindTimeClose( LONG	lFindHandle

);

Parameters

lFindHandle

[in] handle of finding

Return Values

TURE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_FindTime NET_SDK_FindNextTime

Client SDK Instructions

NET_SDK_PlayBackByTime

Play back the record file by time and the main stream is requested.

LONG NET_SDK_PlayBackByTime( LONG	lUserID, LONG	*pChannels, LONG	channelNum,

DD_TIME            *lpStartTime, DD_TIME            *lpStopTime, HWND	*hWnd

);

Parameters

lUserID

[in] return value of NET_SDK_Login pChannels

[in] channel number,start from 0 array for playback channelNum

[in] quatity of channels in array pChannels *lpStartTime

[in] pointer to the start time of the file *lpStopTime

[in] pointer to the stop time of the file *hWnd

[in] window handle for playback,if null,SDK still can receive code stream data,but can't decode to display.

Return Values

-1 means failure and other value is parameter of NET_SDK_StopPlayBack. To get error information,please call NET_SDK_GetLastError

Remarks

This interface specifies which record file to play,after finishing calling this interface,call NET_SDK_SetPlayDataCallBack to register callback function and dispose the captured code stream data by itself.

See Also

NET_SDK_PlayBackControl

SDK_SetLiveDataCallBack

NET_SDK_StopPlayBack NET_

NET_SDK_Login

Client SDK Instructions

NET_SDK_PlayBackControl

control the state of playback

BOOL NET_SDK_PlayBackControl( LONG	lPlayHandle,

DWORD	dwControlCode, DWORD	dwInValue, DWORD	*lpOutValue

);

Parameters

lPlayHandle

[in] play handle,return value of NET_SDK_PlayBackByTime dwControlCode

[in] command of controlling the state of playback,refer to NET_SDK_PLAYCTRL_TYPE:

Type

NET_SDK_PLAYCTRL_PAUSE

Description

pause

NET_SDK_PLAYCTRL_FF NET_SDK_PLAYCTRL_REW NET_SDK_PLAYCTRL_RESUME NET_SDK_PLAYCTRL_STOP NET_SDK_PLAYCTRL_FRAME NET_SDK_PLAYCTRL_NORMAL

NET_SDK_PLAYCTRL_STARTAUDIO

NET_SDK_PLAYCTRL_STOPAUDIO

fast forward rewind resume stop

play one frame normal play

enable audio,choose channel by parameter dwInValue of function NET_SDK_PlayBackControl

stop audio

adjust audio volume,realize by

NET_SDK_PLAYCTRL_AUDIOVOLUMEparameter dwInValue of function NET_SDK_PlayBackControl

NET_SDK_PLAYCTRL_SETPOS	play progress,calculate seconds from January 1st

in 1970

dwInValue

[in] parameter setting,when setting playback progress this parameter means progress value; when playing this parameter means file location of resuming from break point.

lpOutValue

[out] got parameter,if need to get total time of current play file,this parameter meets.

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

Whether assign the third parameter of this interface depends on the control command,under NET_SDK_PLAYSETPOS command this parameter means the playback progress; when starting the control command it means offset of current file;when its value is 0 it means to play from the starting location,if not 0 ,it means the file location of resuming from break point.

The fourth parameter means the parameter got from current control command operation.

See Also

NET_SDK_PlayBackByTime

Client SDK Instructions

NET_SDK_StopPlayBack

stop record file playback

BOOL NET_SDK_StopPlayBack( LONG	lPlayHandle

);

Parameters

lPlayHandle

[in] handle for playback,return value of NET_SDK_PlayBackByTime

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_PlayBackByTime

Client SDK Instructions

PLAY_DATA_CALLBACK

data callback when finding file and playback

void *PLAY_DATA_CALLBACK( LONG

NET_SDK_FRAME_INFO BYTE

void );

lPlayHandle, frameInfo, *pBuffer, *pUser

Parameters

lPlayHandle

[in] play handle frameInfo

[in] file data playback code stream frame information *pBuffer

[in] buffer pointer to find files and playback * pUser

[in] pointer to user information

Return Values

None. To get error code, please call NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_PlayBackByTimeEx

Play back the record files by time.

LONG NET_SDK_PlayBackByTimeEx( LONG	lUserID,

LONG	*pChannels, LONG	channelNum,

DD_TIME            *lpStartTime, DD_TIME            *lpStopTime,

HWND	*hWnds BOOL               bFirstStream );

Parameters

lUserID

[in] the return value of NET_SDK_Login() pChannels

[in] the channel number group of playback channelNum

[in] the channel quantities of pChannels *lpStartTime

[in] a pointer to start time of the file *lpStopTime

[in] a pointer to end time of the file *hWnds

[in] the handle of playback window, if it is null, SDK still can receive the stream data but not decoding.

bFirstStream

[in] Whether to play back the main stream. false is sub-stream.

Return Values

-1 means failure; other vallues is the parameters of the functions, like NET_SDK_StopPlayBack. To get error information, please call NET_SDK_GetLastError

Remarks

This interface specifies the record files needed to play. After successfully call up this interface, please register callback function by the interface of NET_SDK_SetPlayDataCallBack to capture the recording stream data and dispose the captured code stream data by itself.

See Also

NET_SDK_PlayBackControl      NET_SDK_StopPlayBack

NET_SDK_SetLiveDataCallBack       NET_SDK_Login

Client SDK Instructions

NET_SDK_SetPlayDataCallBack

register callback function,capture record data

BOOL NET_SDK_SetPlayDataCallBack( LONG	lPlayHandle,

PLAY_DATA_CALLBACK fPlayDataCallBack, void	*pUser

);

Parameters

lPlayHandle

[in] play handle,return value of NET_SDK_PlayBackByTime

fPlayDataCallBack

[in] callback function of record data pUser

[in] user data

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

This function includes start and stop operation on user dispose captured data by SDK,when assigning callback function fLiveDataCallBack other value except NULL, start callback and disposing data,if assign NULL,stop callback and disposing data.

See Also

NET_SDK_PlayBackByTime

Client SDK Instructions

NET_SDK_SetPlayYUVCallBack

(only in windows now)register callback function,capture record data.it can receive YUV data from the callback function,you should return quickly if you need to process the data.

BOOL NET_SDK_SetPlayYUVCallBack( LONG	lPlayHandle,

PLAY_YUV_DATA_CALLBACK fYuvDataCallBack, void	*pUser

);

Parameters

lPlayHandle

[in] play handle,return value of NET_SDK_PlayBackByTime

fYuvDataCallBack

[in] callback function of record data pUser

[in] user data

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

This function includes start and stop operation on user dispose captured data by SDK,when assigning callback function fYuvDataCallBack other value except NULL, start callback and disposing data,if assign NULL,stop callback and disposing data.

See Also

NET_SDK_PlayBackByTime

Client SDK Instructions

NET_SDK_PlayBackSaveData

capture record data of playback and store them into file

BOOL NET_SDK_PlayBackSaveData( LONG	lPlayHandle,

LONG	lChannel, char	*sFileName );

Parameters

lPlayHandle

[in] play handle,return value of NET_SDK_PlayBackByTime

lChannel

[in] channel number,start from 0 *sFileName

[in] directory of storing data

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_PlayBackByTime

Client SDK Instructions

NET_SDK_StopPlayBackSave

stop saving record data

BOOL NET_SDK_StopPlayBackSave( LONG	lPlayHandle,

LONG	lChannel );

Parameters

lPlayHandle

[in] handle for play,return value of NET_SDK_PlayBackByTime

lChannel

[in] channel number,start from 0

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_PlayBackByTime

Client SDK Instructions

NET_SDK_GetPlayBackOsdTime

get OSD time when playback

BOOL NET_SDK_GetPlayBackOsdTime( LONG	lPlayHandle, DD_TIME          lpOsdTime

);

Parameters

lPlayHandle

[in] player handle,return value of NET_SDK_PlayBackByTime

*lpOsdTime

[out] pointer to OSD time

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_PlayBackByTime

Client SDK Instructions

NET_SDK_RefreshPlay

refresh window to display playback

BOOL NET_SDK_RefreshPlay( LONG	lPlayHandle

);

Parameters

lPlayHandle

[in] playback handle,return value of NET_SDK_PlayBackByTime

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

When pause or playback in one frame,if refresh the window,the image disappears ,at that time call this interface to display,this interface is valid just when pause and playback in one frame.

See Also

NET_SDK_PlayBackByTime

Client SDK Instructions

NET_SDK_PlayBackCaptureFile

capture when playback and store them to file

BOOL NET_SDK_PlayBackCaptureFile( LONG	lPlayHandle,

LONG	lChannel, char	*sFileName );

Parameters

lPlayHandle

[in] play handle,return value of NET_SDK_PlayBackByTime

lChannel

[in] channel number,start from 0 *sFileName

[in] directory of storing picture data

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_PlayBackByTime

Client SDK Instructions

NET_SDK_GetFileByTime

download record file by time

LONG NET_SDK_GetFileByTime( LONG	lUserID, LONG	lChannel,

DD_TIME            *lpStartTime, DD_TIME            *lpStopTime, char	*sSavedFileName );

Parameters

lUserID

[in] return value of NET_SDK_Login lChannel

[in] channel number,start from 0 *lpStartTime

[in] pointer to the start time *lpStopTime

[in] pointer to the stop time *sSavedFileName

[in] directory of storing downloaded file to PC

Return Values

-1 means failure and other value is parameter of NET_SDK_StopGetFile. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_PlayBackControl NET_SDK_StopGetFile

NET_SDK_Login

Client SDK Instructions

NET_SDK_GetDownloadPos

get current progress of downloading record file

int NET_SDK_GetDownloadPos( LONG	lFileHandle

);

Parameters

lFileHandle

[in] handle for downloading,return value of NET_SDK_GetFileByTime()

Return Values

-1 means failure;0-100 means the progress of downloading;100 means finish;normal range is 0-100, if the return value is 200,network is unusual. To get error information,please call NET_SDK_GetLastError

Remarks

This interface is to get progress of downloading record file by file name.

See Also

NET_SDK_GetFileByTime

Client SDK Instructions

NET_SDK_StopGetFile

stop downloading record file

BOOL NET_SDK_StopGetFile( LONG	lFileHandle

);

Parameters

lFileHandle

[in] handle for download,return value of NET_SDK_GetFileByTime

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_GetFileByTime

Client SDK Instructions

NET_SDK_GetFileByTimeEx

Extended interface of download record file by time

LONG NET_SDK_GetFileByTime( LONG	lUserID, LONG	lChannel,

DD_TIME            *lpStartTime, DD_TIME            *lpStopTime, char	*sSavedFileName BOOL	bCustomFormat BOOL	bUseCallBack

BACKUP_DATA_CALLBACK     fBackupDataCallBack void	*pUser

)

Parameters

lUserID

[in] return value of NET_SDK_Login lChannel

[in] the channel number starts from 0 *lpStartTime

[in] a pointer to the start time *lpStopTime

[in] a pointer to the end time *sSavedFileName

[in] the directory of storing downloaded file to PC bCustomFormat

[in] whether to use private protocl format. TRUE means private protocol format

bUseCallBack

[in] whether to use call back function，TRUE means use

fBackupDataCallBack

[in] call back function *pUser

[in] user's pointer，default is NULL

Return Values

-1 means failure and other value is parameter of NET_SDK_StopGetFile. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_PlayBackControl NET_SDK_StopGetFile

NET_SDK_Login

Client SDK Instructions

NET_SDK_GetFileByTimeExV2

Extended interface of download record file by time version 2

LONG NET_SDK_GetFileByTimeExV2( LONG	lUserID, LONG	lChannel,

DD_TIME            *lpStartTime, DD_TIME            *lpStopTime, char	*sSavedFileName char	recFormat

BOOL               bFirstStream BOOL               bUseCallBack

BACKUP_DATA_CALLBACK     fBackupDataCallBack void	*pUser

)

Parameters

lUserID

[in] eturn value of NET_SDK_Login lChannel

[in] the channel number starts from 0 *lpStartTime

[in] a pointer to the start time *lpStopTime

[in] a pointer to the end time *sSavedFileName

[in] the directory of storing downloaded file to PC recFormat

[in] whether to use private protocl format. 1 means private protocol format

bFirstStream

[in] whether to download first stream, TRUE means yes

bUseCallBack

[in] whether to use call back function，TRUE means use

fBackupDataCallBack [in] call back function

*pUser

[in] user's pointer，default is NULL

Return Values

-1 means failure and other value is parameter of NET_SDK_StopGetFile. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_PlayBackControl NET_SDK_StopGetFile

NET_SDK_Login

Client SDK Instructions

BACKUP_DATA_CALLBACK

Call back the data when downloading the records.

void *PLAY_DATA_CALLBACK( LONG

UINT BYTE UINT void );

lFileHandle, dataType, *pBuffer, dataLen, *pUser

Parameters

lFileHandle

[in] download file handle dataType

[in] data type *pBuffer

[in] File download stream and frame information dataLen

[in] data length * pUser

[in] a pointer to user information

Return Values

No return value. To get error information, please callNET_SDK_GetLastError

Client SDK Instructions

NET_SDK_SaveFileToUsbByTime

Save the record to the USB of the device（support only N9000）。

BOOL NET_SDK_SaveFileToUsbByTime( LONG	lUserID,

NET_SDK_REC_FILE           *recordFile, USB_BACKUP_FORMAT	recFormat );

Parameters

lUserID

[in] the return value of NET_SDK_Login. *recordFile

[in] the pointer of the record file recFormat

[in] 0 is avi，1 is private format

Return Values

FALSE means failed，TRUE means success。To get error information, please callNET_SDK_GetLastError

See Also

NET_SDK_GetSaveFileToUsbProcess

Client SDK Instructions

NET_SDK_GetSaveFileToUsbProcess

Get the process and status of the saving record to USB device（support only N9000）。

BOOL NET_SDK_GetSaveFileToUsbProcess( LONG	lUserID, NET_SDK_USB_BACKUP_PROCESS_EX

*pUsbBackProcess,

unsigned int              lBuffSize, unsigned int              *taskCount

);

Parameters

lUserID

[in] the return value of NET_SDK_Login *pUsbBackProcess

[in] the pointer of the results lBuffSize

[in] the expect number of the result *taskCount

[in] the actual number of the result

Return Values

FALSE means failed，TRUE means success。To get error information, please callNET_SDK_GetLastError

See Also

NET_SDK_SaveFileToUsbByTime

Client SDK Instructions

NET_SDK_LockFile

lock record file(only support IPC)

BOOL NET_SDK_LockFile( LONG	lUserID,

NET_SDK_REC_FILE *pFileTolock, LONG	fileNum

);

Parameters

lUserID

[in] user ID pFileTolock

[in] pointer to lock file fileNum

[in] quantity of file

Return Values

TURE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_UnlockFile

Client SDK Instructions

NET_SDK_UnlockFile

unlock record file(only support IPC)

BOOL NET_SDK_UnlockFile( LONG	lUserID,

NET_SDK_REC_FILE *pFileToUnlock, LONG	fileNum

);

Parameters

lUserID

[in] user ID pFileToUnlock

[in] pointer to unlock file fileNum

[in] quantity of files

Return Values

TURE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_LockFile

Client SDK Instructions

NET_SDK_DeleteRecFile

delete recorded file(only support 3.0DVR)

BOOL NET_SDK_DeleteRecFile( LONG	lUserID,

NET_SDK_REC_FILE *pFileToUnlock, LONG	fileNum

);

Parameters

lUserID

[in] user ID *pFileToUnlock

[in] pointer to unlock file fileNum

[in] number of files

Return Values

TURE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_StartDVRRecord

start device to record manually and remotely(only support N9000)

BOOL NET_SDK_StartDVRRecord( LONG	lUserID,

LONG	lChannel, LONG	lRecordType );

Parameters

lUserID

[in] return value of NET_SDK_Login lChannel

[in] channel number,start from 0,0x00ff means all analog channels,0xff00 means all digital channels,0xffff means all analog channels and digital channels

lRecordType

[in] record type

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_StopDVRRecord

Client SDK Instructions

NET_SDK_StopDVRRecord

stop device record manually and remotely(only support N9000)

BOOL NET_SDK_StopDVRRecord( LONG	lUserID,

LONG	lChannel );

Parameters

lUserID

[in] return value of NET_SDK_Login lChannel

[in] channel number,start from 0,0x00ff means all analog channels,0xff00 means all digital channels,0xffff means all analog channels and digital channels

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

See Also

NET_SDK_StartDVRRecord

Client SDK Instructions

NET_SDK_SetDVRMessageCallBack

register callback function,receive alarm information from device etc.

BOOL NET_SDK_SetDVRMessCallBack( NET_MESSAGE_CALLBACK fMessCallBack void	*pUser

);

Parameters

fMessCallBack

[in] callback function pUser

[in] user information

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_SetupAlarmChan

Client SDK Instructions

NET_MESSAGE_CALLBACK

data callback when alarm

BOOL NET_MESSAGE_CALLBACK( LONG	lCommand, LONG	lUserID, char	*pBuf, DWORD	dwBufLen, void	* pUser

);

Parameters

lCommand

[in] command handle,refer to the list below:

Type

NET_SDK_ALARM

NET_SDK_RECORD

NET_SDK_IVM_RULE

NET_SDK_TRADEINFO

NET_SDK_IPCCFG

Description

device alarm information

device record information

Intelligent behavior analysis information(reserved)

ATM trade information(reserved)

IPC information change of mixed DVR(reserved)

lUserID

[in] user ID

*pBuf

[in] pointer to buffer area,when lCommand value is NET_SDK_ALARM pBuf is array of struct NET_SDK_ALARMINFO,when lCommand value is NET_SDK_RECORD pBuf is array of struct NET_SDK_RECORD_STATUS

dwBufLen

[in] length of buffer area * pUser

[in] pointer to user information

Return Values

None. To get error code, please call NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_SetDVRMessageCallBackEx

register callback function,receive alarm information from device etc.

BOOL NET_SDK_SetDVRMessageCallBackEx( NET_MESSAGE_CALLBACK_EX fMessCallBack void	*pUser

);

Parameters

fMessCallBack

[in] callback function pUser

[in] user information

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_SetupAlarmChan

Client SDK Instructions

NET_MESSAGE_CALLBACK_EX

data callback when alarm

BOOL NET_MESSAGE_CALLBACK_EX( LONG	lCommand,

LONG	lUserID, char	*pBuf, DWORD	dwBufLen, void	* pUser );

Parameters

lCommand

[in] command handle,refer to the list below:

Type

NET_SDK_ALARM

NET_SDK_RECORD

NET_SDK_IVM_RULE

NET_SDK_TRADEINFO

NET_SDK_IPCCFG

Description

device alarm information

device record information

Intelligent behavior analysis information(reserved)

ATM trade information(reserved)

IPC information change of mixed DVR(reserved)

lUserID

[in] user ID

*pBuf

[in] pointer to buffer area,when lCommand value is NET_SDK_ALARM pBuf is array of struct NET_SDK_ALARMINFO_EX ,when lCommand value is NET_SDK_RECORD pBuf is array of

struct NET_SDK_RECORD_STATUS_EX dwBufLen

[in] length of buffer area * pUser

[in] pointer to user information

Return Values

None. To get error code, please call NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_SetupAlarmChan

build uploading channel for alarm,get alarm information

LONG NET_SDK_SetupAlarmChan( LONG lUserID

);

Parameters

lUserID

[in] return value of NET_SDK_Login

Return Values

-1 means failure and other value is handle parameter of NET_SDK_CloseAlarmChan. To get error information, please call NET_SDK_GetLastError

Remarks

Start arming and then call the interface of register callback function(just call NET_SDK_SetDVRMessCallBack when need)to get uploading information.

See Also

NET_SDK_CloseAlarmChan NET_SDK_Login

NET_SDK_SetDVRMessCallBack

Client SDK Instructions

NET_SDK_CloseAlarmChan

cancel channel of uploading alarm.

BOOL NET_SDK_CloseAlarmChan( LONG lAlarmHandle

);

Parameters

lAlarmHandle

[in] return value of NET_SDK_SetupAlarmChan()

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

See Also

NET_SDK_SetupAlarmChan

Client SDK Instructions

NET_SDK_PTZControl

PTZ control operation(need to start previewing image)

BOOL NET_SDK_PTZControl( LONG	lLiveHandle, DWORD	dwPTZCommand, DWORD	dwSpeed

);

Parameters

lLiveHandle

[in] return value of NET_SDK_LivePlay dwPTZCommand

[in] PTZ control command,list as follows:

macro definition

PTZ_CMD_STOP

PTZ_CMD_LEFT

PTZ_CMD_RIGHT

PTZ_CMD_UP

PTZ_CMD_DOWN

PTZ_CMD_LEFT_UP

value of maro definition

0

1

2

3

4

5

meaning

PTZ is stopped

PTZ turns left

PTZ turns right

PTZ turns pitch up

PTZ turns pitch under

PTZ turns upleft

PTZ_CMD_LEFT_DOWN	6

PTZ_CMD_RIGHT_UP	7

PTZ_CMD_RIGHT_DOWN 8

PTZ_CMD_NEAR	9

PTZ_CMD_FAR	10

PTZ_CMD_ZOOM_OUT	11

PTZ_CMD_ZOOM_IN	12

PTZ_CMD_IRIS_OPEN	13

PTZ_CMD_IRIS_CLOSE	14

PTZ_CMD_RESET	0xF0

PTZ turns downleft

PTZ turns upright

PTZ turns downright

adjust focus fore

adjust focus aft

focus length decreases

focus length increases

open aperture

close aperture

reset PTZ state

dwSpeed

[in] speed of PTZ ,list as follows:

macro definition

PTZ_SPEED_1 PTZ_SPEED_2 PTZ_SPEED_3

PTZ_SPEED_4

value of macro definition

1 2 3

4

PTZ_SPEED_5	5 PTZ_SPEED_6	6 PTZ_SPEED_7	7 PTZ_SPEED_8	8

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Remarks

Each command operated on PTZ needs to call this interface twice,start command and stop command,which command to call depends on the parameter dwPTZCommand. Before calling this interface preview should be open.Each operation on PTZ corresponding to each control code between device and PTZ,device sends control code to PTZ according to current decoder type and decoder address. If current decoder doesnot match PTZ ,reconfig decoder is necessary.

PTZ default speed is the top speed.

See Also

NET_SDK_LivePlay

Android Interface

The corresponding Android interface

boolean long int int

);

PTZControl( handle, command, speed

Client SDK Instructions

NET_SDK_PTZControl_Other

PTZ control operation(no need to start image preview)

BOOL NET_SDK_PTZControl_Other( LONG	lUserID,

LONG	lChannel, DWORD	dwPTZCommand, DWORD	dwSpeed

);

Parameters

lUserID

[in] return value of NET_SDK_Login lChannel

[in] channel number,start from 0 dwPTZCommand

[in] PTZ control command,list as follows:

macro definition

PTZ_CMD_STOP

PTZ_CMD_LEFT

PTZ_CMD_RIGHT

PTZ_CMD_UP

PTZ_CMD_DOWN

value of macro definition

0

1

2

3

4

meaning

PTZ is stopped

PTZ turns left

PTZ turns right

PTZ turns pitch up

PTZ turns pitch under

PTZ_CMD_LEFT_UP	5

PTZ_CMD_LEFT_DOWN	6

PTZ_CMD_RIGHT_UP	7

PTZ_CMD_RIGHT_DOWN 8

PTZ_CMD_NEAR	9

PTZ_CMD_FAR	10

PTZ_CMD_ZOOM_OUT	11

PTZ_CMD_ZOOM_IN	12

PTZ_CMD_IRIS_OPEN	13

PTZ_CMD_IRIS_CLOSE	14

PTZ_CMD_RESET	0xF0

PTZ turns upleft

PTZ turns downleft

PTZ turns upright

PTZ turns downright

adjust focus fore

adjust focus aft

focus length decreases

focus length increases

open aperture

close aperture

reset PTZ state

dwSpeed

[in] speed of PTZ ,list as follows:

macro definition

PTZ_SPEED_1

PTZ_SPEED_2

value of macro definition

1

2

PTZ_SPEED_3	3 PTZ_SPEED_4	4 PTZ_SPEED_5	5 PTZ_SPEED_6	6 PTZ_SPEED_7	7 PTZ_SPEED_8	8

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Remarks

Each command operated on PTZ needs to call this interface twice,start command and stop command,which command to call depends on the parameter dwPTZCommand. Before calling this interface preview should be open.Each operation on PTZ corresponding to each control code between device and PTZ,device sends control code to PTZ according to current decoder type and decoder address. If current decoder doesnot match PTZ ,reconfig decoder is necessary.

PTZ default speed is the top speed.

See Also

NET_SDK_LivePlay

Android Interface

The corresponding Android interface

boolean long long int int

);

PTZControl_Other( userId,

channel, command, speed

Client SDK Instructions

NET_SDK_PTZPreset

PTZ preset point operation(need to open preview)

BOOL NET_SDK_PTZPreset( LONG	lLiveHandle, DWORD	dwPTZPresetCmd, DWORD	dwPresetIndex );

Parameters

lLiveHandle

[in] return value of NET_SDK_LivePlay dwPTZPresetCmd

[in] operate PTZ preset point command,list as follows:

macro definition

PTZ_CMD_PRESET_SET

PTZ_CMD_PRESET_GO

PTZ_CMD_PRESET_DEL

value of macro definition

16

17

18

meaning

PTZ sets preset points

to appointed preset point

delete preset points

dwPresetIndex

[in] serial number of preset point,at most support 255 points

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

Each operation on PTZ is corresponding to each control code between device and PTZ,device sends control code to PTZ according to current decoder type and decoder address. If current decoder doesnot match PTZ ,reconfig decoder is necessary.

See Also

NET_SDK_LivePlay

Client SDK Instructions

NET_SDK_PTZPreset_Other

PTZ preset point operation

BOOL NET_SDK_PTZPreset_Other( LONG	lUserID,

LONG	lChannel,

DWORD	dwPTZPresetCmd, DWORD	dwPresetIndex );

Parameters

lUserID

[in] return value of NET_SDK_Login lChannel

[in] channel number,start from 0 dwPTZPresetCmd

[in] operate PTZ preset point command,list as follows:

macro definition

PTZ_CMD_PRESET_SET

PTZ_CMD_PRESET_GO

PTZ_CMD_PRESET_DEL

value of macro definition

16

17

18

meaning

PTZ sets preset points

to appointed preset point

delete preset points

dwPresetIndex

[in] serial number of preset point,at most support 255 points

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

Each operation on PTZ is corresponding to each control code between device and PTZ,device sends control code to PTZ according to current decoder type and decoder address. If current decoder doesnot match PTZ ,reconfig decoder is necessary.

See Also

NET_SDK_Login

Client SDK Instructions

NET_SDK_PTZSetCruise

set PTZ cruise line

BOOL NET_SDK_PTZSetCruise(

LONG	lLiveHandle, BYTE	byCruiseRoute, DD_CRUISE_POINT_INFO *pCruisePoint, WORD	pointNum

);

Parameters

lLiveHandle

[in] play handle byCruiseRoute

[in] cruise line *pCruisePoint

[in] cruise point pointNum

[in] count of cruise point

Return Values

TURE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_PTZSetCruise_Other

Client SDK Instructions

NET_SDK_PTZSetCruise_Other

set PTZ cruise line operation

BOOL NET_SDK_PTZSetCruise_Other( LONG	lUserID, LONG	lChannel,

BYTE	byCruiseRoute, DD_CRUISE_POINT_INFO     *pCruisePoint, WORD	pointNum

);

Parameters

lUserID

[in] user ID lChannel

[in] channel number,start from 0 byCruiseRoute

[in] cruise line *pCruisePoint

[in] cruise point pointNum

[in] count of cruise point

Return Values

TURE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_PTZSetCruise

Client SDK Instructions

NET_SDK_PTZCruise

PTZ cruise operation,need to start preview

BOOL NET_SDK_PTZCruise( LONG	lLiveHandle, DWORD	dwPTZCruiseCmd, BYTE	byCruiseRoute, );

Parameters

lLiveHandle

[in] return value of NET_SDK_LivePlay dwPTZCruiseCmd

[in] operate PTZ cruise command,list as follows:

value of

macro definition	 macro	meaning definition

PTZ_CMD_CRUISE_CFG	19

set cruise line,amount to execute Enter,Set and Leave commands

enter cruise mode and then cruise

PTZ_CMD_ENTER_CURISE_MODE20	preset point setting is permitted

PTZ_CMD_LEAVE_CURISE_MODE 22

quit cruise setting

PTZ_CMD_CRUISE_RUN	23

PTZ_CMD_CRUISE_STOP	24

PTZ_CMD_CRUISE_DEL	25

choose a cruise line to cruise

PTZ stops cruise

delete cruise line

byCruiseRoute

[in] cruise path,at most support 32 pathes

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

Each operation on PTZ is corresponding to each control code between device and PTZ,device sends control code to PTZ according to current decoder type and decoder address. If current decoder doesnot match PTZ ,reconfig decoder is necessary.

See Also

NET_SDK_LivePlay

Client SDK Instructions

NET_SDK_PTZCruise_Other

PTZ cruise operation

BOOL NET_SDK_PTZCruise_Other( LONG	lUserID,

LONG	lChannel,

DWORD	dwPTZCruiseCmd, BYTE	byCruiseRoute, );

Parameters

lUserID

[in] return value of NET_SDK_Login lChannel

[in] channel number,start from 0 dwPTZCruiseCmd

[in] operate PTZ cruise command,list as follows:

value of

macro definition	 macro	meaning definition

PTZ_CMD_CRUISE_CFG	19

set cruise line,amount to execute Enter,Set and Leave commands

PTZ_CMD_ENTER_CURISE_MODE20	enter cruise mode and then cruise preset point

PTZ_CMD_LEAVE_CURISE_MODE 22

setting is permitted

quit cruise setting

PTZ_CMD_CRUISE_RUN	23

PTZ_CMD_CRUISE_STOP	24

PTZ_CMD_CRUISE_DEL	25

choose a cruise line to cruise

PTZ stops cruise

delete cruise line

byCruiseRoute

[in] cruise path,at most support 32 pathes

Return Values

TRUE means success; FALSE means failure. to get error information,please call NET_SDK_GetLastError

Remarks

Each operation on PTZ is corresponding to each control code between device and PTZ,device sends control code to PTZ according to current decoder type and decoder address. If current decoder doesnot match PTZ ,reconfig decoder is necessary.

See Also

NET_SDK_Login

Client SDK Instructions

NET_SDK_PTZTrack

PTZ track operation,need to open preview

BOOL NET_SDK_PTZTrack( LONG	lLiveHandle, DWORD	dwPTZTrackCmd );

Parameters

lLiveHandle

[in] return value of NET_SDK_LivePlay dwPTZTrackCmd

[in] operate PTZ track command,list as follows:

value of

macro definition	 macro meaning definition

PTZ_CMD_TRACK_START	26

PTZ_CMD_TRACK_STOP	27

start track

stop track

start to PTZ_CMD_TRACK_START_RECORD28	store

track

stop PTZ_CMD_TRACK_STOP_RECORD 29	storing

track

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_DVR_GetLastError

Remarks

Each operation on PTZ is corresponding to each control code between device and PTZ,device sends control code to PTZ according to current decoder type and decoder address. If current decoder doesnot match PTZ ,reconfig decoder is necessary.

See Also

NET_SDK_LivePlay

Client SDK Instructions

NET_SDK_PTZTrack_Other

PTZ track operation

BOOL NET_SDK_PTZTrack_Other( LONG	lUserID,

LONG	lChannel, DWORD	dwPTZTrackCmd );

Parameters

lUserID

[in] return value of NET_SDK_Login lChannel

[in] channel number,starting with 0 dwPTZTrackCmd

[in] operate PTZ track command,list as follows:

macro definition

PTZ_CMD_TRACK_START PTZ_CMD_TRACK_STOP

PTZ_CMD_TRACK_SET

PTZ_CMD_TRACK_DEL

value of macro definition

26 27

28

29

meaning

start track stop track

start to store track

stop storing track

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_DVR_GetLastError

Remarks

Each operation on PTZ is corresponding to each control code between device and PTZ,device sends control code to PTZ according to current decoder type and decoder address. If current decoder doesnot match PTZ ,reconfig decoder is necessary.

See Also

NET_SDK_Login

Client SDK Instructions

NET_SDK_PTZAutoScan

PTZ automatic scan operation

BOOL NET_SDK_PTZAutoScan( LONG	lLiveHandle, DWORD	dwPTZAutoScanCmd, DWORD	dwSpeed,

BOOL	bIsAutoScan );

Parameters

lLiveHandle

[in] play handle dwPTZAutoScanCmd

[in] PTZ automatic scan command, list as follows:

value of

macro definition	 macro meaning definition

start PTZ_CMD_AUTO_SCAN_START29	automatic

scan

stop PTZ_CMD_AUTO_SCAN_STOP 30	automatic

scan

dwSpeed

[in] speed of PTZ ,list as follows:

macro definition

value of macro definition

PTZ_SPEED_1	1 PTZ_SPEED_2	2 PTZ_SPEED_3	3 PTZ_SPEED_4	4 PTZ_SPEED_5	5 PTZ_SPEED_6	6 PTZ_SPEED_7	7 PTZ_SPEED_8	8

bIsAutoScan

[in] Scan mode, TURE means automatic scan, and FALSE represents random scan

Return Values

TURE means success,FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_PTZAutoScan_Other

Client SDK Instructions

NET_SDK_PTZAutoScan_Other

PTZ automatic scan operation

BOOL NET_SDK_PTZAutoScan_Other( LONG	lUserID,

LONG	lChannel,

DWORD	dwPTZAutoScanCmd );

Parameters

lUserID

[in] user ID lChannel

[in] channel number,starts from 0 dwPTZAutoScanCmd

[in] PTZ automatic scan command, list as follows:

value of

macro definition	 macro meaning definition

start PTZ_CMD_AUTO_SCAN_START30	automatic

scan

stop PTZ_CMD_AUTO_SCAN_STOP 31	automatic

scan

dwSpeed

[in] PTZ speed

Macro Definition	Meaning

PTZ_SPEED_1	1 PTZ_SPEED_2	2 PTZ_SPEED_3	3 PTZ_SPEED_4	4 PTZ_SPEED_5	5 PTZ_SPEED_6	6 PTZ_SPEED_7	7 PTZ_SPEED_8	8

Return Values

TURE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_PTZAutoScan

Android Interface

The corresponding Android interface

boolean long long int

);

PTZAutoScanOther( lUserID, lChannel, dwPTZAutoScanCmd

Client SDK Instructions

NET_SDK_PTZControl_3D

PTZ 3D control operation(need to start previewing image)

BOOL NET_SDK_PTZControl_3D( LONG	lLiveHandle, LONG	lChannel,

PTZ_3D_POINT_INFO       *pPtz3DInfo );

Parameters

lLiveHandle

[in] return value of NET_SDK_LivePlay lChannel

[in] channel number,starting with 0 *pPtz3DInfo

[in] information about PTZ 3D control,as follows PTZ_3D_POINT_INFO

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_PTZControl

Client SDK Instructions

NET_SDK_PTZControl_3D_Ex

PTZ 3D control operation(no need to start previewing image)

BOOL NET_SDK_PTZControl_3D_Ex( LONG	lUserID,

LONG	lChannel, PTZ_3D_POINT_INFO       *pPtz3DInfo );

Parameters

lUserID

[in] return value of NET_SDK_Login lChannel

[in] channel number,starting with 0 *pPtz3DInfo

[in] information about PTZ 3D control,as follows PTZ_3D_POINT_INFO

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_PTZControl

Client SDK Instructions

NET_SDK_GetPTZCameraType

get camera type(only support IPC)

BOOL NET_SDK_PTZPreset_Other( LONG	lUserID,

LONG	NET_SDK_CAMERA_TYPE *pCameraType );

Parameters

lUserID

[in] return value of userID NET_SDK_CAMERA_TYPE *pCameraType

[in] reture value of camera type,list as follows:

value of

camera type	camera meaning type

NET_SDK_NOT_SUPPORT_PTZ	0

NET_SDK_DOME_SUPPORT_PTZ 1

NET_SDK_SUPPORT_PTZ	2

NET_SDK_PTZ_END

bullet camera don't support ptz

bullet camera support ptz

dome camera support ptz

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

Each operation on PTZ is corresponding to each control code between device and PTZ,device sends control code to PTZ according to current decoder type and decoder address. If current decoder doesnot match PTZ ,reconfig decoder is necessary.

See Also

NET_SDK_Login

Client SDK Instructions

typedef enum __Channel_type__ {

E_NULL_CHL_TYPE, E_DIGITAL_CHL_TYPE,	//Digital

Channel E_ANALOG_CHL_TYPE, E_ALARMOUT_CHL_TYPE,

Output Channel E_SENSOR_CHL_TYPE,

//Analog Channel //Alarm

//Sensor Channel

}CHANNEL_TYPE;

typedef struct _net_sdk_channel_ptz {

unsigned int	dwChannel;//Channel No.(starting with zero)

CHANNEL_TYPE Type

unsigned char }NET_SDK_CHANNEL_PTZ;

eChanneltype;//Channel

resv[8];

NET_SDK_GetSupportPtzList

Get the channel of NVR in favor of PTZ list（only for N9000 device）

BOOL NET_SDK_GetSupportPtzList( LONG	lUserID,

int	listNum,

NET_SDK_CHANNEL_PTZ	*pOutChannelPtz, int	*returnListNum,

);

Parameters

lUserID

[in] the return value of NET_SDK_Login() listNum

[in] the number of NET_SDK_CHANNEL_PTZ supported by the request memory of pOutChannelPtz

*pOutChannelPtz

[out] Return PTZ information list *returnListNum

[out] the effective number of returning PTZ information list

Return Values

TRUE means success; FALSE means failure. To get error information, call NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_GetPTZConfig

Get the related configuration of PTZ (only for N9000).

BOOL NET_SDK_GetPTZConfig( LONG	lUserID,

LONG	lChannel, DWORD     dwCommand, LPVOID lpInBuffer, DWORD     dwInBufferSize, LPVOID lpOutBuffer,

DWORD	dwOutBufferSize, LPDWORD lpBytesReturned );

Parameters

lUserID

[in] the return value of NET_SDK_Login() lChannel

[in] channel number starts from 0. -1 means all channels.If the command don't need channel number, this parameter is invalid.

dwCommand

[in] device configuration command, see Configuration Commands lpInBuffer

[in] buffer pointer to sending data. it is can not be NULL . dwInBufferSize

[in] the buffer length of sending data (in byte). It is can not be 0. lpOutBuffer

[out] the buffer pointer to receiving data. dwOutBufferSize

[in] the buffer length of receiving data (in byte). It is can not be 0 lpBytesReturned

[out] the length pointer to the data actually received. it is can not be null

Remarks

Get prese	unsigned

Different functions has different structures and commands as shown below.

dwCommand

Macro Definition

dwCommand

Meanings

Sending Structure

Receiving Structu

DD_PTZ_CONFIG_PRESET

DD_PTZ_CONFIG_CRUISE

Get preset	NULL

Get cruise	NULL

DD_PTZ_PRESET_CON

DD_CH_CRUISE

DD_PTZ_CONFIG_CRUISE_POINTthe curise ts f int(cruiseIndex)DD_CRUISE_POINT_IN

See Also

NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_StartVoiceCom

start talkback

LONG NET_SDK_StartVoiceCom( LONG	lUserID,

BOOL	bNeedCBNoEncData, TALK_DATA_CALLBACK     fVoiceDataCallBack, void	* pUser

);

Parameters

lUserID

[in] return value of NET_SDK_Login bNeedCBNoEncData

[in] whether need to encode voice when local voice data callback. If FALSE is selected, the stream from the device will be decoded first and then it is called back.

fVoiceDataCallBack

[in] function of audio data callback PUser

[in] user data

Return Values

-1 means failure and other value is handle parameter of NET_SDK_StopVoiceCom. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_StopVoiceCom NET_SDK_Login

Client SDK Instructions

NET_SDK_SetVoiceComClientVolume

set client volume of talkback

BOOL NET_SDK_SetVoiceComClientVolume( LONG	lVoiceComHandle,

WORD	wVolume );

Parameters

lVoiceComHandle

[in] return value of NET_SDK_StartVoiceCom wVolume

[in] set volume,range[0,0xffff]

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

See Also

NET_SDK_StartVoiceCom

Client SDK Instructions

TALK_DATA_CALLBACK

data callback in talkback

void *TALK_DATA_CALLBACK(

LONG	lVoiceComHandle, char	*pRecvDataBuffer, DWORD	dwBufSize,

BYTE	byAudioFlag, void	* pUser

);

Parameters

lVoiceComHandle

[in] interface handle of talkback *pRecvDataBuffer

[in] buffer pointer to receive talkback data,received talkback data is PCM data without encoding

dwBufSize

[in] size of buffer area byAudioFlag

[in] audio mark,the only value is 1,and it means the talkback data from device

pUser

[in] pointer to user information

Return Values

None. To get error information, please call NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_GetAudioInfo

Get audio information.

BOOL NET_SDK_GetAudioInfo( LONG	lVoiceComHandle, void      *pAudioInfo,

LONG	infoLen );

Parameters

lVoiceComHandle

[in] the return value of NET_SDK_StartVoiceCom() pAudioInfo

[in] a pointer to audio information infoLen

[in] the length of audio information

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_ReleaseAudioEncoder      NET_SDK_EncodeAudi

oFrame

>

Client SDK Instructions

NET_SDK_StopVoiceCom

stop talkback or voice forward

BOOL NET_SDK_StopVoiceCom( LONG	lVoiceComHandle );

Parameters

lVoiceComHandle

[in] return value of NET_SDK_StartVoiceCom or NET_SDK_StartVoiceCom_MR

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_StartVoiceCom NET_SDK_StartVoiceCom_MR

Client SDK Instructions

NET_SDK_StartVoiceCom_MR

start voice forward function

LONG NET_SDK_StartVoiceCom_MR( LONG	lUserID,

TALK_DATA_CALLBACK      fVoiceDataCallBack, void	* pUser

);

Parameters

lUserID

[in] user ID bNeedNoEncodeData

[in] whether to encode the language data. If TRUE is selected, the stream from the device will be directly called back and the stream transfered to the device is encoded by SDK . If FALSE is selected, the stream from the deivce is decoded first and then called back; the stream transfered to the device is not encoded by SDK.

fVoiceDataCallBack

[in] return value of voice data callback function pUser

[in] user information

Return Values

-1 means failure and other value is return value of voice forward. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_VoiceComSendData

Client SDK Instructions

NET_SDK_VoiceComSendData

send data when voice forward

BOOL NET_SDK_VoiceComSendData( LONG	lVoiceComHandle, char	*pSendBuf,

DWORD	dwBufSize );

Parameters

lVoiceComHandle

[in] handle of voice component pSendBuf

[in] buffer pointer to send data, the talkback data in buffer area is PCM data without encoding

dwBufSize

[in] size of buffer area

Return Values

TURE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_StartVoiceCom_MR

Client SDK Instructions

NET_SDK_EncodeAudioFrame

encode audio

BOOL NET_SDK_EncodeAudioFrame( LONG	lEncodeHandle, unsigned char *pInBuffer, LONG	inLen, unsigned char * pOutBuffer , LONG	*pOutLen

);

Parameters

IEncodeHandle

[in] handle of encoding audio, the return value of NET_SDK_InitAudioEncoder()

*pInBuffer

[in] buffer area for input,get PCM audio data according to sample standard(sample frequency is 16000,16 bytes, signal channel),the standard size of input data is 1280 bytes.

inLen

[out] buffer area for output,data length after encoding pOutBuffer

[out] buffer area for output,the output data size after encoding is 80 bytes.

*pOutLen

[out] buffer area for output,output data length after encoding

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

It's set to mate with talkback & forward function.When client sends original audio data to device, firstly compress and encode audio through function of audio encoding,and then send to device. Client gets the compressed code stream,and then call NET_SDK_DecodeAudioFrame to decode data. Before calling encode and decode function initialization is needed, also when call function is finished,free resource.

See Also

NET_SDK_InitAudioEncoder

Client SDK Instructions

NET_SDK_InitAudioEncoder

initialize audio encoder

void* NET_SDK_InitAudioEncoder( void	*pAudioInfo,

LONG	infoLen );

Parameters

pAudioInfo

[in] pointer to audio information infoLen

[in] length of audio information

Return Values

-1 means failure and other value is handle of audio encoding. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_ReleaseAudioEncoder NET_SDK_EncodeAudioFr

ame

Client SDK Instructions

NET_SDK_ReleaseAudioEncoder

free audio encoding resource

void NET_SDK_ReleaseAudioEncoder( LONG *IEncodeHandle

);

Parameters

IEncodeHandle

[in] handle of audio encoding,return value of NET_SDK_InitAudioEncoder

Return Values

None. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_InitAudioEncoder

Client SDK Instructions

NET_SDK_DecodeAudioFrame

audio decode

BOOL NET_SDK_DecodeAudioFrame( LONG	lDecodeHandle, unsigned char	*pInBuffer, LONG	inLen, unsigned char	* pOutBuffer, LONG	*pOutLen

);

Parameters

IDecodeHandle

[in] audio encode handle,return value of NET_SDK_InitAudioDecoder()

*pInBuffer

[in] input buffer area,get PCM audio data according to sample standard(sampling frequency is in000,16 bits, signal channel),standard data size of input is 1280 bytes

inLen

[out] output buffer are,data length after encoding pOutBuffer

[out] output buffer area,output data size after encoding is 80 bytes

*pOutLen

[out] output buffer area,output data length after encoding

Return Values

TRUE means success and FALSE means failure. To get error information,please call NET_SDK_GetLastError

Remarks

It's set to mate with talkback & forward function.When client send original audio data to device, firstly compress and encode audio through function of audio encoding,and then send to device. Client gets the compressed code stream,and then call this interface to decode data. Before calling encode and decode function initialization is needed, also when call function is finished,free resource.

See Also

NET_SDK_InitAudioDecoder

Client SDK Instructions

NET_SDK_InitAudioDecoder

initialize audio decoder

LONG NET_SDK_InitAudioDecoder( void	*pAudioInfo,

LONG	infoLen );

Parameters

pAudioInfo

[in] pointer to audio information infoLen

[in] length of audio information

Return Values

-1 means failure and other value is handle of audio decoding. To get error information, please call NET_SDK_GetLastError

See Also

NET_SDK_ReleaseAudioDecoder NET_SDK_DecodeAudioFr

ame

Client SDK Instructions

NET_SDK_ReleaseAudioDecoder

free audio decoding resource

void NET_SDK_ReleaseAudioDecoder( LONG IDecodeHandle

);

Parameters

IDecodeHandle

[in] handle of audio decoding,return value of NET_SDK_InitAudioDecoder

Return Values

None. To get error information, please call NET_SDK_GetLastError

See Also

NET_SDK_InitAudioDecoder

Client SDK Instructions

NET_SDK_FormatDisk

format harddisk remotely(only support 3.0DVR)

LONG NET_SDK_FormatDisk( LONG	lUserID,

LONG	lDiskNumber );

Parameters

lUserID

[in] return value of NET_SDK_Login() lDiskNumber

[in] harddisk number,start from 0,0xff is valid to all harddisks(except read-only harddisk)

Return Values

-1 means failure and other value is parameter of function NET_SDK_CloseFormatHandle. To get error information,please call NET_SDK_GetLastError

Remarks

If network breaks in formation process,formation on device continues but client can't receive the state.

See Also

NET_SDK_CloseFormatHandle NET_SDK_GetFormatProgre

ss     NET_SDK_Login

Client SDK Instructions

NET_SDK_GetFormatProgress

get progress of formatting harddisk(only support 3.0DVR)

BOOL NET_SDK_GetFormatProgress( LONG	lFormatHandle,

LONG	*pCurrentFormatDisk, LONG	*pCurrentDiskPos, LONG	*pFormatStatic

);

Parameters

lFormatHandle

[in] handle of save formatting harddisk,return value of NET_SDK_FormatDisk

pCurrentFormatDisk

[out] pointer to save current formatting harddisk number,harddisk number starts from 0,-1 is the initial value

pCurrentDiskPos

[out] pointer to save current progress of formatting harddisk,progress range is 0-100

pFormatStatic

[out] pointer to save the state of formatting harddisk,0-formatting;1-finish formation; 2-formatting current harddisk makes mistake,formation can't continue,this error shows both in local and network harddisks; 3-formation of current harddisk can't start due to lost of network harddisk caused by network exception

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_FormatDisk

Client SDK Instructions

NET_SDK_CloseFormatHandle

close handle of formatting harddisk,and free resource(only support 3.0DVR)

BOOL NET_SDK_CloseFormatHandle( LONG	lFormatHandle

);

Parameters

lFormatHandle

[in] return value of NET_SDK_ FormatDisk()

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

See Also

NET_SDK_FormatDisk

Client SDK Instructions

NET_SDK_FindDisk

Get the HDD information of the device.

POINTERHANDLE NET_SDK_FindDisk( LONG	lUserID,

);

Parameters

lUserID

[in] the return value of NET_SDK_Login()

Return Values

Return to the handle of getting HDD information. The value is greater than 0, which means

success. NET_SDK_GetLastError

See Also

NET_SDK_GetNextDiskInfo NET_SDK_FindDiskClose NET_SDK_Login

Client SDK Instructions

NET_SDK_GetNextDiskInfo

Get the information of the HDD in order (call up serveral times until return failure).

BOOL NET_SDK_GetNextDiskInfo(

POINTERHANDLE lDiskHandle , NET_SDK_DISK_INFO *pDiskInfo,

);

Parameters

lDiskHandle

[in] the return value of NET_SDK_FindDisk pDiskInfo

[out] return to the HDD information

Return Values

The return value is greater than 0 which means success. NET_SDK_GetLastError

See Also

NET_SDK_FindDisk NET_SDK_FindDiskClose NET_SDK_Login

Client SDK Instructions

NET_SDK_FindDiskClose

Having finished getting the HDD information of the device, release resources.

BOOL NET_SDK_FindDiskClose( POINTERHANDLE lDiskHandle, );

Parameters

lDiskHandle

[in] the return value of NET_SDK_FindDisk

Return Values

The value is greater than 0, which means success. NET_SDK_GetLastError

See Also

NET_SDK_FindDisk	NET_SDK_GetNextDiskInfo

Client SDK Instructions

NET_SDK_ActiveDevice

active the IPC device

BOOL NET_SDK_ActiveDevice( char *pIp,

int	iPort,

char *password );

Parameters

pIp

[in] the IP address of the device to be activated(can be the second ip starting with 169.254)

iPort

[in] the http port of the device is to be activated password

[in] the password to activate the device

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

See Also

NET_SDK_GetUpgradeState      NET_SDK_GetUpgradePro gres

Client SDK Instructions

NET_SDK_Upgrade

remote upgrade

LONG NET_SDK_Upgrade( LONG	lUserID, char *sFileName

);

Parameters

lUserID

[in] return value of NET_SDK_Login wVolume

[in] directory of file upgrade(include file name)

Return Values

-1 means failure and other values are parameter of NET_SDK_GetUpgradeState. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_Login NET_SDK_CloseUpgradeHandle

NET_SDK_GetUpgradeState NET_SDK_GetUpgradeProgres

Client SDK Instructions

NET_SDK_GetUpgradeState

get state of remote upgrading

int NET_SDK_GetUpgradeState( LONG lUpgradeHandle

);

Parameters

lUpgradeHandle

[in] return value of NET_SDK_Upgrade

Return Values

-1 means failure. Other values' meanings are as follows: 1-finish upgrading

2-upgrading 3-failed

4-network breaks,unknown state

5-language version of upgrading file doesnot match

Get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_Upgrade

Client SDK Instructions

NET_SDK_GetUpgradeProgress

get progress of remote upgrading

int NET_SDK_GetUpgradeProgress( LONG lUpgradeHandle

);

Parameters

lUpgradeHandle

[in] return value of NET_SDK_Upgrade

Return Values

-1 means failure and 0-100 means progress of upgrading. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_Upgrade

Client SDK Instructions

NET_SDK_CloseUpgradeHandle

close handle of remote upgrade,and free resource

BOOL NET_SDK_CloseUpgradeHandle( LONG lUpgradeHandle

);

Parameters

lUpgradeHandle

[in] return value of NET_SDK_Upgrade()

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_Upgrade

Client SDK Instructions

NET_SDK_UpgradeIPC

remote upgrade IPC

LONG NET_SDK_UpgradeIPC( LONG	lUserID,

char *sFileName, unsigned int fileType );

Parameters

lUserID

[in] return value of NET_SDK_Login sFileName

[in] directory of file upgrade(include file name) fileType

[in] type of the upgrade

file; 0:software;1:kernel;2:Uboot;3:AIlib

Return Values

-1 means failure and other values are parameter of NET_SDK_GetUpgradeState. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_Login NET_SDK_CloseUpgradeHandle

NET_SDK_GetUpgradeState NET_SDK_GetUpgradeProgres

Client SDK Instructions

NET_SDK_FindDVRLog

find log information of device(not support IPC)

LONG NET_SDK_FindDVRLog( LONG	lUserID, DWORD	dwType,

DD_TIME            *lpStartTime, DD_TIME            *lpStopTime );

Parameters

lUserID

[in] return value of NET_SDK_Login() dwType

[in] type of log(examples see N9000_LOG_MAJOR_TYPE )

*lpStartTime

[in] the start time of file *lpStopTime

[in] the stop time of file

Return Values

-1 means failure and other values are part of parameters in function NET_DVR_FindNextLog. To get error information, please call NET_SDK_GetLastError

Remarks

This interface is used to search normal log information,and the maximum capacity is 2000.

See Also

NET_SDK_Login NET_SDK_FindNextLog NET_SDK_FindLog

Close

Client SDK Instructions

NET_SDK_FindNextLog

get log information one by one(not support IPC)

LONG NET_SDK_FindNextLog(

LONG	lLogHandle, LPNET_SDK_LOG        lpLogData );

Parameters

lLogHandle

[in] handle for finding log.return value of NET_SDK_FindDVRLog()

lpLogData

[out] pointer to store log information

Return Values

-1 means failure and other values are current state information. To get error information, please call NET_SDK_GetLastError

Remarks

Before calling this interface,call NET_SDK_FindDVRLog to get current handle for finding.

See Also

NET_SDK_FindDVRLog

Client SDK Instructions

NET_SDK_FindLogClose

free resource of finding log(not support IPC)

BOOL NET_SDK_FindLogClose( LONG lLogHandle

);

Parameters

lLogHandle

[in] handle of finding log,return value of NET_SDK_FindDVRLog()

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

See Also

NET_SDK_FindDVRLog

Client SDK Instructions

NET_SDK_FindEventInfo

find event info(only support 3.0DVR)

LONG NET_SDK_FindEvent( LONG	lUserID, DWORD	dwType

ULONGLONG	channlMask, DD_TIME             *lpStartTime, DD_TIME             *lpStopTime );

Parameters

lUserID

[in] returned value of NET_SDK_Login() dwType

[in] event type,refer to DD_EVENT_TYPE:

Type	Value

DD_EVENT_TYPE_MOTION	0x0001 DD_EVENT_TYPE_SENSOR	0x0002 DD_EVENT_TYPE_V_LOSS	0x0004 DD_EVENT_TYPE_V_COVER	0x0008

channlMask

[in] event happend in which channel, ((ULONGLONG)0x1 << N) N is search channel.

*lpStartTime

[in] event starting time *lpStopTime

[in] event ending time

Return Values

-1 means failure and other values are parameter of NET_SDK_FindNextEventInfo. To get error code, please call NET_SDK_GetLastError

See Also

NET_SDK_FindNextEventInfo NET_SDK_FindEventInfoClos

e

Client SDK Instructions

NET_SDK_FindNextEventInfo

find event info one by one(only support 3.0DVR)

LONG NET_SDK_FindNextEventInfo(

LONG	lEventHandle, LPNET_SDK_EVENT         lpEventData

);

Parameters

lEventHandle

[in] handle of searching event info, the return value of NET_SDK_FindEventInfo()

lpEventData

[out] pointer of saving event info

Return Values

-1 means failure and other values are event info. To get error code, please call NET_SDK_GetLastError

Remarks

Before call this interface to search event info, please call NET_SDK_FindEventInfo() to get the search handle.

See Also

NET_SDK_FindEventInfo NET_SDK_FindEventInfoClose

Client SDK Instructions

NET_SDK_FindEventInfoClose

Close searching event info,free resource(only support 3.0DVR)

BOOL NET_SDK_FindEventInfoClose( LONG	lEventHandle

);

Parameters

lEventHandle

[in] search handle

Return Values

TURE means success; FALSE means failure. To get error code,please call NET_SDK_GetLastError

See Also

NET_SDK_FindEventInfo NET_SDK_FindNextEventInfo

Client SDK Instructions

NET_SDK_GetDefaultVideoEffect

get default video effect

BOOL NET_SDK_GetDefaultVideoEffect( LONG	lUserID,

DWORD	*pBrightValue, DWORD	*pContrastValue, DWORD	*pSaturationValue, DWORD	*pHueValue

);

Parameters

lUserID

[in] user ID *pBrightValue

[in] pointer to brightness value *pContrastValue

[in] pointer to color contrast *pSaturationValue

[in] pointer to color saturation *pHueValue

[in] pointer to gray scale

Return Values

TURE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_GetVideoEffect NET_SDK_SaveVideoEffect NET

_SDK_SetVideoEffect

Client SDK Instructions

NET_SDK_SetConfigFile

import configuration file

BOOL NET_SDK_SetConfigFile( LONG	lUserID,

char	*sFileName );

Parameters

lUserID

[in] user ID,return value of NET_SDK_Login sFileName

[in] directory of saving configuration file(binary file)

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

See Also

NET_DVR_Login

Client SDK Instructions

NET_SDK_GetConfigFile

export configuration file

BOOL NET_SDK_GetConfigFile( LONG	lUserID,

char	*sFileName );

Parameters

lUserID

[in] user ID,return value of NET_SDK_Login() sFileName

[in] directory of storing configuration file(binary file)

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

See Also

NET_SDK_Login

Client SDK Instructions

NET_SDK_GetNvrRecordDays

Query the number of days of video exists on the NVR device（works for NVRonly）

BOOL NET_SDK_GetNvrRecordDays( LONG	lUserID, NET_SDK_NVR_DISKREC_DATE_ITEM*

pDiskRecDateInfo LONG lBuffSize, LONG* pDISKCount

);

Parameters

lUserID

[in] returned value of NET_SDK_Login() pDiskRecDateInfo

[out] video days information structure pointer for the NVR device

lBuffSize

[in] size of NET_SDK_NVR_DISKREC_DATE_ITEM structure

pDISKCount

[out] number of hard drives for the NVR

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Remarks

Each hard disk of the NVR device corresponds to a query result, with the video days format 2021-

11-27~2021-12-

07, indicating the existence of video on the hard d isk during this time period

Client SDK Instructions

NET_SDK_ShutDownDVR

close device

BOOL NET_SDK_ShutDownDVR( LONG	lUserID

);

Parameters

lUserID

[in] user ID,return value of NET_SDK_Login

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_Login

Client SDK Instructions

NET_SDK_RebootDVR

reboot device

BOOL NET_SDK_RebootDVR( LONG lUserID

);

Parameters

lUserID

[in] user ID,return value of NET_SDK_Login

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_Login

Client SDK Instructions

NET_SDK_ChangTime

modify system time of device

BOOL NET_SDK_ChangTime( LONG	lUserID, unsigned long     time );

Parameters

lUserID

[in] user ID time

[in] device system time

Return Values

TURE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_FormatTime

Transform inter time to format time

void NET_SDK_FormatTime( LONGLONG	intTime, DD_TIME            *pFormatTime );

Parameters

intTime

[in] inter time form NET_SDK_FRAME_INFO, from 1970-01-01,00:00:00,unit is microsecond

*pFormatTime

[in] DD_TIME format time

Return Values

None.This interface transforms the time parameter of NET_SDK_FRAME_INFO to DD_TIMEformat time. To get error code, please call NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_SaveVideoEffect

save setting of video effect

BOOL NET_SDK_SaveVideoEffect( LONG	lUserID,

LONG	lChannel, DWORD	dwBrightValue,

DWORD	dwContrastValue, DWORD	dwSaturationValue, DWORD	dwHueValue

);

Parameters

lUserID

[in] user ID lChannel

[in] channel number,start from 0 dwBrightValue

[in] value of brightness dwContrastValue

[in] value of constract dwSaturationValue

[in] value of saturation dwHueValue

[in] value of gray scale

Return Values

TURE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

See Also

NET_SDK_GetVideoEffect NET_SDK_GetDefaultVideoEffect

NET_SDK_SetVideoEffect

Client SDK Instructions

NET_SDK_ModifyDeviceNetInfo

Modify the network configuration of the device accoding to the matching MAC.

BOOL NET_SDK_ModifyDeviceNetInfo( NET_SDK_DEVICE_IP_INFO *pDeviceIPInfo

);

Parameters

pDeviceIPInfo

[in] the network configuration of the device

Return Values

-1 means failure; other values means the returned information value. To get error information, please call NET_SDK_GetLastError

See Also

Client SDK Instructions

NET_SDK_TransparentConfig

Transparent API protocol Interface

BOOL NET_SDK_TransparentConfig( LONG	lUserID,

char	*sendXML, char	*strUrl, LPVOID lpOutBuffer,

DWORD dwOutBufferSize, LPDWORD lpBytesReturned );

Parameters

lUserID

[in] return value of NET_SDK_Login sendXML

[in] xml contents in API strUrl

[in] URL of API。（IP and port are not included. eg：the origninal URL of API protocol ishttp://[:port]/PtzStopCruise[/channelId]. Here the URL is PtzStopCruise/channelId） .

lpOutBuffer

[out] the buffer pointer of receiving data dwOutBufferSize

[in]the buffer size of receiving data in bytes can not be zero. lpBytesReturned

[out]a pointer to the data length actually received can not be null.

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Remarks

This function can be used to send http command to the device eg.

Test the function with the sdk demo: click the "Config" tab, input the strUrl and sendXML,click the "Transparent" button, the lpOutBuffer will display under

Client SDK Instructions

NET_SDK_GetDeviceInfo

Get parameters of decoding device

BOOL NET_SDK_GetDeviceInfo( LONG	lUserID,

LPNET_SDK_DEVICEINFO pdecviceInfo );

Parameters

lUserID

[in] return value of NET_SDK_Login() pdecviceInfo

[out] information about device parameter

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_Login

Client SDK Instructions

NET_SDK_GetDeviceTypeName

get device type name

LONG NET_SDK_GetDeviceTypeName( LONG	lUserID,

char *pNameBuffer, long	bufferLen

);

Parameters

lUserID

[in] the return value of NET_SDK_Login() *pNameBuffer

[out] type name buffer of device bufferLen

[out] buffer length of device type name

Return Values

The returned value is the device type name. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_Login

Client SDK Instructions

NET_SDK_GetDeviceTime

get system time of device

BOOL NET_SDK_GetDeviceTime( LONG	lUserID,

DD_TIME      *pTime );

Parameters

lUserID

[in] user ID *pTime

[in] pointer of device system time

Return Values

TURE means success,FALSE means failure.If live frame hasn't arrived client in 50 milliseconds, return value is False.Please try to call this interface more times until return value is True. To get error information,please call NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_GetPTZCameraType

Get camera type

BOOL NET_SDK_GetPTZCameraType( LONG	lUserID, NET_SDK_CAMERA_TYPE *pCameraType , );

Parameters

lUserID

[in] the return value of NET_SDK_Login() pCameraType

[in] Camera type as shown below:

Return Values

TRUE means sucess; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Remarks

NET_SDK_CAMERA_TYPE is an example. Please refer to the corresponding type of the libary function.

See Also

NET SDK GetDVRConfig

Camera

Camera type	 type Meanings value

The camera

NET_SDK_NOT_SUPPORT_PTZ 0	don't support PTZ

NET_SDK_DOME_SUPPORT_PTZ1

The camera supports PTZ

NET_SDK_SUPPORT_PTZ	2

NET_SDK_PTZ_END

The camera supports PTZ

Client SDK Instructions

NET_SDK_GetAlarmStatus

Get the alarm information of the device.

BOOL NET_SDK_GetAlarmStatus( LONG lUserID,

LPVOID lpOutBuffer, DWORD dwOutBufferSize,

LPDWORD lpBytesReturned);

Parameters

lUserID

[in]the return value of NET_SDK_Login() lpOutBuffer

[out]buf of the alarm event output dwOutBufferSize

[in]the space of lpOutBuffer applied for lpBytesReturned

[out]the size of the valid data of the returned lpOutBuffer

typedef struct _alarm_status

{

unsigned int iSize;	//The length of the structure

int chanl;	//Alarm channel.The alarm unrelated to the channel is -1.

unsigned int alarmType;	//Alarm event NET_SDK_N9000_ALARM_TYPE }DD_ALARM_STATUS_INFO;

enum NET_SDK_N9000_ALARM_TYPE {

NET_SDK_N9000_ALARM_TYPE_RANGE_BEGIN, NET_SDK_N9000_ALARM_TYPE_MOTION=0x01,/////Motion

detection alarm input NET_SDK_N9000_ALARM_TYPE_SENSOR,/////Sensor alarm

input

NET_SDK_N9000_ALARM_TYPE_VLOSS,////Video loss alarm input

NET_SDK_N9000_ALARM_TYPE_FRONT_OFFLINE, //////Camera offline alarm

NET_SDK_N9000_ALARM_TYPE_OSC, removal detection alarm

NET_SDK_N9000_ALARM_TYPE_AVD, video signal detection alarm

NET_SDK_N9000_ALARM_TYPE_AVD_SCENE,

////Object

////Abnormal

////Abnormal

video signal detection alarm for scene changed,only for ipc

NET_SDK_N9000_ALARM_TYPE_AVD_CLARITY, ////Abnormal video signal detection alarm for video fuzzy,only for ipc

NET_SDK_N9000_ALARM_TYPE_AVD_COLOR,	////Abnormal video signal detection alarm for color problem,only for ipc

NET_SDK_N9000_ALARM_TYPE_PEA_TRIPWIRE, crossing detection alarm

NET_SDK_N9000_ALARM_TYPE_PEA_PERIMETER, Intrusion detection alarm

NET_SDK_N9000_ALARM_TYPE_VFD,

////Line

////Region

////Face

detection(only for ipc)

NET_SDK_N9000_ALARM_TYPE_CDD, density

NET_SDK_N9000_ALARM_TYPE_IPD, intrusion

NET_SDK_N9000_ALARM_TYPE_CPC, counting

NET_SDK_N9000_ALARM_TYPE_FACE_MATCH,

////Crowdy

////people

////people

////face comparation alarm(for nvr) NET_SDK_N9000_ALARM_TYPE_FACE_MATCH_FOR_IPC,

////face comparation alarm(for ipc) NET_SDK_N9000_ALARM_TYPE_PEA_FOR_IPC,

////Line crossing and region intrusion detection NET_SDK_N9000_ALARM_TYPE_TRAJECT,

////target tracking trajectory NET_SDK_N9000_ALARM_TYPE_VEHICE,

////license plate for ipc NET_SDK_N9000_ALARM_TYPE_AOIENTRY,

////enter region for ipc NET_SDK_N9000_ALARM_TYPE_AOILEAVE,

////leave region for ipc NET_SDK_N9000_ALARM_TYPE_PASSLINE,

////passline counting for ipc

NET_SDK_N9000_ALARM_TYPE_GPS_SPEED_OVER=0x21,//overspeed alarm

NET_SDK_N9000_ALARM_TYPE_GPS_CROSS_BOADER,//line crossing

NET_SDK_N9000_ALARM_TYPE_GPS_TEMPERATURE_OVER,//temperat ure alarm

NET_SDK_N9000_ALARM_TYPE_GPS_GSENSOR_X,//GSENSOR alarm

NET_SDK_N9000_ALARM_TYPE_GPS_GSENSOR_Y,

NET_SDK_N9000_ALARM_TYPE_GPS_GSENSOR_Z,

NET_SDK_N9000_ALARM_TYPE_EXCEPTION = 0x41,

NET_SDK_N9000_ALARM_TYPE_IP_CONFLICT, address conflict

NET_SDK_N9000_ALARM_TYPE_DISK_IO_ERROR,

/////IP

/////Disk IO

error

NET_SDK_N9000_ALARM_TYPE_DISK_FULL, /////Disk full

NET_SDK_N9000_ALARM_TYPE_RAID_SUBHEALTH, //Raid subhealth

NET_SDK_N9000_ALARM_TYPE_RAID_UNAVAILABLE, //Raid unavailabe

NET_SDK_N9000_ALARM_TYPE_ILLEIGAL_ACCESS, /////Illegal access

NET_SDK_N9000_ALARM_TYPE_NET_DISCONNECT, /////Network disconnection

NET_SDK_N9000_ALARM_TYPE_NO_DISK,	////No disk

NET_SDK_N9000_ALARM_TYPE_SIGNAL_SHELTER, //Signal obstruction

NET_SDK_N9000_ALARM_TYPE_HDD_PULL_OUT, //HDD pulled out

NET_SDK_N9000_ALARM_TYPE_ALARM_OUT = 0x51, /////Alarm output tpye.

NET_SDK_N9000_ALARM_TYPE_RANGE_END = 0xFF,////It is unable to exceed this value, or there will be a risk of error.

};

Return Values

TRUE means success, FALSE means failed. To get error information, please call NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_GetAlarmStatusEx

Get the alarm information(including the raid alarm) of the device.

BOOL NET_SDK_GetAlarmStatusEx( LONG lUserID,

LPVOID lpOutBuffer, DWORD dwOutBufferSize, LPDWORD lpBytesReturned,

int *exStructNum );

Parameters

lUserID

[in]the return value of NET_SDK_Login() lpOutBuffer

[out]buf of the alarm event output dwOutBufferSize

[in]the space of lpOutBuffer applied for lpBytesReturned

[out]the size of the valid data of the returned lpOutBuffer

exStructNum

[out]the number of the raid alarms

typedef struct _alarm_status_ex

{

unsigned int iSize;	//The length of the structure

int chanl;	//Alarm channel.The alarm unrelated to the channel is -1.

unsigned int alarmType;	//Alarm event NET_SDK_N9000_ALARM_TYPE, NET_SDK_N9000_ALARM_TYPE_RAID_SUBHEALTH,NET_ SDK_N9000_ALARM_TYPE_RAID_UNAVAILABLE

char alarmNode[32];	//The length of the structure

char recv[32];	//reserved. }DD_ALARM_STATUS_INFO_Ex;

enum NET_SDK_N9000_ALARM_TYPE {

NET_SDK_N9000_ALARM_TYPE_RANGE_BEGIN, NET_SDK_N9000_ALARM_TYPE_MOTION=0x01,/////Motion

detection alarm input NET_SDK_N9000_ALARM_TYPE_SENSOR,/////Sensor alarm

input

NET_SDK_N9000_ALARM_TYPE_VLOSS,////Video loss alarm input

NET_SDK_N9000_ALARM_TYPE_FRONT_OFFLINE, //////Camera offline alarm

NET_SDK_N9000_ALARM_TYPE_OSC, removal detection alarm

NET_SDK_N9000_ALARM_TYPE_AVD, video signal detection alarm

NET_SDK_N9000_ALARM_TYPE_AVD_SCENE,

////Object

////Abnormal

////Abnormal

video signal detection alarm for scene changed,only for

ipc

NET_SDK_N9000_ALARM_TYPE_AVD_CLARITY, ////Abnormal video signal detection alarm for video fuzzy,only for ipc

NET_SDK_N9000_ALARM_TYPE_AVD_COLOR,	////Abnormal video signal detection alarm for color problem,only for ipc

NET_SDK_N9000_ALARM_TYPE_PEA_TRIPWIRE, ////Line crossing detection alarm

NET_SDK_N9000_ALARM_TYPE_PEA_PERIMETER, ////Region Intrusion detection alarm

NET_SDK_N9000_ALARM_TYPE_VFD, detection(only for ipc)

NET_SDK_N9000_ALARM_TYPE_CDD, density

NET_SDK_N9000_ALARM_TYPE_IPD, intrusion

NET_SDK_N9000_ALARM_TYPE_CPC, counting

NET_SDK_N9000_ALARM_TYPE_FACE_MATCH,

////Face

////Crowdy

////people

////people

////face comparation alarm(for nvr) NET_SDK_N9000_ALARM_TYPE_FACE_MATCH_FOR_IPC,

////face comparation alarm(for ipc) NET_SDK_N9000_ALARM_TYPE_PEA_FOR_IPC,

////Line crossing and region intrusion detection NET_SDK_N9000_ALARM_TYPE_TRAJECT,

////target tracking trajectory NET_SDK_N9000_ALARM_TYPE_VEHICE,

////license plate for ipc NET_SDK_N9000_ALARM_TYPE_AOIENTRY,

////enter region for ipc NET_SDK_N9000_ALARM_TYPE_AOILEAVE,

////leave region for ipc NET_SDK_N9000_ALARM_TYPE_PASSLINE,

////passline counting for ipc

NET_SDK_N9000_ALARM_TYPE_GPS_SPEED_OVER=0x21,//overspeed alarm

NET_SDK_N9000_ALARM_TYPE_GPS_CROSS_BOADER,//line crossing

NET_SDK_N9000_ALARM_TYPE_GPS_TEMPERATURE_OVER,//temperat ure alarm

NET_SDK_N9000_ALARM_TYPE_GPS_GSENSOR_X,//GSENSOR alarm

NET_SDK_N9000_ALARM_TYPE_GPS_GSENSOR_Y, NET_SDK_N9000_ALARM_TYPE_GPS_GSENSOR_Z,

NET_SDK_N9000_ALARM_TYPE_EXCEPTION = 0x41,

NET_SDK_N9000_ALARM_TYPE_IP_CONFLICT, address conflict

NET_SDK_N9000_ALARM_TYPE_DISK_IO_ERROR, error

/////IP

/////Disk IO

NET_SDK_N9000_ALARM_TYPE_DISK_FULL, /////Disk full

NET_SDK_N9000_ALARM_TYPE_RAID_SUBHEALTH, //Raid subhealth

NET_SDK_N9000_ALARM_TYPE_RAID_UNAVAILABLE, //Raid unavailabe

NET_SDK_N9000_ALARM_TYPE_ILLEIGAL_ACCESS, /////Illegal access

NET_SDK_N9000_ALARM_TYPE_NET_DISCONNECT, /////Network disconnection

NET_SDK_N9000_ALARM_TYPE_NO_DISK, disk

NET_SDK_N9000_ALARM_TYPE_SIGNAL_SHELTER,

////No

//Signal

obstruction

NET_SDK_N9000_ALARM_TYPE_HDD_PULL_OUT, //HDD pulled out

NET_SDK_N9000_ALARM_TYPE_ALARM_OUT /////Alarm output tpye.

NET_SDK_N9000_ALARM_TYPE_RANGE_END

= 0x51,

= 0xFF,////It is

unable to exceed this value, or there will be a risk of error.

};

Return Values

TRUE means success, FALSE means failed. To get error information, please call NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_GetDeviceSupportFunction

get the functions of the IPC（only support IPC）。

BOOL NET_SDK_GetDeviceSupportFunction( LONG	lUserID, NET_SDK_DEV_SUPPORT* pDevSupport; );

Parameters

lUserID

[in] the return value of NET_SDK_Login() pDevSupport

[out] the functions of the IPC

Return Values

TRUE means success, FALSE means failed。To get error information, please callNET_SDK_GetLastError

See Also

NET_SDK_Login

Client SDK Instructions

NET_SDK_GetDeviceIPCInfo

get device management information

LONG NET_SDK_GetDeviceIPCInfo( LONG	lUserID,

NET_SDK_IPC_DEVICE_INFO *pDeviceIPCInfo, LONG	lBuffSize,

LONG	*pIPCCount );

Parameters

lUserID

[in] return value of NET_SDK_Login() *pDeviceIPCInfo

[in] added IPC structural buffer lBuffSize

[in] size of pDeviceIPCInfo(sizeof(NET_SDK_IPC_DEVICE_INFO)* the number of digital channel)

*pIPCCount

[in] the number of IPC has been added

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_Login

Client SDK Instructions

NET_SDK_GetDeviceCHStatus

Get the channel information of the NVR, like channel type, online or offline status, etc.

BOOL NET_SDK_GetDeviceCHStatus( LONG	lUserID,

NET_SDK_CH_DEVICE_STATUS* pDeviceCHStatus, LONG	lBuffSize,

LONG	*pCHCount );

Parameters

lUserID

[in] the return value of NET_SDK_Login() *pDeviceCHStatus

[in] the connection status of the current channel configured

lBuffSize [in] size of

pDeviceCHStatus(sizeof(NET_SDK_CH_DEVICE_STATU S) * support how many channels )

*pCHCount

[in] the actual numbers of the current channels

Return Values

TRUE means success; FALSE means failure. To get error information,please call NET_SDK_GetLastError

See Also

NET_SDK_Login

Client SDK Instructions

NET_SDK_SetDeviceManualAlarm

set device manual alarm

BOOL NET_SDK_SetDeviceManualAlarm( LONG	lUserID,

LONG	*pAramChannel, LONG	*pValue,

LONG	lAramChannelCount BOOL	bAlarmOpen

);

Parameters

lUserID

[in] return value of NET_SDK_Login() *pAramChannel

[in] List of alarm output channels,an array witch is needed to asign values,its size is lAramChannelCount

*pValue

[in] Alarm channel status(1 means enable alarm channel,0 means disable,if all 0 means all alarm channel is disable)

lAramChannelCount

[in] Number of alarm channels(return value of NET_SDK_DEVICEINFO::sensorOutputNum )

BOOL bAlarmOpen

[in] TRUE means open alarm,FALSE means close alarm

Return Values

TRUE means success; FALSE means failure. To get error information, please callNET_SDK_GetLastError

Client SDK Instructions

NET_SDK_GetIVMRuleConfig

Get IVM configuration information of the device(only support IPC).

BOOL NET_SDK_GetIVMRuleConfig( LONG	lUserID,

DWORD	dwCommand, LONG      lChannel, LPVOID lpOutBuffer,

DWORD	dwOutBufferSize, LPDWORD lpBytesReturned,

);

Parameters

lUserID

[in] the return value of NET_SDK_Login() dwCommand

[in] the configuration command of the device. Refer to configuration command. lChannel

[in] the channel number starts from 0 lpOutBuffer

[out] a pointer to the buffer of receiving data. dwOutBufferSize

[in] the buffer length receiving data (in bytes) can not be zero. lpBytesReturned

[out] a pointer to the data length actually received can not be NULL

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Remarks

confguration

schedule of

The structures and commands are as follows:

dwCommand

macro definition

dwCommand dwCommand

structu value	Meanings

IVM_RULE_VFD_CONFIG	0x0

IVM_RULE_VFD_TRIGGER_CONFIG 0x1

faceirecognitionNET_SDK_VFD_CON

alarm trigger

configuration of NET_SDK_VFD_TRIG face recognition

IVM_RULE_VFD_SCHEDULE_CONFIG0x2	face recognitionNET_DVR_SCHEDUL

IVM_RULE_AVD_CONFIG	0x3	abnormal video NET_SDK_AVD_CON signal detection

configuration

See Also

NET_SDK_SetDVRConfig       NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_SetIVMRuleConfig

Set the configuration information of the device.(only support IPC)

BOOL NET_SDK_SetIVMRuleConfig( LONG	lUserID,

DWORD	dwCommand, LONG      lChannel, LPVOID lpInBuffer, DWORD	dwInBufferSize );

Parameters

lUserID

[in] the return value of NET_SDK_Login() dwCommand

[in] the configuration command of the device. Refer to configuration command. lChannel

[in] the channel number starts from 0 lpInBuffer

[in] a pointer to the buffer of input data dwOutBufferSize

[in] the buffer length of the input data (in bytes)

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Remarks

confguration

schedule of

The structures and commands are as follows:

dwCommand

macro definition

dwCommand dwCommand

structu value	Meanings

IVM_RULE_VFD_CONFIG	0x0

IVM_RULE_VFD_TRIGGER_CONFIG 0x1

faceirecognitionNET_SDK_VFD_CON

alarm trigger

configuration of NET_SDK_VFD_TRIG face recognition

IVM_RULE_VFD_SCHEDULE_CONFIG0x2	face recognitionNET_DVR_SCHEDUL

abnormal video IVM_RULE_AVD_CONFIG	0x3	signal detection NET_SDK_AVD_CON

configuration

See Also

NET_SDK_GetIVMRuleConfig	NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_SmartSubscrib

Subscribe the smart alarm events.(only support IPC)

BOOL NET_SDK_SmartSubscrib( LONG	lUserID,

DWORD	dwCommand, LONG      lChannel,

NET_DVR_SUBSCRIBE_REPLY *pOutBuffer );

Parameters

lUserID

[in] the return value of NET_SDK_Login() dwCommand

[in] the configuration command of the device. Refer to configuration command.

lChannel

[in] the channel number starts from 0 pOutBuffer

[out] a pointer to the buffer of input data

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Remarks

The dwCommand is as follows:

enum definition	value meanings

NET_DVR_SMART_AVD	0x0

NET_DVR_SMART_VFD	0x1

NET_DVR_SMART_VFD_MATCH	0x2

NET_DVR_SMART_PEA	0x3

NET_DVR_SMART_OSC	0x4

NET_DVR_SMART_CPC(obsolete)	0x5

NET_DVR_SMART_CDD	0x6

NET_DVR_SMART_IPD	0x7

NET_DVR_SMART_VIHICLE	0x8

NET_IPC_SMART_AOIENTRY	0x9

NET_IPC_SMART_AOILEAVE	0xA

Abnormal video signal diagnosis

Face detection

Face comparison

Region intrusion

Object removal

People counting

Crowd density detection

People intrusion

Vehicle detection

Enter region

Leave region

Face match NET_DVR_SMART_VFD_MATCH_FAILED0xB failed, for

stranger

NET_IPC_SMART_PASSLINE	0xC pass line

See Also

NET_SDK_UnSmartSubscrib	NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_UnSmartSubscrib

Cancel the subscription of smart alarm events(only support IPC).

BOOL NET_SDK_UnSmartSubscrib( LONG	lUserID,

DWORD	dwCommand, LONG      lChannel,

char *pInServerAddress, int *dwResult

);

Parameters

lUserID

[in] the return value of NET_SDK_Login() dwCommand

[in] the configuration command of the device. Refer to configuration command.

lChannel

[in]the channel number starts from 0 pInServerAddress

[in] a pointer to the buffer of input data dwResult

[out] the length of output data

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Remarks

The structures and commands are as follows:

enum definition	value meanings

NET_DVR_SMART_AVD	0x0

NET_DVR_SMART_VFD	0x1

NET_DVR_SMART_VFD_MATCH	0x2

NET_DVR_SMART_PEA	0x3

NET_DVR_SMART_OSC	0x4

NET_DVR_SMART_CPC(obsolete)	0x5

NET_DVR_SMART_CDD	0x6

NET_DVR_SMART_IPD	0x7

NET_DVR_SMART_VIHICLE	0x8

NET_IPC_SMART_AOIENTRY	0x9

NET_IPC_SMART_AOILEAVE	0xA

Abnormal video signal diagnosis

Face detection

Face comparison

Region intrusion

Object removal

People counting

Crowd density detection

People intrusion

Vehicle detection

Enter region

Leave region

Face match NET_DVR_SMART_VFD_MATCH_FAILED0xB failed, for

stranger

NET_IPC_SMART_PASSLINE	0xC pass line

See Also

NET_SDK_SmartSubscrib	NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_SetSubscribCallBack

Set the report and callback of the smart alarm events.

BOOL NET_SDK_SetSubscribCallBack( SUBSCRIBE_CALLBACK	fSubscribCallBack, void	*pUser,

);

Parameters

fSubscribCallBack

[in] callback function pUser

[in] client data

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Client SDK Instructions

SUBSCRIBE_CALLBACK

When the subscribed smart alarm event happens, the uploading analytic data is called back

void *SUBSCRIBE_CALLBACK( LONG	lUserID, DWORD	dwCommand, char	*pBuf, DWORD	dwBufLen, void	*pUser

);

Parameters

lUserID

[in] the return value of NET_SDK_Login() dwCommand

[in] the configuration command of the device. Refer to configuration command. pBuf

[in] the return data （different data types have different structures） dwBufLen

[out] the reture data length （different data types have different data length） pUser

[in] user data

Remarks

The dwCommand is as follows:

dwCommand enum	value	mean

NET_SDK_SMART_EVENT_TYPE_AVD

NET_SDK_SMART_EVENT_TYPE_VFD

NET_SDK_SMART_EVENT_TYPE_FACE_MATCH

6	Abnoral video sign

12	Face detection

16	Face comparison

NET_SDK_SMART_EVENT_TYPE_FACE_MATCH_FOR_IPC17	Face comparison

NET_SDK_SMART_EVENT_TYPE_PEA_FOR_IPC

NET_SDK_SMART_EVENT_TYPE_VEHICE

NET_SDK_SMART_EVENT_TYPE_PASSLINE

18	Line crossing and

20	Vehicle number d

23	pass line

See Also

NET_SDK_UnSmartSubscrib NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_FaceMatchOperate

(only support N9000, not support IPC, IPC refer to NET_SDK_TransparentConfig)The relevant operation of face comparison: whether to support face comparison, face picture database management, face match alarm, getting the data of target.

BOOL NET_SDK_FaceMatchOperate( LONG	lUserID,

DWORD	dwCommand, LPVOID lpInBuffer, DWORD	dwInBufferSize, LPVOID lpOutBuffer, DWORD	dwOutBufferSize, LPDWORD lpBytesReturned,

);

Parameters

lUserID

[in] the return value of NET_SDK_Login() dwCommand

[in] Command types refer to configuration command lpInBuffer

[in] a buffer pointer to send data dwInBufferSize

[in] the buffer size of sending data (in bytes) lpOutBuffer

[out] a buffer pointer to receive data dwOutBufferSize

[in] the buffer size of receiving data (in bytes) lpBytesReturned

[out] the data length pointer that actually receives can not larger than dwOutBufferSize

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Remarks

Different functions have different structure and commands as shown below.

NULL

dwCommand	dwCommanddwCommand

Macro Definition	Value

NET_SDK_GET_FACE_MATCH_SUPPORT	0x01

Definition

Whether to support face comparison or not

NET_SDK_GET_FACE_INFO_GROUP_LIST	0x02	Get group list NULL

NET_SDK_ADD_FACE_INFO_GROUP	0x03 NET_SDK_SET_FACE_INFO_GROUP	0x04 NET_SDK_DEL_FACE_INFO_GROUP	0x05

NET_SDK_GET_FACE_INFO_LIST	0x06

NET_SDK_ADD_FACE_INFO	0x07

NET_SDK_SET_FACE_INFO	0x08

NET_SDK_DEL_FACE_INFO	0x09

NET_SDK_GET_FACE_MATCH_ALARM	0x0A

NET_SDK_SET_FACE_MATCH_ALARM	0x0B

NET_SDK_GET_FACE_INFO_IMG	0x0C

NET_SDK_SEARCH_IMAGE_BY_IMG	0x0D

Add group Edit group Delete group

Get target face list

Add target face

Edit face information

Delete face picture

Get face match alarm linkage

Set face match alarm linkage

Get target face data

Search image by image

NET_SDK_ NET_SDK_ NET_SDK_

NET_SDK_

NET_SDK_

NET_SDK_

NET_SDK_

NULL

NET_SDK_

NET_SDK_

NET_SDK_

NET_SDK_SEARCH_CH_SNAP_FACE_IMG_LIST0x0E

Search faces	NET_SDK_ of the camera H

NET_SDK_SEARCH_CH_SNAP_FACE_IMG	0x0F

Search the face information of the camera

NET_SDK_

Client SDK Instructions

NET_SDK_SetFishEyeAdjust

(windows only)Set the fisheye correction mode，

This interface is called to enter and exit fisheye correct ion mode both, only in single window mode.

BOOL NET_SDK_SetFishEyeAdjust( POINTERHANDLE lPlayHandle, FISHEYE_MODE fishEyeMode

);

Parameters

lPlayHandle

[in] the handel of play video fishEyeMode

[in]

fisheye mode:Installation mode + correction mode, The specifi c defined values are as follows:

typedef enum {

FISHEYE_ORIGNAL = 0, //Original mode ,That is, the fisheye map in the top / wall / botto m mount mode, equivalent to quitting the fisheye

correction mod

FISHEYE_ROOF = 0x0100, //Top (suction top) FISHEYE_ROOF_360, //Top-

mounted 360 rectangular expansion panorama + independent sub-screen;Sub-

frames and rectangularexpansion panorama suppor t doubling and moving operations，

Rectangular expansion panorama also supports le ft and right start point movement

FISHEYE_ROOF_2x180, //Two associated 180 re ctangular expansion screens of top-mounted，

At any moment, the two sub-

Windows constitute 360 panoramic views, also kn own as the "double panorama",Both rectangular e xpansion pictures support the left and right move ment start point operation, and linkage with each other;

FISHEYE_ROOF_FISH_3PTZ, //Top-mounted original image + 3 independent sub-images，Both sub-

frames and frames in the original image support d oubling and moving，

The original image also supports rotation change start point operations;

FISHEYE_ROOF_FISH_4PTZ, //Top-mounted original image + 4 independent subimages， Both sub-

frames and frames in the original image support d oubling and moving，

The original image also supports rotation change start point operations;

FISHEYE_ROOF_360_6PTZ, //Top-

mounted 360 rectangular expansion panorama +6 independent sub-screens，Sub-

frames and rectangular expansion panorama suppor t doubling and moving operations，

Rectangular expansion panorama also supports le

ft and right start point movement FISHEYE_ROOF_FISH_8PTZ, //Top-

mounted original image + 8 independent sub-images，Both sub-

frames and frames in the original image support d oubling and moving，

The original image also supports rotation change start point operations;

FISHEYE_WALL = 0x0200, //Wall-mounted FISHEYE_WALL_180, //The 180 wall-

mounted panorama, from left to right 180 rect angular expansion panorama, 180 rectangular expansion panorama support up and down mo vement operation, change the vertical viewing angle;

FISHEYE_WALL_180_3PTZ, //180 Rectangle panoramic panorama of wall-

mounted+ 3independent sub-frames，sub-

frames and rectangular panoramic panorama s upport doubling and moving operations， Rectangular expansion panorama supports up and down movement,to change the vertical per spective

FISHEYE_WALL_180_4PTZ, //180 Rectangle panoramic panorama of wall-

mounted+4 independent sub-frames，sub-

frames and rectangular panoramic panorama s upport doubling and moving operations， Rectangular expansion panorama supports up and down movement,to change the vertical per spective

FISHEYE_WALL_180_8PTZ, //180 Rectangl e panoramic panorama of wall-

mounted+ 8 independent sub-frames，sub-

frames and rectangular panoramic panorama s upport doubling and moving operations， Rectangular expansion panorama supports up and down movement,to change the vertical per spective

FISHEYE_DESKTOP = 0x0300, //Bottom-mounted(desktop)

FISHEYE_DESKTOP_360, // 360 rectangula r expansion panorama of bottom-mounted+independent sub-frames;sub-

frames and rectangular expansion panorama sup port doubling and moving operations， Rectangular expansion panorama also support s left and right start point movement

FISHEYE_DESKTOP_2x180, //Two associate d 180 rectangular expansion screens of bottom-mounted，At any moment, the two sub-

Windows constitute 360 panoramic views, also known as the "double panorama",Both rectan

gular expansion pictures support the left and ri ght movement start point operation, and linka ge with each other;

FISHEYE_DESKTOP_FISH_3PTZ, //Bottom-mounted original image + 3 independent sub-images，Both sub-

frames and frames in the original image suppo rt doubling and moving，

The original image also supports rotation chan

ge start point operations; FISHEYE_DESKTOP_FISH_4PTZ, //Bottom-

mounted original image + 4 independent sub-images，Both sub-

frames and frames in the original image suppo rt doubling and moving，

The original image also supports rotation chan ge start point operations;

FISHEYE_DESKTOP_360_6PTZ, //Bottom-mounted 360 rectangular expansion panorama

+6 independent sub-screens，Sub-

frames and rectangular expansion panorama sup port doubling and moving operations， Rectangular expansion panorama also support s left and right start point movement

FISHEYE_DESKTOP_FISH_8PTZ, //Bottom-mounted original image + 8 independent sub-images，Both sub-

frames and frames in the original image suppo rt doubling and moving，

The original image also supports rotation chan ge start point operations;

}FISHEYE_MODE;

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_FishEyeAdjustFocus

(windows only)set focus，

to identify which segmentation region of the current ac tion acts on fisheye correction

BOOL NET_SDK_FishEyeAdjustFocus( POINTERHANDLE lPlayHandle,

int nX, int nY

);

Parameters

lPlayHandle

[in] the handel of play video nX

[in] the X coordinate value of current focus,relative to the coordinate system of current play window

nY

[in] the Y coordinate value of current focus,relative to the coordinate system of current play window

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_FishEyeAdjustFocusEx

(windows only)set focus，

to identify which segmentation region of the current ac tion acts on fisheye correction

BOOL NET_SDK_FishEyeAdjustFocusEx( POINTERHANDLE lPlayHandle,

int nX, int nY,

int &nIndex );

Parameters

lPlayHandle

[in] the handel of play video nX

[in] the X coordinate value of current focus,relative to the coordinate system of current play window

nY

[in] the Y coordinate value of current focus,relative to the coordinate system of current play window

nIndex

[in] fccus index

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_FishEyeAdjustMove

(windows only)E-cloud platform movement,the segmentation belong to the e-cloud platform can be moved only when it's under the fisheye correction mode

BOOL NET_SDK_FishEyeAdjustMove( POINTERHANDLE lPlayHandle,

int nMoveX, int nMoveY

);

Parameters

lPlayHandle

[in] the handel of play video nMoveX

[in] the left mouse button drags horizontally against the X axis of the starting point,positive to the right and negative to the left,with the starting point as the origin

nMoveY

[in] the left mouse button drags vertically against the Y axis of the starting point,positive up and negative down,with the starting point as the origin

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_FishEyeAdjustGetArea

(windows only)obtain the correction area location of the current focus

BOOL NET_SDK_FishEyeAdjustGetArea( POINTERHANDLE lPlayHandle,

RECT &AreaRect );

Parameters

lPlayHandle

[in] the handel of play video AreaRect

[in] the correction area location of the current focus,relative to the current play window coordinate system

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Client SDK Instructions

NET_SDK_FishEyeAdjustZoom

(windows only)E-cloud platform amplification,the division belongs to the e-cloud platform can be amplified only when it's into the fisheye correction mode

BOOL NET_SDK_FishEyeAdjustZoom( POINTERHANDLE lPlayHandle,

const RECT &ZoomRect );

Parameters

lPlayHandle

[in] the handel of play video ZoomRect

[in] Specifies the area location information to zoom in,relative to the current play window coordinate system

Return Values

TRUE means success; FALSE means failure. To get error information, please call NET_SDK_GetLastError

Client SDK Instructions

DD_ACCOUNT_CONFIG

struct of account configuration

struct _dd_account_config{ unsigned long	iSize; unsigned long	enable; unsigned long	bindMAC; unsigned long	group; char	MAC [8];

char	name [DD_MAX_USER_NAME_BUF_LEN]; char	password [DD_MAX_PASSWORD_BUF_LEN]; unsigned char	logSearch;

unsigned char	systemSetup; unsigned char	fileManagement; unsigned char	diskManagement; unsigned char	remoteLogin; unsigned char	twoWayAudio; unsigned char	systemMaintain;

unsigned char	OnlineUserManagement; unsigned char	shutdown;

unsigned char	alarmOutCtrl; unsigned char	netAlarm; unsigned char	netSerialCtrl; unsigned char	authLive; unsigned char	authRecord; unsigned char	authPlayback; unsigned char	authBackup; unsigned char	authPTZ; unsigned char	netAuthView; unsigned char	netauthRecord; unsigned char	netauthPlayback; unsigned char	netauthBackup; unsigned char	netauthPTZ; unsigned char	recv[2];

unsigned char	authLiveCH [DD_MAX_CAMERA_NUM_BYTE_LEN]; unsigned char	authRecordCH [DD_MAX_CAMERA_NUM_BYTE_LEN]; unsigned char	authPlaybackCH [DD_MAX_CAMERA_NUM_BYTE_LEN]; unsigned char	authBackupCH [DD_MAX_CAMERA_NUM_BYTE_LEN]; unsigned char	authPTZCH [DD_MAX_CAMERA_NUM_BYTE_LEN]; unsigned char	netAuthViewCH [DD_MAX_CAMERA_NUM_BYTE_LEN]; unsigned char	netAuthRecordCH [DD_MAX_CAMERA_NUM_BYTE_LEN]; unsigned char	netAuthPlaybackCH [DD_MAX_CAMERA_NUM_BYTE_LEN]; unsigned char	netAuthBackupCH [DD_MAX_CAMERA_NUM_BYTE_LEN]; unsigned char	netAuthPTZCH [DD_MAX_CAMERA_NUM_BYTE_LEN]; }DD_ACCOUNT_CONFIG;

Members

iSize

size of the struct enable

whether to use the account bindMAC

whether to bind MAC group

the group belonged to,refer to DD_USER_GROUP:

Group Name	Value	Description

DD_USER_GROUP_NONE	0x00

DD_USER_GROUP_ADMIN	0x01 administrator,have all rights

DD_USER_GROUP_ADVANCE0x02

advanced user,default rights:basic,record,config,playback,backup,data management,disk management,PTZ control,remote login and all channels rights

normal user,default DD_USER_GROUP_NORMAL 0x03 rights:basic,record,playback,backup,PTZ

control,remote login and all channels rights

MAC [8] binded MAC

name [DD_MAX_USER_NAME_BUF_LEN] user name

password [DD_MAX_PASSWORD_BUF_LEN] password

logSearch

limit of log search systemSetup

system configuration fileManagement

file management diskManagement

disc management remoteLogin

remote login twoWayAudio

audio talkback systemMaintain

system maintain OnlineUserManagement

online user management shutdown

shutdown or reboot alarmOutCtrl

alarm output control netAlarm

network alarm netSerialCtrl

network serial port control authLive

live preview authRecord

local record authPlayback

local search playback authBackup

local backup authPTZ

local PTZ netAuthView

local control

netauthRecord remote record

netauthPlayback remote live playback

netauthBackup remote backup

netauthPTZ remote PTZ

recv[2] reserved bytes

authLiveCH [DD_MAX_CAMERA_NUM_BYTE_LEN] live preview channel

authRecordCH [DD_MAX_CAMERA_NUM_BYTE_LEN] local record manually

authPlaybackCH [DD_MAX_CAMERA_NUM_BYTE_LEN] local search and playback

authBackupCH [DD_MAX_CAMERA_NUM_BYTE_LEN] local backup

authPTZCH [DD_MAX_CAMERA_NUM_BYTE_LEN] local PTZ control

netAuthViewCH [DD_MAX_CAMERA_NUM_BYTE_LEN] remote live preview

netAuthRecordCH [DD_MAX_CAMERA_NUM_BYTE_LEN] remote record manually

netAuthPlaybackCH [DD_MAX_CAMERA_NUM_BYTE_LEN] remote playback

netAuthBackupCH [DD_MAX_CAMERA_NUM_BYTE_LEN] remote backup

netAuthPTZCH [DD_MAX_CAMERA_NUM_BYTE_LEN] remote PTZ control

Client SDK Instructions

DD_AREA

area struct

struct _dd_area_{ unsigned short x; unsigned short y; unsigned short cx; unsigned short cy; }DD_AREA;

Members

x

abscissa,range 0-99 y

ordinate,range 0-99 cx

width,range 1-100 cy

height,range 1-100

Notice: x+cx <= 100, y+cy <=100

Client SDK Instructions

DD_AUTO_REPORT

Passively receive the struct of DVR register.

struct _dd_auto_report_{ unsigned long bUse; char	host[256]; unsigned long dwPort; unsigned long ID; }DD_AUTO_REPORT;

Members

bUse

Whether to enable auto report register function host[256]

server address of register platform dwPort

server port of register platform ID

assigned register ID

Client SDK Instructions

DD_BASIC_CONFIG

struct of basic configuration information

struct _dd_basic_config_{ unsigned long iSize; unsigned long videoFormat; unsigned long videoOut;

unsigned long videoOutResolution; unsigned long VGARefresh; unsigned long screensaver; unsigned long deviceLanguage; unsigned long passwordCheck; unsigned long RecycleRecord; unsigned long videoFormatMask; unsigned long videoOutMask;

unsigned long videoOutResolutionMask; unsigned long languageMask; }DD_BASIC_CONFIG;

Members

iSize

size of the struct videoFormat

video format, refer to DD_VIDEO_FORMAT:

Type	Value

DD_VIDEO_FORMAT_NTSC       0x01 DD_VIDEO_FORMAT_PAL          0x02

videoOut

video output device(reserved) videoOutResolution

video output resolution,refer to DD_VGA_RESOLUTION:

Type	Value

DD_VGA_640X480	0x0001 DD_VGA_720X480	0x0002 DD_VGA_720X576	0x0004 DD_VGA_800X600	0x0008 DD_VGA_1024X768	0x0010 DD_VGA_1280X960	0x0020 DD_VGA_1280X1024	0x0040 DD_VGA_1920X1080	0x0080

VGARefresh

VGA refresh rate(reserved) screensaver

screensaver time(0 means close) deviceLanguage

device language passwordCheck

whether to open password check RecycleRecord

whether permit overlap record videoFormatMask

supportive video format mask(read only) videoOutMask

supportive video output device mask(read only) videoOutResolutionMask

supportive video output device resolution mask(read only)

languageMask

language mask group supported by device(read only),refer to the list below:

类型	对应值

LANGUAGE_ENGLISH LANGUAGE_CHINESE_S LANGUAGE_CHINESE_B LANGUAGE_PORTUGUESE LANGUAGE_GREECE LANGUAGE_SPAISH LANGUAGE_RUSSIAN LANGUAGE_NORWAY LANGUAGE_TURKEY LANGUAGE_ITALY LANGUAGE_CZECH LANGUAGE_GERMAN LANGUAGE_HEBREW LANGUAGE_JAPANESE LANGUAGE_FRENCH LANGUAGE_POLISH LANGUAGE_BULGARIAN LANGUAGE_INDONESIA LANGUAGE_RUSSIAN_D LANGUAGE_THAI LANGUAGE_HUNGARY

LANGUAGE_LITHUANIA

0x0000001 0x0000002 0x0000004 0x0000008 0x0000010 0x0000020 0x0000040 0x0000080 0x0000100 0x0000200 0x0000400 0x0000800 0x0001000 0x0002000 0x0004000 0x0008000 0x0010000 0x0020000 0x0040000 0x0080000 0x0100000

0x0200000

Client SDK Instructions

DD_BUZZER_CONFIG

struct of buzzer configuration

struct _dd_buzzer_config_{ unsigned char

unsigned char unsigned short }DD_BUZZER_CONFIG;

enable; recv; holdTime;

Members

enable

buzzer enable switch recv

reserved bytes holdTime

delay time

Client SDK Instructions

DD_CHANNEL_CONFIG

struct of channel configuration information

struct _dd_channel_config_{ unsigned long	iSize; unsigned long	hide;

char	name [DD_MAX_CAMERA_NAME_BUF_LEN]; }DD_CHANNEL_CONFIG;

Members

iSize

size of the struct hide

whether hide channel

name [DD_MAX_CAMERA_NAME_BUF_LEN] channel name

Client SDK Instructions

DD_CRUISE_POINT_INFO

struct of setting cruise position information.

struct _dd_cruise_point_info{ unsigned long

unsigned long unsigned long

}DD_CRUISE_POINT_INFO;

presetIndex; dwellSpeed; dwellTime;

Members

presetIndex

index of cruise position(1-128). dwellSpeed

speed of cruise(1-8). dwellTime

seconds of cruise .

Client SDK Instructions

DD_DATE

struct of date configuration infomation of device.

struct _dd_date_{ unsigned char	mday; unsigned char	month; unsigned short	year; }DD_DATE, *LP_DD_DATE;

Members

mday

day of month, range(1-31). month

month, range(1-12). year

current solar year.

Client SDK Instructions

DD_DATE_SCHEDULE

struct of data schedule

struct _dd_date_schedule_{

unsigned long long	hour [24]; }DD_DATE_SCHEDULE;

Members

hour [24]

data schedule formation,24 stands for 24 hours format,each position of unsigned long long stands for each minute's state

Client SDK Instructions

DD_DATE_TIME_CONFIG

struct of data and time schedule configuration

struct _dd_date_time_config{ unsigned long	iSize; unsigned char	dateFormat; unsigned char	timeFormat; unsigned char	timeZone; unsigned char	enableNTP; unsigned short      ntpPort; unsigned short      recv;

char ntpServerAddr[DD_MAX_URL_BUF_LEN];

}DD_DATE_TIME_CONFIG;

Members

iSize

size of the struct dateFormat

data format, refer to DD_DATE_MODE:

Type

DD_DATE_MODE_YMD DD_DATE_MODE_MDY

DD_DATE_MODE_DMY

Value	Description

0x01       YMD format 0x02       MDY format

0x03	DMY format

timeFormat

time format,refer to the list below:

Type

TIME_MODE_12

TIME_MODE_24

Value	Description

0x01	12 hours

0x02	24 hours

timeZone

time zone, refer to DD_TIME_ZOME_NAME:

Type	Value

DD_TIME_ZONE_GMT_D12	0 DD_TIME_ZONE_GMT_D11	1 DD_TIME_ZONE_GMT_D10	2 DD_TIME_ZONE_GMT_D9	3 DD_TIME_ZONE_GMT_D8	4 DD_TIME_ZONE_GMT_D7	5 DD_TIME_ZONE_GMT_D6	6 DD_TIME_ZONE_GMT_D5	7 DD_TIME_ZONE_GMT_D4_30	8 DD_TIME_ZONE_GMT_D4	9 DD_TIME_ZONE_GMT_D3_30	10 DD_TIME_ZONE_GMT_D3	11 DD_TIME_ZONE_GMT_D2	12 DD_TIME_ZONE_GMT_D1	13 DD_TIME_ZONE_GMT	14 DD_TIME_ZONE_GMT_A1	15 DD_TIME_ZONE_GMT_A2	16 DD_TIME_ZONE_GMT_A3	17 DD_TIME_ZONE_GMT_A3_30	18 DD_TIME_ZONE_GMT_A4	19 DD_TIME_ZONE_GMT_A4_30	20 DD_TIME_ZONE_GMT_A5	21 DD_TIME_ZONE_GMT_A5_30	22 DD_TIME_ZONE_GMT_A5_45	23 DD_TIME_ZONE_GMT_A6	24 DD_TIME_ZONE_GMT_A6_30	25

DD_TIME_ZONE_GMT_A7	26 DD_TIME_ZONE_GMT_A8	27 DD_TIME_ZONE_GMT_A9	28 DD_TIME_ZONE_GMT_A9_30	29 DD_TIME_ZONE_GMT_A10	30 DD_TIME_ZONE_GMT_A11	31 DD_TIME_ZONE_GMT_A12	32 DD_TIME_ZONE_GMT_A13	33

enableNTP

whether open NTP synchronization service ntpPort

NTP port ntpServerAddr[DD_MAX_URL_BUF_LEN]

NTP service address

Client SDK Instructions

DD_DAYLIGHT_INFO

struct of daylight saving time information

struct _dd_daylight_info_{ unsigned char	InMonth; unsigned char	InMday; unsigned char	OutMonth; unsigned char	OutMday;

unsigned char	InWeekIndex; unsigned char	InWday; unsigned char	OutWeekIndex; unsigned char	OutWday; unsigned short	InYear; unsigned short	OutYear; unsigned short	enable; unsigned short	type; unsigned long	InSecond; unsigned long	OutSecond; unsigned long	offSet; }DD_DAYLIGHT_INFO;

Members

InMonth

which month to enter DST InMday

which day to enter DST(data mode is valid) OutMonth

which month to exit DST OutMday

which day to exit DST(data mode is valid) InWeekIndex

which week to enter DST(week mode is valid) InWday

which weekday to enter DST(week mode is valid) OutWeekIndex

which week to exit DST(week mode is valid) OutWday

which weekday to exit DST(week mode is valid) InYear

which year to enter DST,reserved due to align the struct

OutYear

which year to exit DST,reserved due to align the struct enable

whether enable DST function type

DST setting modern: week or data mode InSecond

second offset in one day of DST(0-86399),it can switch to be hour and minute and second

OutSecond

second offset out of one day of DST(0-86399),it can switch to be hour and minute and second

offSet

offset second in DST(0-86399)

Client SDK Instructions

DD_DDNS_CONFIG

struct of DDNS configuration

struct _dd_ddns_config_{ unsigned long	iSize; unsigned short	enable;

unsigned short	interval; unsigned long	useDDNSServer; unsigned long	userHostDomain; char	userName

[DD_MAX_DDNS_ACCOUNT_BUF_LEN]; char	password

[DD_MAX_PASSWORD_BUF_LEN];

char	hostDomain [DD_MAX_URL_BUF_LEN]; }DD_DDNS_CONFIG;

Members

iSize

size of the struct enable

whether enable DDNS interval

report upgrade interval useDDNSServer

type or address of DDNS server in use userHostDomain

whether enable host domain name

userName [DD_MAX_DDNS_ACCOUNT_BUF_LEN] DDNS account

password [DD_MAX_PASSWORD_BUF_LEN] DDNS password

hostDomain [DD_MAX_URL_BUF_LEN]

host domain name(correspond to a certain protocol,specifying server is permitted)

Client SDK Instructions

DD_DDNS_SERVER_INFO

struct of DDNS server information

struct _dd_ddns_server_info{ unsigned char	DDNSID;

unsigned char	supportproperty; unsigned char	noused[2];

char	DDNSServerName[64]; }DD_DDNS_SERVER_INFO;

Members

DDNSID

DDNS ID,server name is valid only if ID value is greater than 0

supportproperty NCFG_ENUM_DDNS_SUPPORT_DOMAIN1 =0x01(support domain 1,maybe need to support two domains)

noused[2]

unenable DDNS server DDNSServerName[64]

address of DDNS server

Client SDK Instructions

DD_DEVICEINFO

struct of device basic information.

struct _dd_device_info_{ unsigned long	iSize; unsigned long          deviceID;

char	deviceNo[DD_MAX_SERIAL_NUMBER_LEN]; char	deviceName [DD_MAX_NAME_LEN];

char	firmwareVersion [DD_MAX_VERSION_BUF_LEN];

char	firmwareBuildDate [DD_MAX_VERSION_BUF_LEN];

char	hardwareVersion [DD_MAX_VERSION_BUF_LEN];

char	kernelVersion [DD_MAX_VERSION_BUF_LEN];

char	mcuVersion [DD_MAX_VERSION_BUF_LEN]; unsigned char	audioNum;

unsigned char	localVideoInNum; unsigned char	netVideoInNum; unsigned char	sensorInNum; unsigned char	relayOutNum; unsigned char	rs232Num; unsigned char	rs485Num; unsigned char	networkPortNum; unsigned char	diskCtrlNum; unsigned char	DiskNum; unsigned char	vgaNum;

unsigned char	usbNum; }DD_DEVICE_INFO;

Members

iSize

size of the struct. deviceID

device ID(0~255). deviceNo[DD_MAX_SERIAL_NUMBER_LEN]

serial number of device,letter is usable. deviceName [DD_MAX_NAME_LEN]

device name(attention to double byte character). firmwareVersion [DD_MAX_VERSION_BUF_LEN]

software version number.

firmwareBuildDate [DD_MAX_VERSION_BUF_LEN] software building date.

hardwareVersion [DD_MAX_VERSION_BUF_LEN] hardware version.

kernelVersion [DD_MAX_VERSION_BUF_LEN] system core version.

mcuVersion [DD_MAX_VERSION_BUF_LEN] MCU version.

audioNum

audio number. localVideoInNum

channel number of local video input. netVideoInNum

channel number of network video input. sensorInNum

number of sensor for input. relayOutNum

number of relay for output. rs232Num

channel number of 232 remote sensing. rs485Num

channel number of 485 remote sensing. networkPortNum

number of network port. diskCtrlNum

number of harddisk for control. DiskNum

number of harddisk. vgaNum

number of displayer. usbNum

number of USB soket.

Client SDK Instructions

DD_ENCODE_CONFIG

struct of encoding configuration

struct _dd_encode_config_{ unsigned long	iSize; unsigned short	resolution; unsigned short	rate; unsigned short	encodeType; unsigned short	quality; unsigned short	minBitrate; unsigned short	maxBitrate; }DD_ENCODE_CONFIG;

Members

iSize

size of the struct resolution

resolution, refer to DD_VIDEO_SIZE:

Type	Value

DD_VIDEO_SIZE_QCIF	0x0001 DD_VIDEO_SIZE_CIF	0x0002 DD_VIDEO_SIZE_HD1	0x0004 DD_VIDEO_SIZE_D1	0x0008 DD_VIDEO_SIZE_QVGA	0x0010 DD_VIDEO_SIZE_VGA	0x0020 DD_VIDEO_SIZE_XVGA	0x0040 DD_VIDEO_SIZE_QQVGA 0x0080 DD_VIDEO_SIZE_480P	0x0100 DD_VIDEO_SIZE_720P	0x0200

DD_VIDEO_SIZE_1080P	0x0400

Video Format

QCIF CIF HD1 D1 QVGA VGA XVGA QQVGA 480P 720P

1080P

rate

frame rate encodeType

encoding type,refer to the following list: rate

frame rate encodeType

encoding type,refer to the following list:

type	ValueDescription

DD_VIDEO_ENCODE_MODE_VBR0x01

DD_VIDEO_ENCODE_MODE_CBR0x02

mutable code stream

fixed code stream

quality

image quality, refer to DD_IMAGE_QUALITY:

Type	ValueDescription

DD_IMAGE_QUALITY_LOWEST	0x01

DD_IMAGE_QUALITY_LOWER	0x02

DD_IMAGE_QUALITY_LOW	0x03

DD_IMAGE_QUALITY_MEDIUM	0x04

DD_IMAGE_QUALITY_HEIGHTER 0x05

lowest image quality

lower image quality

low image quality

medium image quality

heighter image quality

heightest DD_IMAGE_QUALITY_HEIGHTEST0x06 image

quality

minBitrate

code stream lower limit,in kbps maxBitrate

code stream upper limit,in kbps

Client SDK Instructions

DD_ENCODE_CONFIG_SUPPORT

struct of encode config supported by device

struct _dd_encode_config_support_ {

DD_ENCODE_CONFIG encodeConfig[DD_MAX_SUPPORT_RESOLUTION];

unsigned long	num; }DD_ENCODE_CONFIG_SUPPORT;

Members

encodeConfig[DD_MAX_SUPPORT_RESOLUTION] DD_MAX_SUPPORT_RESOLUTION equals 7, which is the max supported resolution type num, encodeConfig includes main stream(sub stream) resolution, fps, max and min bitrate.

num

the real supported resolution num by device

Client SDK Instructions

DD_FRAME_INFO

struct of data frame information

struct _dd_frame_info_{

unsigned long	frameType; unsigned long	length; unsigned long	keyFrame; unsigned long              width; unsigned long              height; unsigned long              *pData; unsigned short             deviceIndex; unsigned short             channel; unsigned long              bufIndex; unsigned long	frameIndex; unsigned long	frameAttrib; unsigned long	streamID; LONGLONG                        time;

LONGLONG	relativeTime; DD_TIME                    localTime; }DD_FRAME_INFO, *LP_DD_FRAME_INFO;

Members

frameType

data frame type, refer to DD_FRAME_TYPE:

Type	Value

DD_FRAME_TYPE_NONE	0x00 DD_FRAME_TYPE_VIDEO	0x01 DD_FRAME_TYPE_AUDIO	0x02 DD_FRAME_TYPE_TALK_AUDIO	0x03 DD_FRAME_TYPE_JPEG	0x04 DD_FRAME_TYPE_VIDEO_FORMAT	0x05 DD_FRAME_TYPE_AUDIO_FORMAT	0x06 DD_FRAME_TYPE_TALK_AUDIO_FORMAT0x07 DD_FRAME_TYPE_END

length

data length keyFrame

keyframe,0:non-key-frame,1:keyframe width

width of data frame height

height of data frame *pData

pointer to data deviceIndex

device index number channel

data channel bufIndex

buffer area index frameIndex

data frame index frameAttrib

data frame attribute, refer to DD_FRAME_ATTRIB:

Type	Value Description

DD_PLAY_FRAME_NO_SHOW	0x01

DD_PLAY_FRAME_SHOW	0x02

DD_PLAY_FRAME_ALL_END	0x04

DD_PLAY_FRAME_SEC_END	0x08

no show this frame

the frame can be showed

reading data is finished,no more data

the event section is ended

the frame includes

DD_PLAY_FRAME_NO_TIME_STAMP 0x10 timestamp,shield time function when capture

DD_PLAY_FRAME_FF

DD_LIVE_FRAME_FIRST_STREAM

the frame 0x20 applied to

fastforward

0x40 the frame is live main code

0x100

DD_LIVE_FRAME_SECOND_STREAM0x80

stream

the frame is live sub code stream

DD_LIVE_FRAME_JPEG

DD_LIVE_FRAME_TALK

the frame is JPEG image

the frame is 0x200talkback audio

data

streamID

data stream ID time

absolute time,calculate from 00:00:00 on Jan.1st in 1970,in microsecond,it changes when change device time

relativeTime

relative time,in microsecond,it won't change when change device time,because it is continuous

localTime

device local time, later fill in

Client SDK Instructions

DD_LIVE_AUDIO_GROUP

struct of audio group

struct _dd_live_audio_group_{ unsigned short	holdTime; unsigned char	volume; unsigned char	channel; }DD_LIVE_AUDIO_GROUP;

Members

holdTime

hold time(in second), 0 means invalid volume

volume(0-100) channel

channel number,start from 0

Client SDK Instructions

DD_LIVE_DISPLAY

struct of real time display

struct _dd_live_display_{ unsigned long	iSize; unsigned long	showTime; unsigned long	showNetwork; unsigned long	showHDD; unsigned long	showUSB; unsigned short	alarmInNum; unsigned short	alarmOutNum;

unsigned long	showAlarmIn; unsigned long	showAlarmOut; unsigned long         cameraNum;

unsigned char	showCameraName [DD_MAX_CAMERA_NUM]; unsigned char	showRecordStatus [DD_MAX_CAMERA_NUM]; }DD_LIVE_DISPLAY;

Members

iSize

size of the struct showTime

whether show system time showNetwork

whether show network status showHDD

whether show harddisc information showUSB

whether show movable storage infomation alarmInNum

alarm input number(read only) alarmOutNum

alarm output number(read only) showAlarmIn

whether show alarm input information showAlarmOut

whether show alarm output information cameraNum

valid channel number(read only) showCameraName [DD_MAX_CAMERA_NUM]

whether show channel name showRecordStatus [DD_MAX_CAMERA_NUM]

whether show record status

Client SDK Instructions

DD_LIVE_VIDEO_GROUP

struct of preview video group

struct _dd_live_video_group_{ unsigned short holdTime; unsigned short channelNum; unsigned long	splitMode;

unsigned char	channel [DD_MAX_CAMERA_NUM]; }DD_LIVE_VIDEO_GROUP;

Members

holdTime

hold time,(in second), 0 means invalid channelNum

valid channel number(read only) splitMode

split mode, refer to DD_VIEW_SPLIT_MODE:

type

DD_VIEW_SPLIT_1X1 DD_VIEW_SPLIT_2X2

DD_VIEW_SPLIT_1A2/ DD_VIEW_SPLIT_2X3 DD_VIEW_SPLIT_1A5/DD_VIEW_SPLIT_3X3 DD_VIEW_SPLIT_1A7/DD_VIEW_SPLIT_1A12/DD_VIEW_SPLIT_4X4 DD_VIEW_SPLIT_2A6/DD_VIEW_SPLIT_4X6 DD_VIEW_SPLIT_1A9/DD_VIEW_SPLIT_4A9/DD_VIEW_SPLIT_1A16/DD_VIEW_SPL DD_VIEW_SPLIT_1A11/DD_VIEW_SPLIT_1A20/DD_VIEW_SPLIT_4A20/DD_VIEW_S

channel [DD_MAX_CAMERA_NUM]

channel number corresponding to each area,array index stands for channel number,element value stands for window area number,0xff means invalid channel

Client SDK Instructions

DD_LOG_INFO

struct of log information

struct _dd_log_info_{ unsigned long	majorType; unsigned long	minorType; unsigned long	time; unsigned long	IP;

char	name [36]; DD_TIME          localTime; unsigned long      infoLen; char	info[1024]; }DD_LOG_INFO, *LP_DD_LOG_INFO;

Members

majorType

major type, refer to DD_LOG_CONTENT:

Type

DD_LOG_CONTENT_SYSTEM_CTRL DD_LOG_CONTENT_CONFIG DD_LOG_CONTENT_PLAYBACK DD_LOG_CONTENT_BACKUP DD_LOG_CONTENT_SEARCH DD_LOG_CONTENT_VIEW_INFO DD_LOG_CONTENT_EVENT_INFO

DD_LOG_CONTENT_ERROR_INFO

Value

0x00000001 0x00000002 0x00000004 0x00000008 0x00000010 0x00000020 0x00000040

0x00000080

minorType

minor type, refer to DD_LOG_TYPE:

DD_LOG_TYPE_SYSTEM_CTRL

Type

DD_LOG_TYPE_BOOT

0x01000000

description

boot system

DD_LOG_TYPE_SHUTDOWN

DD_LOG_TYPE_REBOOT

DD_LOG_TYPE_FORMAT_SUCC

DD_LOG_TYPE_FORMAT_FAIL

DD_LOG_TYPE_UPGRADE_SUCC

DD_LOG_TYPE_UPGRADE_FAIL DD_LOG_TYPE_CLEAR_ALARM DD_LOG_TYPE_OPEN_ALARM

DD_LOG_TYPE_MANUAL_START

DD_LOG_TYPE_MANUAL_STOP

DD_LOG_TYPE_PTZ_ENTER

DD_LOG_TYPE_PTZ_CTRL

DD_LOG_TYPE_PTZ_EXIT

DD_LOG_TYPE_AUDIO_CH_CHANGE

DD_LOG_TYPE_VOLUME_ADJUST

DD_LOG_TYPE_MUTE_ENABLE

DD_LOG_TYPE_MUTE_DISENABLE

DD_LOG_TYPE_DWELL_ENABLE

DD_LOG_TYPE_DWELL_DISENABLE

DD_LOG_TYPE_LOG_IN DD_LOG_TYPE_LOG_OFF

DD_LOG_TYPE_CHANGE_TIME

shutdown system

reboot system

format disc successfully

formatting disc fail

upgrade successfully

upgrade fail clear alarm open alarm

open manual record

stop manual record

start PTZ control

PTZ operation

exit PTZ control

chang audio channel

adjust volume

enable mute

disenable mute

enable dwell

disenable dwell

login logout

change system time

DD_LOG_TYPE_MANUAL_SNAP_SUCC

DD_LOG_TYPE_MANUAL_SNAP_FAIL

DD_LOG_TYPE_CONFIG

DD_LOG_TYPE_CHGE_VIDEO_FORMAT

DD_LOG_TYPE_CHGE_VGA_RESOLUTION

DD_LOG_TYPE_CHGE_LANGUAGE

DD_LOG_TYPE_CHGE_NET_USER_NUM

DD_LOG_TYPE_CHGE_TIME_ZONE

DD_LOG_TYPE_NTP_MANUAL

DD_LOG_TYPE_NTP_ON

DD_LOG_TYPE_NTP_OFF

DD_LOG_TYPE_CHGE_NTP_SERVER

DD_LOG_TYPE_CHGE_DST

manual capture succeed

manual capture fail

0x02000000

change video format

change VGA resolution

change language

change network user number

change time zone

manual network time check

enable automatic network time check

disenable automatic network time check

change network time server address

change daylight saving time setting

DD_LOG_TYPE_PASSWD_ON

DD_LOG_TYPE_PASSWD_OFF

DD_LOG_TYPE_CHGE_CAM_NAME

DD_LOG_TYPE_MODIFY_COLOR

DD_LOG_TYPE_CHGE_HOST_MONITOR

DD_LOG_TYPE_CHGE_SPOT

DD_LOG_TYPE_CHGE_OSD

DD_LOG_TYPE_CHGE_LOCAL_ENCODE

enable operation password

disappear operation password

change channel name

modify color

change host monitor image setting

change auxiliary output image setting

change character overlap setting

change encoding parameter of record stream

change DD_LOG_TYPE_CHGE_REC_VIDEO_SWITCH record video

switch setting

change DD_LOG_TYPE_CHGE_REC_AUDIO_SWITCHrecord audio

switch setting

DD_LOG_TYPE_CHGE_REC_REDU_SWITCH

DD_LOG_TYPE_CHGE_REC_PRE_TIME

change redundant record switch setting

change the time before record

DD_LOG_TYPE_CHGE_REC_POST_TIME

DD_LOG_TYPE_CHGE_REC_HOLD_TIME

DD_LOG_TYPE_CHGE_SCH_SCHEDULE

DD_LOG_TYPE_CHGE_SCH_MOTION

DD_LOG_TYPE_CHGE_SCH_ALARM

DD_LOG_TYPE_CHGE_SENSOR_SWITCH

DD_LOG_TYPE_CHGE_SENSOR_TYPE

DD_LOG_TYPE_CHGE_SENSOR_TRIGGER

DD_LOG_TYPE_CHGE_SENSOR_SCH

DD_LOG_TYPE_CHGE_MOTION_SWITCH

DD_LOG_TYPE_CHGE_MOTION_SENS

change the time after record

change record data expiry time

change the plan of regular record

change motion detection record schedule

change alarm record schedule

change alarm input switch setting

change alarm input sensor type

change alarm input semsor trigger setting

change alarm input detection schedule

change motion detection switch setting

change motion

DD_LOG_TYPE_CHGE_MOTION_AREA

DD_LOG_TYPE_CHGE_MOTION_TRIGGER

DD_LOG_TYPE_CHGE_MOTION_SCH

DD_LOG_TYPE_CHGE_VL_TRIGGER

DD_LOG_TYPE_CHGE_RELAY_SWITCH

DD_LOG_TYPE_CHGE_RELAY_SCH

DD_LOG_TYPE_BUZZER_ON

DD_LOG_TYPE_BUZZER_OFF

DD_LOG_TYPE_CHGE_BUZZER_SCH

DD_LOG_TYPE_CHGE_HTTP_PORT

DD_LOG_TYPE_CHGE_SER_PORT

DD_LOG_TYPE_CHGE_IP

detection sensitivity

change motion detection area setting

change motion detection process mode

change motion detection schedule

change video lost process mode setting

change alarm output relay setting

change alarm output schedule

enable buzzer alarm

disenable buzzer alarm

change buzzer alarm schedule

modify HTTP server port

modify network server port

change network IP

DD_LOG_TYPE_DHCP_SUCC

DD_LOG_TYPE_DHCP_FAIL

DD_LOG_TYPE_CHGE_PPPOE DD_LOG_TYPE_CHGE_DDNS

DD_LOG_TYPE_NET_STREAM_CFG

DD_LOG_TYPE_CHGE_SERIAL

DD_LOG_TYPE_PRESET_MODIFY

DD_LOG_TYPE_CRUISE_MODIFY

DD_LOG_TYPE_TRACK_MODIFY DD_LOG_TYPE_USER_ADD

DD_LOG_TYPE_USER_MODIFY

DD_LOG_TYPE_USER_DELETE

DD_LOG_TYPE_CHANGE_PASSWD

DD_LOG_TYPE_LOAD_DEFAULT

DD_LOG_TYPE_IMPORT_CONFIG

DD_LOG_TYPE_EXPORT_CONFIG

DD_LOG_TYPE_CHGE_IMAGE_MASK

obtain DHCP automatically succeed

obtain DHCP automatically fail

set PPPoE set DDNS

change network stream edcoding setting

change PTZ serial port setting

modify preset point

modify cruise line

modify track add users

modify user authority

delete user

modify user password

recover default configuration

import configuration

export configuration

image shield

DD_LOG_TYPE_RECYCLE_REC_ON

DD_LOG_TYPE_RECYCLE_REC_OFF

DD_LOG_TYPE_CHGE_DISK_ALARM

DD_LOG_TYPE_CHGE_SEND_EMAIL

DD_LOG_TYPE_CHGE_RECV_EMAIL

DD_LOG_TYPE_CHGE_SNAP_SETTING

DD_LOG_TYPE_PLAYBACK

DD_LOG_TYPE_PLAYBACK_PLAY DD_LOG_TYPE_PLAYBACK_PAUSE DD_LOG_TYPE_PLAYBACK_RESUME DD_LOG_TYPE_PLAYBACK_FF DD_LOG_TYPE_PLAYBACK_REW DD_LOG_TYPE_PLAYBACK_STOP

DD_LOG_TYPE_PLAYBACK_NEXT_SECTION

DD_LOG_TYPE_PLAYBACK_PREV_SECTION

DD_LOG_TYPE_BACKUP

DD_LOG_TYPE_BACKUP_START

DD_LOG_TYPE_BACKUP_COMPLETE

DD_LOG_TYPE_BACKUP_CANCEL

DD_LOG_TYPE_BACKUP_FAIL

DD_LOG_TYPE_SEARCH

enable loop record

close loop record

change disc alarm space

set Email sender information

set Email receiver information

change capture configuration

0x03000000

play pause

resume play fast forward rewind

stop

play next section

play previous section

0x04000000

start to backup

backup is completed

cancel backup

backup fails

0x05000000

DD_LOG_TYPE_SEARCH_TIME

DD_LOG_TYPE_SEARCH_EVENT

DD_LOG_TYPE_SEARCH_FILE_MAN

DD_LOG_TYPE_SEARCH_PICTURE

DD_LOG_TYPE_DELETE_FILE DD_LOG_TYPE_LOCK_FILE DD_LOG_TYPE_UNLOCK_FILE DD_LOG_TYPE_DELETE_PICTURE DD_LOG_TYPE_LOCK_PICTURE

DD_LOG_TYPE_UNLOCK_PICTURE

DD_LOG_TYPE_EVENT_INFO

DD_LOG_TYPE_SENSOR_START

DD_LOG_TYPE_SENSOR_END

DD_LOG_TYPE_MOTION_START

DD_LOG_TYPE_MOTION_END

DD_LOG_TYPE_VLOSS_START

DD_LOG_TYPE_VLOSS_END

DD_LOG_TYPE_SHELTER_START

DD_LOG_TYPE_SHELTER_END

DD_LOG_TYPE_BEHAVIOR_INFO

search by time

search by event

search file management

search picture

delete file lock file unlock file delete picture lock picture

unlock picture

0x07000000

start sensor alarm

sensor alarm ends

motion detection starts

motion detection ends

video loss start

video loss ends

video shelter starts

video shelter ends

0x08000000

DD_LOG_TYPE_ENTER_AREA DD_LOG_TYPE_EXIT_AREA DD_LOG_TYPE_INTRUSION DD_LOG_TYPE_LOITER

DD_LOG_TYPE_LEFT_TAKE

DD_LOG_TYPE_PARKING DD_LOG_TYPE_RUN

DD_LOG_TYPE_HIGH_DENSITY

DD_LOG_TYPE_ERROR_INFO

DD_LOG_TYPE_IP_CONFLICT

DD_LOG_TYPE_NETWORK_ERR

DD_LOG_TYPE_DDNS_ERR

DD_LOG_TYPE_DISK_IO_ERR

DD_LOG_TYPE_UNKNOWN_OFF

DD_LOG_TYPE_UNKNOWN_ERR

enter area exit area intrusion loiter

across left cordon

parking run

high density behaviour

0x09000000

network IP conflict

network exception

DDNS error

disc read-write error

electricity outage exception

unknown error

time

log occurrence time IP

user IP name [36]

user name localTime

local time, later fill in infoLen

length of log information info[1024]

length of the log information

Client SDK Instructions

DD_MOTION_AREA

struct of motion area

struct _dd_motion_area_{

unsigned long	sensitivity; unsigned short	widthNum; unsigned short	hightNum; unsigned char	area

[DD_MAX_MOTION_AREA_HIGHT_NUM] [DD_MAX_MOTION_AREA_WIDTH_NUM];

}DD_MOTION_AREA;

Members

sensitivity

sensitivity(0-7),high number means high sensitivity widthNum

width grid number of area hightNum

height grid number of area

area [DD_MAX_MOTION_AREA_HIGHT_NUM] [DD_MAX_MOTION_AREA_WIDTH_NUM]

grid mask data of area,compatible 1920x1080,each size is 8X8

Client SDK Instructions

DD_MOTION_CONFIG

struct of motion configuration

struct _dd_motion_config_{

unsigned long	iSize; unsigned char                   enable; unsigned char                   recv; unsigned short           holdTime; DD_MOTION_AREA           area; }DD_MOTION_CONFIG;

Members

iSize

size of the struct enable

whether enable motion detection recv

reserved byte holdTime

delay time area

area setting

Client SDK Instructions

DD_NETWORK_ADVANCE_CONFIG

struct of network advanced configuration

struct _dd_network_advance_config_{ unsigned long	iSize; unsigned char	bMessagePort; unsigned char	bAlarmPort; unsigned char	bMultiCastIP; unsigned char	bMTUByteNum; unsigned short	httpPort; unsigned short	datePort; unsigned short	messagePort; unsigned short	alarmPort;

unsigned short	maxOnlineUserNum; unsigned short	OnlineUserNum; unsigned long	multiCastIP; unsigned long	mtuByteNum; }DD_NETWORK_ADVANCE_CONFIG;

Members

iSize

size of the struct bMessagePort

whether support message port(read only) bAlarmPort

whether support alarm port(read only) bMultiCastIP

whether support multicast address(read only) bMTUByteNum

whether support MTU byte number(read only) httpPort

HTTP port datePort

data port messagePort

message command port

alarmPort alarm port

maxOnlineUserNum

supportable maximum online user number(read only) OnlineUserNum

number of online users multiCastIP

multicast address mtuByteNum

MTU byte number

Client SDK Instructions

DD_NETWORK_IP_CONFIG

struct of network IP configuration

struct _dd_network_ip_config_{ unsigned long	iSize; unsigned long        useDHCP; unsigned long        IP; unsigned long        subnetMask; unsigned long        gateway;

unsigned long	preferredDNS; unsigned long	alternateDNS; unsigned long	usePPPoE;

char	account[DD_MAX_PPPOE_ACCOUNT_BUF_LEN]; char	password[DD_MAX_PASSWORD_BUF_LEN]; }DD_NETWORK_IP_CONFIG;

Members

iSize

size of the struct useDHCP

whether enable DHCP IP

network IP subnetMask

subnet mask gateway

gateway preferredDNS

host DNS alternateDNS

alternate DNS usePPPoE

whether enable PPPoE account[DD_MAX_PPPOE_ACCOUNT_BUF_LEN]/i>

PPPoE account password[DD_MAX_PASSWORD_BUF_LEN]

PPPoE password

Client SDK Instructions

DD_POSITION

struct of position

struct _dd_position_{ unsigned short x; unsigned short y; }DD_POSITION;

Members

x

abscissa y

ordinate

Client SDK Instructions

DD_PTZ_CONFIG

struct of PTZ configuration

struct _dd_ptz_config_{ unsigned long

unsigned char unsigned char unsigned char unsigned char unsigned long DD_SERIAL_CONFIG }DD_PTZ_CONFIG;

iSize; enable; address; recv1; recv2; protocol; serial;

Members

iSize

size of the struct enable

whether enable PTZ function address

address recv1

reserved byte recv2

reserved byte protocol

protocol,refer to PROTOCOL_TYPE:

Type	Value

PROTOCOL_NULL	0 PROTOCOL_PELCOP	1 PROTOCOL_PELCOD	2 PROTOCOL_LILIN	3 PROTOCOL_MINKING	4

PROTOCOL_NEON	5 PROTOCOL_STAR	6 PROTOCOL_VIDO	7 PROTOCOL_DSCP	8 PROTOCOL_VISCA	9 PROTOCOL_SAMSUNG	10 PROTOCOL_RM110	11 PROTOCOL_HY	12

serial

serial port

Client SDK Instructions

DD_PTZ_PRESET_CONFIG

struct of PTZ preset configuration

struct _dd_ptz_preset_config_{ unsigned long	iSize;

unsigned char	enablePreset [DD_MAX_PRESET_NUM];

}DD_PTZ_PRESET_CONFIG;

Members

iSize

size of the struct

enablePreset [DD_MAX_PRESET_NUM] whether enable preset point

Client SDK Instructions

DD_PTZ_PROTOCOL_INFO

struct of PTZ protocol information

struct _dd_ptz_protocol_info unsigned long

unsigned long char

}DD_PTZ_PROTOCOL_INFO;

{ protocolID;

pportproperty; ProtocolName[64];

Members

protocolID protocol type ID

pportproperty

other attribute's MASK except ID,baud rate, such as whether support some special attribute track etc.

ProtocolName[64] protocal name

Client SDK Instructions

DD_RECORD_CONFIG

struct of record configuration

struct _dd_record_config_{ unsigned long	iSize; unsigned char	bOnlyVideo; unsigned char	bWithAudio;

unsigned char	bindAudioChannel; unsigned char	bRedundancy; unsigned short	preAlarmTime; unsigned short	postAlarmTime; unsigned short	expired;

unsigned short	recv; }DD_RECORD_CONFIG;

Members

iSize

size of the struct bOnlyVideo

transcript video(only video) bWithAudio

transcript audio(based on transcript video) bindAudioChannel

corresponding audio channel(may different from video channel number)

bRedundancy

whether redundant record preAlarmTime

time of record before alarm postAlarmTime

time of record after alarm expired

record expiry time recv

reserved byte

Client SDK Instructions

DD_RECORD_CONFIG_MASK

struct of record configuration mask

struct _dd_record_config_mask_{ unsigned long	iSize;

unsigned char	bindAudioChannel; unsigned char	bRedundancy; unsigned char	recv1;

unsigned char	recv2;

unsigned short	minPreAlarmTime; unsigned short	maxPreAlarmTime; unsigned short	minPostAlarmTime; unsigned short	maxPostAlarmTime; unsigned short	minExpired; unsigned short	maxExpired; }DD_RECORD_CONFIG_MASK;

Members

iSize

size of the struct bindAudioChannel

whether support to bind audio and video channel bRedundancy

whether support redundant record recv1

reserved byte recv2

reserved byte minPreAlarmTime

the minimum time of record before alarm maxPreAlarmTime

the maximum time of record before alarm minPostAlarmTime

the minimum time of record after alarm maxPostAlarmTime

the maximum time of record after alarm

minExpired

the minimum record data expiry time maxExpired

the maximum record data expiry time

Client SDK Instructions

DD_RECORD_LOG

struct of record log information

struct_dd_record_log_{ unsigned char bLocked; unsigned char bUnofficial; unsigned char enableCard; unsigned char recv1; unsigned short diskIndex; unsigned short fileIndex; unsigned short logIndex; unsigned short recv2; unsigned short deviceID; unsigned short cameraID; unsigned long channel; unsigned long type; unsigned long size;

DD_TIME	startTime; DD_TIME                 endTime; char            cardNo[32]; }DD_RECORD_LOG, *LP_DD_RECORD_LOG;

Members

bLocked

0 means unlocked,1 means locked bUnofficial

0 means official record,1 means unofficial record(overlap record after modifying time)

enableCard

whether enable card recv1

reserved byte diskIndex

disk number fileIndex

file index logIndex

log index recv2

reserved byte deviceID

device ID cameraID

camera ID channel

virtual channel number type

record type size

size of the record data startTime

start time endTime

end time cardNo[32]

card number

Client SDK Instructions

DD_RELAY_CONFIG

struct of delay configuration

struct _dd_relay_config_{ unsigned char	enable; unsigned char	recv; unsigned short	holdTime;

char	name [DD_MAX_NAME_BUF_LEN]; }DD_RELAY_CONFIG;

Members

enable

alarm output device enable switch recv

reserved byte holdTime

delay time

name [DD_MAX_NAME_BUF_LEN] device name

Client SDK Instructions

DD_SENSOR_CONFIG

struct of sensor configuration

struct _dd_sensor_config_{ unsigned long	iSize;

unsigned char	enable; unsigned char	bNO; unsigned short          holdTime;

char	name [DD_MAX_NAME_BUF_LEN]; }DD_SENSOR_CONFIG;

Members

iSize

size of the struct enable

whether enable detection bNO

device type:normal open or normal close holdTime

delay time

name [DD_MAX_NAME_BUF_LEN] device name

Client SDK Instructions

DD_SERIAL_CONFIG

struct of serial configuration

struct _dd_serial_config_{ unsigned long	baudRate; unsigned long	dataBit; unsigned long	stopBit; unsigned long	parity;

unsigned long	dataFlowControl; }DD_SERIAL_CONFIG;

Members

baudRate

baud rate,refer to the list below:

Type	Value

SBR_110	0 SBR_300	1 SBR_600	2 SBR_1200	3 SBR_2400	4 SBR_4800	5 SBR_9600	6 SBR_19200	7 SBR_38400	8 SBR_57600	9 SBR_115200	10 SBR_230400	11 SBR_460800	12 SBR_921600	13

dataBit

data bit,refer to the list below:

Type	Value

DATABITS7                        7 DATABITS8                        8

stopBit

stop bit,refer to the list below:

Type	Value

STOPBITS1	2 STOPBITSONEHALF	3 STOPBITS2	4

parity

parity check bit,refer to the list below:

Type

PARITYEVEN PARITYODD PARITYMARK PARITYSPACE

PARITYNONE

Value	Description

'E'	even parity check 'O'	odd parity check 'M'	mark parity check 'S'	space parity check

'N'	no parity check

dataFlowControl

data stream control

Client SDK Instructions

DD_SMTP_CONFIG

struct of SMTP configuration

struct _dd_smtp_config_{

unsigned long	iSize; unsigned short	port; unsigned short	enableSSL;

char	server [DD_MAX_URL_BUF_LEN]; char	sendAddress

[DD_MAX_URL_BUF_LEN];

char	password [DD_MAX_PASSWORD_BUF_LEN];

unsigned long	enableRecvAddrNum;	£© char	receiveAddress

[DD_MAX_EMAIL_RECEIVE_ADDR_NUM][DD_MAX_URL_BUF_LEN]; }DD_SMTP_CONFIG;

Members

iSize

size of the struct port

SMTP server port enableSSL

whether enable SSL check server [DD_MAX_URL_BUF_LEN]

send server address

sendAddress [DD_MAX_URL_BUF_LEN] send SMTP address

password [DD_MAX_PASSWORD_BUF_LEN] password

enableRecvAddrNum

available address number for receiving(read only) receiveAddress [DD_MAX_EMAIL_RECEIVE_ADDR_NUM] [DD_MAX_URL_BUF_LEN]

list of acception address

Client SDK Instructions

DD_TIME

struct of system time setting information.

struct _dd_time_{

unsigned char	second; unsigned char	minute; unsigned char	hour; unsigned char	wday; unsigned char	mday; unsigned char	month; unsigned short year; }DD_TIME, *LP_DD_TIME;

Members

second

seconds after minute,range (0-59). minute

minutes after hour,range (0-59). hour

hours since midnight,range (0-23). wday

day of week ,range(0-6; Sunday=0). mday

day of month,range (1-31). month

month range(0-11; January=0). year

year (current year minus 1900).

Client SDK Instructions

DD_TRIGGER_ALARM_OUT

struct of triggering alarm

struct _dd_trigger_alarm_out_{ unsigned char	toBuzzer;

unsigned char	ShowFullScreen; unsigned char	sendEmail;

unsigned char	toUploadToAlarmCentre; unsigned long	toAlarmOut; }DD_TRIGGER_ALARM_OUT;

Members

toBuzzer

trigger buzzer alarm ShowFullScreen

trigger full screen alarm(no trigger when channel number is 0xff)

sendEmail send email

toUploadToAlarmCentre upload to alarm center

toAlarmOut

alarm output(bit matches output device)

Client SDK Instructions

DD_TRIGGER_PTZ

struct of triggering PTZ

struct _dd_trigger_ptz_{ unsigned char

unsigned char unsigned char unsigned char }DD_TRIGGER_PTZ;

toPTZType; toIndex; backIndex; recv;

Members

toPTZType

linkage typeŁ¬refer to the list below:

Type	Value

DD_PTZ_TYPE_PRESET	1 DD_PTZ_TYPE_CRUISE	2 DD_PTZ_TYPE_TRACE	3

toIndex

linkage number(preset point, cruise line, track) backIndex

linkage returned number(preset point, cruise line, track)

recv

reserved byte

Client SDK Instructions

DD_TRIGGER_RECORD

struct of triggering video record

struct _dd_trigger_record_{ unsigned char	snapCH

[DD_MAX_CAMERA_NUM_BYTE_LEN]; unsigned char	recordCH

[DD_MAX_CAMERA_NUM_BYTE_LEN]; }DD_TRIGGER_RECORD;

Members

snapCH [DD_MAX_CAMERA_NUM_BYTE_LEN] trigger capture

recordCH [DD_MAX_CAMERA_NUM_BYTE_LEN] trigger record

Client SDK Instructions

DD_VIDEO_COLOR

struct of video color

struct _dd_video_color_{ unsigned long	startTime; unsigned char	brightness; unsigned char	hue; unsigned char	saturation; unsigned char	contrast; }DD_VIDEO_COLOR;

Members

startTime

start time of the color(relative time in one day) brightness

brightness ,range 0-255 hue

hue ,range 0-255 saturation

saturation ,range 0-255 contrast

contrast ,range 0-255

Client SDK Instructions

DD_VIDEO_COLOR_CONFIG

struct of video color configuration

struct _dd_video_color_config_{ unsigned long	iSize; unsigned long	usedNum;

DD_VIDEO_COLOR videoColor[DD_MAX_COLOR_CFG_NUM]; }DD_VIDEO_COLOR_CONFIG;

Members

iSize

size of the struct usedNum

scheme number in use, default value is 1 videoColor[DD_MAX_COLOR_CFG_NUM]

color scheme

Client SDK Instructions

DD_VIDEO_OSD_CONFIG

struct of video OSD configuration

struct _dd_video_osd_config_{ unsigned long iSize;

unsigned char enableCameraName; unsigned char enableTimeStamp; unsigned char enableTimeStampWithWeek; unsigned char enableDefineText; DD_POSITION cameraName;

DD_POSITION timeStamp; DD_POSITION defineText;

char text [DD_MAX_TEXT_BUF_LEN]; struct

{

unsigned long enable; DD_AREA                 area;

}cover[DD_MAX_VIDEO_COVER_NUM]; }DD_VIDEO_OSD_CONFIG;

Members

iSize

size of the struct enableCameraName

overlap channel name enableTimeStamp

overlap timestamp enableTimeStampWithWeek

timestamp with week enableDefineText

overlap self-defined text cameraName

channel name position,range from (0,0) to (100,100) timeStamp

timestamp position,range from (0,0) to (100,100) defineText

self-defined text position,range from (0,0) to (100,100)

text [DD_MAX_TEXT_BUF_LEN] self-defined text

enable

area overlap is valid or not area

area parameter cover[DD_MAX_VIDEO_COVER_NUM]

covered area parameter

Client SDK Instructions

DD_WEEK_SCHEDULE

struct of week schedule

struct _dd_week_schedule_{ DD_DATE_SCHEDULE week[7]; }DD_WEEK_SCHEDULE;

Members

week[7]

week schedule structure,7 stands for each day's schedule in a week of 7 days

Client SDK Instructions

DEC_ADVANCE_NETWORK

struct of decoder advanced network configuration

struct _dec_advance_network{ unsigned long	iSize; unsigned char               TimeZone; unsigned char               hour; unsigned char               min; unsigned char               sec; unsigned char               mday; unsigned char               month; unsigned short              year;

bool	enableFlag; char	name [132]; int	NTP_Port;

int	syncInterval; }DEC_ADVANCE_NETWORK;

Members

iSize

size of the struct TimeZone

time zone, refer to DD_TIME_ZONE_NAME:

Type	Value

DD_TIME_ZONE_GMT_D12	0 DD_TIME_ZONE_GMT_D11	1 DD_TIME_ZONE_GMT_D10	2 DD_TIME_ZONE_GMT_D9	3 DD_TIME_ZONE_GMT_D8	4 DD_TIME_ZONE_GMT_D7	5 DD_TIME_ZONE_GMT_D6	6 DD_TIME_ZONE_GMT_D5	7

DD_TIME_ZONE_GMT_D4_30	8 DD_TIME_ZONE_GMT_D4	9 DD_TIME_ZONE_GMT_D3_30	10 DD_TIME_ZONE_GMT_D3	11 DD_TIME_ZONE_GMT_D2	12 DD_TIME_ZONE_GMT_D1	13 DD_TIME_ZONE_GMT	14 DD_TIME_ZONE_GMT_A1	15 DD_TIME_ZONE_GMT_A2	16 DD_TIME_ZONE_GMT_A3	17 DD_TIME_ZONE_GMT_A3_30	18 DD_TIME_ZONE_GMT_A4	19 DD_TIME_ZONE_GMT_A4_30	20 DD_TIME_ZONE_GMT_A5	21 DD_TIME_ZONE_GMT_A5_30	22 DD_TIME_ZONE_GMT_A5_45	23 DD_TIME_ZONE_GMT_A6	24 DD_TIME_ZONE_GMT_A6_30	25 DD_TIME_ZONE_GMT_A7	26 DD_TIME_ZONE_GMT_A8	27 DD_TIME_ZONE_GMT_A9	28 DD_TIME_ZONE_GMT_A9_30	29 DD_TIME_ZONE_GMT_A10	30 DD_TIME_ZONE_GMT_A11	31 DD_TIME_ZONE_GMT_A12	32 DD_TIME_ZONE_GMT_A13	33

hour hour

min minute

sec

second mday

which day in a week month

which month in a year year

a particular year,2008-2025 enableFlag

NTP enable flag name [132]

NTP server address NTP_Port

NTP server port syncInterval

synchronous time interval,in hour

Client SDK Instructions

DEC_DATE_SCHEDULE

struct of decoder date schedule

struct _dec_date_schedule_{

unsigned long long	hour [24]; }DEC_DATE_SCHEDULE;

Members

hour [24]

time format,24 means in 24-hours time system, each bit of unsigned long long stands for each minute's status

Client SDK Instructions

DEC_DEVICE_CONFIG

struct of decoder configuration

struct _dec_device_config{ unsigned long

char [DEC_MAX_NAME_LEN];

unsigned long unsigned long unsigned long unsigned long char

[DEC_MAX_VERSION_BUF_LEN]; char

[DEC_MAX_VERSION_BUF_LEN]; char

[DEC_MAX_VERSION_BUF_LEN]; }DEC_DEVICE_CONFIG;

iSize; deviceName

channelNum; productID; productSubID; softVersion;

mcuVersion

kernelVersion

hardwareVersion

Members

iSize

size of the struct

deviceName [DEC_MAX_NAME_LEN]

device name(notice double byte character) channelNum

sum of decoder channels productID

product ID productSubID

product sub ID softVersion

soft version

mcuVersion [DEC_MAX_VERSION_BUF_LEN] MCU version

kernelVersion [DEC_MAX_VERSION_BUF_LEN]

kernel version

hardwareVersion [DEC_MAX_VERSION_BUF_LEN] hardware version

Client SDK Instructions

DEC_NETWORK_CONFIG

struct of decoder network configuration

struct _dec_network_config{ unsigned long

unsigned long unsigned long unsigned long unsigned short unsigned short char

unsigned long unsigned long unsigned long unsigned long }DEC_NETWORK_CONFIG;

iSize; IP;

subnetMask; gateway;

httpPort; decoderPort; MAC [8]; multiCastIP; bDHCP;

dns1; dns2;

Members

iSize

size of the struct IP

network address subnetMask

sub net mask gateway

gateway httpPort

HTTP port decoderPort

decoder port MAC [8]

binded MAC multiCastIP

multicast address

dns1 DNS1

dns2 DNS2

Client SDK Instructions

DEC_OTHER_ALARM

other alarm struct of decoder

struct _dec_other_alarm_{ unsigned long	iSize;

unsigned long	toBuzzerForIPConflict; unsigned long	toAlarmOutForDisconnect; unsigned long	toBuzzerForDisconnect; unsigned long	toAlarmOutForIPConflict; }DEC_OTHER_ALARM;

Members

iSize

size of the struct toBuzzerForIPConflict

IP conflict triggers buzzer toAlarmOutForDisconnect

IP conflict alarm output(bit matches output device) toBuzzerForDisconnect

network disconnection triggers buzzer toAlarmOutForIPConflict

network disconnection alarm output(bit matches output device)

Client SDK Instructions

DEC_SENSOR_SETUP

struct of decoder sensor setting

struct _dec_sensor_setup_{ unsigned long	iSize;

unsigned char	enable; unsigned char	bNO; unsigned short         holdTime;

char	name [DEC_MAX_BUF_LEN]; unsigned long	toBuzzer;

unsigned long	toAlarmOut; }DEC_SENSOR_SETUP;

Members

iSize

size of the struct enable

whether enable detection bNO

device type:normal open or normal close holdTime

delay time

name [DEC_MAX_BUF_LEN] device name

toBuzzer

trigger buzzer alarm toAlarmOut

alarm output(bit matches output device)

Client SDK Instructions

DEC_WEEK_SCHEDULE

struct of decoder week schedule

struct _dec_week_schedule_{ DEC_DATE_SCHEDULE         week[7]; }DEC_WEEK_SCHEDULE;

Members

week[7]

week schedule structure,7 stands for 7 days' schedule in a week

Client SDK Instructions

NET_SDK_ALARMINFO

struct of alarm information

struct _net_sdk_alarminfo{ DWORD	dwAlarmType; DWORD	dwSensorIn; DWORD	dwChannel; DWORD	dwDisk; }NET_SDK_ALARMINFO;

Members

dwAlarmType

alarm type,refer to the following list:

Type NET_SDK_N9000_ALARM_TYPE_MOTION NET_SDK_N9000_ALARM_TYPE_SENSOR NET_SDK_N9000_ALARM_TYPE_VLOSS NET_SDK_N9000_ALARM_TYPE_FRONT_OFFLINE NET_SDK_N9000_ALARM_TYPE_OSC NET_SDK_N9000_ALARM_TYPE_AVD

NET_SDK_N9000_ALARM_TYPE_AVD_SECENE

NET_SDK_N9000_ALARM_TYPE_AVD_CLARITY

NET_SDK_N9000_ALARM_TYPE_AVD_COLOR NET_SDK_N9000_ALARM_TYPE_PEA_TRIPWIRE NET_SDK_N9000_ALARM_TYPE_PEA_PERIMETER NET_SDK_N9000_ALARM_TYPE_VFD NET_SDK_N9000_ALARM_TYPE_CDD NET_SDK_N9000_ALARM_TYPE_IPD NET_SDK_N9000_ALARM_TYPE_CPC NET_SDK_N9000_ALARM_TYPE_FACE_MATCH NET_SDK_N9000_ALARM_TYPE_FACE_MATCH_FOR_IPC NET_SDK_N9000_ALARM_TYPE_TRAJECT NET_SDK_N9000_ALARM_TYPE_VEHICE NET_SDK_N9000_ALARM_TYPE_AOIENTRY NET_SDK_N9000_ALARM_TYPE_AOILEAVE NET_SDK_N9000_ALARM_TYPE_PASSLINE NET_SDK_N9000_ALARM_TYPE_IP_CONFLICT NET_SDK_N9000_ALARM_TYPE_DISK_IO_ERROR NET_SDK_N9000_ALARM_TYPE_DISK_FULL NET_SDK_N9000_ALARM_TYPE_RAID_SUBHEALTH NET_SDK_N9000_ALARM_TYPE_RAID_UNAVAILABLE NET_SDK_N9000_ALARM_TYPE_ILLEIGAL_ACCESS NET_SDK_N9000_ALARM_TYPE_NET_DISCONNECT NET_SDK_N9000_ALARM_TYPE_NO_DISK NET_SDK_N9000_ALARM_TYPE_SIGNAL_SHELTER

Description Motiondetection alarm Sensor alarm input

Video loss alarm

Front-end deviceofflinealarm Object Abandoned/Missing al Exception alarm

Exception detection-

Scene change, for IPC only Exception detection-

video blurred, for IPC only Exception detection-

video color cast, for IPC only Tripwire alarm

Intrusion alarm

Face Detection (currently IPC only) Crowddensity detection Intrusion persondetection People counting

Face matchalarm(NVR) Face matchalarm(IPC) Target tracking trajectory license plate(IPC)

Enter the area(IPC) Leave the area(IPC) Tripwirecounting IP address conflict Disk IO error

Full disk

Array sub-health Array unavailable Illegal access

Network disconnect No disk in disk group Signal shelter

NET_SDK_N9000_ALARM_TYPE_HDD_PULL_OUT	Front panel HDDpull out

the alarmtype NET_SDK_ALARM_TYPE(refer to the following list) is not be used now,it is invalid

Type

NET_SDK_ALARM_TYPE_MOTION NET_SDK_ALARM_TYPE_SENSOR NET_SDK_ALARM_TYPE_VLOSS NET_SDK_ALARM_TYPE_SHELTER

NET_SDK_ALARM_TYPE_DISK_FULL

Description

motion detection sensor alarm single loss shelter alarm

full harddisk

NET_SDK_ALARM_TYPE_DISK_UNFORMATTEDdisc unformatted

NET_SDK_ALARM_TYPE_DISK_WRITE_FAIL

NET_SDK_ALARM_TYPE_EXCEPTION

harddisk read-write error

exception alarm

dwSensorIn

sensor alarm input port number dwChannel

when alarm is relative to channel,dwChannel means alarm channel dwDisk

in disc alarming, it means disc number which alarms

Client SDK Instructions

NET_SDK_ALARMINFO_EX

struct of alarm information

struct _net_sdk_alarminfo_ex{ DWORD	dwAlarmType; DWORD	dwSensorIn; DWORD	dwChannel; DWORD	dwDisk;

char	sensorName[36];

char	alarmTime[20]; char            resv[128];

}NET_SDK_ALARMINFO_EX;

Members

dwAlarmType

alarm type,refer to the following list:

Type NET_SDK_N9000_ALARM_TYPE_MOTION NET_SDK_N9000_ALARM_TYPE_SENSOR NET_SDK_N9000_ALARM_TYPE_VLOSS NET_SDK_N9000_ALARM_TYPE_FRONT_OFFLINE NET_SDK_N9000_ALARM_TYPE_OSC NET_SDK_N9000_ALARM_TYPE_AVD

NET_SDK_N9000_ALARM_TYPE_AVD_SECENE

NET_SDK_N9000_ALARM_TYPE_AVD_CLARITY

NET_SDK_N9000_ALARM_TYPE_AVD_COLOR NET_SDK_N9000_ALARM_TYPE_PEA_TRIPWIRE NET_SDK_N9000_ALARM_TYPE_PEA_PERIMETER NET_SDK_N9000_ALARM_TYPE_VFD NET_SDK_N9000_ALARM_TYPE_CDD NET_SDK_N9000_ALARM_TYPE_IPD NET_SDK_N9000_ALARM_TYPE_CPC NET_SDK_N9000_ALARM_TYPE_FACE_MATCH NET_SDK_N9000_ALARM_TYPE_FACE_MATCH_FOR_IPC NET_SDK_N9000_ALARM_TYPE_TRAJECT NET_SDK_N9000_ALARM_TYPE_VEHICE NET_SDK_N9000_ALARM_TYPE_AOIENTRY NET_SDK_N9000_ALARM_TYPE_AOILEAVE NET_SDK_N9000_ALARM_TYPE_PASSLINE NET_SDK_N9000_ALARM_TYPE_IP_CONFLICT NET_SDK_N9000_ALARM_TYPE_DISK_IO_ERROR NET_SDK_N9000_ALARM_TYPE_DISK_FULL NET_SDK_N9000_ALARM_TYPE_RAID_SUBHEALTH NET_SDK_N9000_ALARM_TYPE_RAID_UNAVAILABLE NET_SDK_N9000_ALARM_TYPE_ILLEIGAL_ACCESS NET_SDK_N9000_ALARM_TYPE_NET_DISCONNECT

Description Motiondetection alarm Sensor alarm input

Video loss alarm

Front-end deviceofflinealarm Object Abandoned/Missing al Exception alarm

Exception detection-

Scene change, for IPC only Exception detection-

video blurred, for IPC only Exception detection-

video color cast, for IPC only Tripwire alarm

Intrusion alarm

Face Detection (currently IPC only) Crowddensity detection Intrusion persondetection People counting

Face matchalarm(NVR) Face matchalarm(IPC) Target tracking trajectory license plate(IPC)

Enter the area(IPC) Leave the area(IPC) Tripwirecounting IP address conflict Disk IO error

Full disk

Array sub-health Array unavailable Illegal access

Network disconnect

NET_SDK_N9000_ALARM_TYPE_NO_DISK	No disk in disk group NET_SDK_N9000_ALARM_TYPE_SIGNAL_SHELTER	Signal shelter NET_SDK_N9000_ALARM_TYPE_HDD_PULL_OUT	Front panel HDDpull out

the alarmtype NET_SDK_ALARM_TYPE(refer to the following list) is not be used now,it is invalid

Type

NET_SDK_ALARM_TYPE_MOTION NET_SDK_ALARM_TYPE_SENSOR NET_SDK_ALARM_TYPE_VLOSS NET_SDK_ALARM_TYPE_SHELTER

NET_SDK_ALARM_TYPE_DISK_FULL

Description

motion detection sensor alarm single loss shelter alarm

full harddisk

NET_SDK_ALARM_TYPE_DISK_UNFORMATTEDdisc unformatted

NET_SDK_ALARM_TYPE_DISK_WRITE_FAIL

NET_SDK_ALARM_TYPE_EXCEPTION

harddisk read-write error

exception alarm

dwSensorIn

sensor alarm input port number dwChannel

when alarm is relative to channel,dwChannel means alarm channel dwDisk

in disc alarming, it means disc number which alarms sensorName

in sensor alarming, it means the alarming sensor's name alarmTime

alarm time resv

preserve

Client SDK Instructions

NET_SDK_CLIENTINFO

struct of log information

struct _net_sdk_clientinfo{ LONG lChannel;

LONG streamType; HWND hPlayWnd; int bNoDecode;

}NET_SDK_CLIENTINFO, *LPNET_SDK_CLIENTINFO;

Members

lChannel

channel number,start from 0 streamType

data stream type,two types:NET_SDK_MAIN_STREAM and NET_SDK_SUB_STREAM

hPlayWnd

play window handle bNoDecode

0:decode,1:not decode.only for windows os,default value is 0

Client SDK Instructions

NET_SDK_DEVICE_DISCOVERY_INFO

discover device automatically on LAN

struct _net_sdk_device_discovery_info{ unsigned long	deviceType;

char	productType[16]; char	strIP[16];

char	strNetMask[16]; char                             strGateWay[16]; unsigned char byMac[8];

unsigned short	netPort; unsigned short	httpPort; unsigned long	softVer; unsigned long	softBuildDate; }NET_SDK_DEVICE_DISCOVERY_INFO;

Members

deviceType

device type,refer to the follow:

Type

NET_SDK_DVR NET_SDK_DVS NET_SDK_IPCAMERA NET_SDK_SUPERDVR

NET_SDK_DECODER

Description

Digital video record Network Video Server IP camera

board card

decoder

productType[16] product type

strIP[16] IP

strNetMask[16] subnet mask

strGateWay[16]

gateway byMac[8]

MAC address netPort

network port httpPort

http port softVer

software version softBuildDate

software build date

Client SDK Instructions

NET_SDK_DEVICEINFO

structure of device login

struct _net_sdk_deviceinfo{ unsigned char localVideoInputNum; unsigned char audioInputNum; unsigned char sensorInputNum; unsigned char sensorOutputNum;

unsigned long	displayResolutionMask; unsigned char videoOuputNum;

unsigned char netVideoOutputNum; unsigned char netVideoInputNum; unsigned char IVSNum;

unsigned char presetNumOneCH; unsigned char cruiseNumOneCH; unsigned char presetNumOneCruise; unsigned char trackNumOneCH; unsigned char userNum;

unsigned char netClientNum; unsigned char netFirstStreamNum; unsigned char deviceType; unsigned char doblueStream; unsigned char audioStream; unsigned char talkAudio; unsigned char bPasswordCheck; unsigned char defBrightness; unsigned char defContrast; unsigned char defSaturation; unsigned char defHue;

unsigned short videoInputNum; unsigned short deviceID; unsigned long	videoFormat; unsigned long function[8]; unsigned long deviceIP; unsigned char deviceMAC[8]; unsigned long buildDate; unsigned long buildTime;

char	deviceName[36];

char	firmwareVersion[36]; char	kernelVersion[64]; char	hardwareVersion[36]; char	MCUVersion[36];

char firmwareVersionEx[100];	//firmware version extension for new product

char deviceProduct[28];	//Device model

}NET_SDK_DEVICEINFO, *LPNET_SDK_DEVICEINFO;

Members

localVideoInputNum

number of local video input channel audioInputNum

number of audio input channel sensorInputNum

number of sensor input channel sensorOutputNum

number of relay output displayResolutionMask

monitor optional resolution videoOuputNum

number of video output(supportable maximum playback channel number)

netVideoOutputNum

maximum channel number of network playback netVideoInputNum

channel number of digital single input IVSNum

number of smart analytics channel presetNumOneCH

number of preset point in each channel cruiseNumOneCH

number of cruise line in each channel presetNumOneCruise

number of preset point in each cruise line trackNumOneCH

track number in each channel userNum

user number netClientNum

maximum client number netFirstStreamNum

channel maximum number of main code stream transmission,namely how many clients check main code stream meanwhile

deviceType device type

doblueStream

whether provide dual stream audioStream

whether provide audio stream talkAudio

whether enable talkback function:1 means yes;0 means no

bPasswordCheck

whether need to input password defBrightness

default brightness defContrast

default contrast defSaturation

default saturation defHue

default hue videoInputNum

channel number of video input(add network channel number if local video input)

deviceID device ID

videoFormat

video format,refer to the following list:

Type	Value DD_VIDEO_FORMAT_NTSC       0x01

DD_VIDEO_FORMAT_PAL	0x02

function[8]

function description deviceIP

device network address deviceMAC[8]

device MAC buildDate

building date:year<<16 + month<<8 + mday buildTime

building time:hour<<16 + min<<8 + sec deviceName[36]

device name firmwareVersion[36]

firmware version kernelVersion[64]

kernel version hardwareVersion[36]

hardware version MCUVersion[36]

MCU version firmwareVersionEx[100]

Reserved characters deviceProduct[28] Device model

Client SDK Instructions

NET_SDK_EVENT

struct of event log

struct _net_sdk_event{ unsigned short chnn; unsigned short type; DD_TIME	startTime; DD_TIME	endTime;

}NET_SDK_EVENT,*LPNET_SDK_EVENT;

Members

chnn

event happend in which channel type

event type startTime

event starting time endTime

event ending time

Client SDK Instructions

NET_SDK_FRAME_INFO

struct of data frame information

struct _net_sdk_frame_info{ unsigned long	deviceID; unsigned long         channel; unsigned long	frameType; unsigned long	length; unsigned long	keyFrame; unsigned long        width; unsigned long        height; unsigned long	frameIndex;

unsigned long	frameAttrib; unsigned long	streamID; LONGLONG	time; LONGLONG	relativeTime; }NET_SDK_FRAME_INFO;

Members

deviceID device ID

channel

data channel,channel number starts from 0 frameType

data frame type,refer to DD_FRAME_TYPE:

Type	Value

DD_FRAME_TYPE_NONE	0x00 DD_FRAME_TYPE_VIDEO	0x01 DD_FRAME_TYPE_AUDIO	0x02 DD_FRAME_TYPE_TALK_AUDIO	0x03 DD_FRAME_TYPE_JPEG	0x04 DD_FRAME_TYPE_VIDEO_FORMAT	0x05 DD_FRAME_TYPE_AUDIO_FORMAT	0x06 DD_FRAME_TYPE_TALK_AUDIO_FORMAT0x07

DD_FRAME_TYPE_EVENT	0x08 DD_FRAME_TYPE_TEXT	0x09 DD_FRAME_TYPE_END

length

data length keyFrame

keyframe,0:minor frame;1:key frame width

width of data frame height

height of data frame frameIndex

data frame index frameAttrib

data frame attribute,refer to DD_FRAME_ATTRIB:

Type	ValueDescription

DD_PLAY_FRAME_NO_SHOW	0x01

DD_PLAY_FRAME_SHOW	0x02

DD_PLAY_FRAME_ALL_END	0x04

DD_PLAY_FRAME_SEC_END	0x08

DD_PLAY_FRAME_NO_TIME_STAMP 0x10

DD_PLAY_FRAME_FF	0x20

no show the frame

show the frame

data read is finished,no more data

the section ends

the frame with time stamp, so shield time function when capture

fast forward frame

frame

live main DD_LIVE_FRAME_FIRST_STREAM	0x40 code stream

frame

live sub DD_LIVE_FRAME_SECOND_STREAM0x80 code stream

frame

DD_LIVE_FRAME_JPEG

DD_LIVE_FRAME_TALK

0x100JPEG image

talkback 0x200audio data

frame

streamID

data stream ID time

absolute time,calculate from 00:00:00 on Jan.1st in 1970,in microsecond,it changes when change device time

relativeTime

relative time,in microsecond,it won't change when change device time,because it is continuous

Client SDK Instructions

NET_SDK_JPEGPARA

struct of JPEG image

struct{

WORD	wPicSize; WORD	wPicQuality;

}NET_DVR_JPEGPARA,*LPNET_DVR_JPEGPARA;

Members

wPicSize

Picture size：0-CIF，1-QCIF，2-D1，3-UXGA，4-SVGA，5-HD720p，6-VGA，7-XVGA，8-HD900p

wPicQuality

Picture quality level：0-best，1-better，2-ordinary

Client SDK Instructions

NET_SDK_LOG

struct of device log information.

struct{

DD_TIME                strLogTime; DWORD	dwMajorType; DWORD	dwMinorType;

char	sNetUser[MAX_NAMELEN]; DWORD	dwRemoteHostAddr;

char	sContent[MAX_CONTENTLEN]; }NET_SDK_LOG,*LPNET_SDK_LOG;

Members

strLogTime time of log.

dwMajorType main type.

dwMinorType minor type.

sNetUser[MAX_NAMELEN] network user.

dwRemoteHostAddr remote host address.

sContent[MAX_CONTENTLEN] Details.

Client SDK Instructions

NET_SDK_REC_EVENT

struct of record file information according to event.

struct _net_sdk_rec_event{

DWORD	dwChannel; DD_TIME                startTime; DD_TIME	stopTime; DWORD	dwRecType; }NET_SDK_REC_EVENT;

Members

dwChannel channel number.

startTime start time.

stopTime stop time.

dwRecType

record event type,refer to DD_RECORD_TYPE:

type

record

record at

Type	Value Description

DD_RECORD_TYPE_NONE

DD_RECORD_TYPE_MANUAL

0x0000no record

0x0001manually

DD_RECORD_TYPE_SCHEDULE0x0002regular time

DD_RECORD_TYPE_MOTION

DD_RECORD_TYPE_SENSOR

motion 0x0004detection

record

sensor 0x0008alarm

record

alarm

behaviour DD_RECORD_TYPE_BEHAVIOR 0x0010analysis

record

Client SDK Instructions

NET_SDK_REC_FILE

struct of record file information.

struct _net_sdk_rec_file{

DWORD	dwChannel; DWORD	bFileLocked; DD_TIME	startTime; DD_TIME	stopTime; DWORD	dwRecType; DWORD	dwPartition; DWORD	dwFileIndex; }NET_SDK_REC_FILE;

Members

dwChannel channel number.

bFileLocked

whether record file is locked. startTime

start time. stopTime

stop times. dwRecType

record event type,refer to DD_RECORD_TYPE:

type

record

record at

Type	Value Description

DD_RECORD_TYPE_NONE

DD_RECORD_TYPE_MANUAL

0x0000no record

0x0001manually

DD_RECORD_TYPE_SCHEDULE0x0002regular time

alarm

DD_RECORD_TYPE_MOTION

DD_RECORD_TYPE_SENSOR

motion 0x0004detection

record

sensor 0x0008alarm

record

behaviour DD_RECORD_TYPE_BEHAVIOR 0x0010analysis

record

dwPartition

record file partition. dwFileIndex

index of record filename.

Client SDK Instructions

NET_SDK_REC_TIME

struct of record file information by time.

struct _net_sdk_rec_time{ DWORD

DD_TIME DD_TIME

}NET_SDK_REC_TIME;

dwChannel; startTime; stopTime;

Members

dwChannel channel number.

startTime

the start time of videotape. stopTime

the stop time of videotape.

Client SDK Instructions

NET_SDK_RECORD_STATUS

struct of record status

struct _net_sdk_record_status{ DWORD	dwRecordType; DWORD	dwChannel; }NET_SDK_RECORD_STATUS;

Members

dwRecordType

record event type,refer to DD_RECORD_TYPE:

type

manua

record

alarm

Type	Value Description

DD_RECORD_TYPE_NONE

DD_RECORD_TYPE_MANUAL

0x0000no record

0x0001recordl

DD_RECORD_TYPE_SCHEDULE0x0002regular

DD_RECORD_TYPE_MOTION

DD_RECORD_TYPE_SENSOR

motion 0x0004detection

record

sensor 0x0008alarm

record

behaviour DD_RECORD_TYPE_BEHAVIOR 0x0010analysis

record

dwChannel record channel

Client SDK Instructions

NET_SDK_RECORD_STATUS_EX

struct of record status

struct _net_sdk_record_status_ex{ DWORD	dwRecordType; DWORD	dwChannel;

DWORD	dwRecordStatus; }NET_SDK_RECORD_STATUS_EX;

Members

dwRecordType

record event type,refer to DD_RECORD_TYPE:

type

manua

record

alarm

Type	Value Description

DD_RECORD_TYPE_NONE

DD_RECORD_TYPE_MANUAL

0x0000no record

0x0001recordl

DD_RECORD_TYPE_SCHEDULE0x0002regular

DD_RECORD_TYPE_MOTION

DD_RECORD_TYPE_SENSOR

motion 0x0004detection

record

sensor 0x0008alarm

record

behaviour DD_RECORD_TYPE_BEHAVIOR 0x0010analysis

record

dwChannel

record channel dwRecordStatus

0 stoped,1 recording,2 abnormal

Client SDK Instructions

PTZ_3D_POINT_INFO

information about PTZ 3D control

struct PTZ_3D_POINT_INFO{ int	selBeginX;

int	selBeginY; int	selEndX; int	selEndY;

int	displayWidth; int	displayHeight; int	reserve[2]; }PTZ_3D_POINT_INFO;

Members

selBeginX

X coordinates of the starting point selBeginY

Y coordinates of the starting point selEndX

End point X coordinates selEndY

End point Y coordinates displayWidth

Image width displayHeight

Image height reserve[2]

Retention value,not enable

Remarks

4 coordinate variable is the mouse position relative to the current window of the upper left corner of the screen. enlarge:selBeginX < selEndX, narrow:selBeginX > selEndX

Client SDK Instructions

NET_SDK_IPC_DEVICE_INFO

struct of IPC in device management

_net_sdk_ipc_device_info_{ unsigned long deviceID; unsigned short channel; unsigned short status;

char	szEtherName[16]; char	szServer[64]; unsigned short nPort;

unsigned short nHttpPort; unsigned short nCtrlPort; char	szID[64]; char	username[36];

unsigned long manufacturerId;

char	manufacturerName[36]; char	productModel[36]; unsigned char bUseDefaultCfg; unsigned char bPOEDevice;

unsigned char resv[2]; }NET_SDK_IPC_DEVICE_INFO;

Members

deviceID

device ID(reserved) channel

Channel number of IPC(Start from 0) status

Connection status(1 means online,0 means offline) szEtherName[16]

If it is null,default is eth0 szServer[64]

IP address of IPC nPort

Port of IPC nHttpPort

http port

nCtrlPort

Control ports, generally the same as the nPort szID[64]

Device identification (or MAC address) username[36]

username manufacturerId

(reserved) manufacturerName[36]

(reserved) productModel[36]

(reserved) bUseDefaultCfg

(reserved) bPOEDevice

(reserved) resv[2]

(reserved)

Client SDK Instructions

NET_SDK_SEARCH_IMAGE_ITEM

searched face picture information

typedef struct _net_sdk_search_image_item_ {

DD_TIME_EX	recStartTime; DD_TIME_EX	recEndTime;

unsigned int	similarity;	//similarity

unsigned int	faceFeatureId; //the matched feature when searching by face features

NET_SDK_FACE_IMG_INFO_CH sfaceImg; //the matched picture when searching by face image

unsigned char	resv[4];//reserved }NET_SDK_SEARCH_IMAGE_ITEM;

Members

recStartTime

start time of face based recording recEndTime

end time of face based recording similarity

similarity faceFeatureId

target face ID sfaceImg

matched image information resv

reserved

Client SDK Instructions

NET_SDK_SEARCH_IMAGE_BY_IMAG E_LIST

the return information of search image by image

typedef struct _net_sdk_search_image_by_image_list_ {

unsigned int	bEnd; //1 indicates finishing searching images; 0 means there are still images behind.

unsigned int	listNum;//return NET_SDK_SEARCH_IMAGE_ITEM num

NET_SDK_SEARCH_IMAGE_ITEM *pSearchImageItem; }NET_SDK_SEARCH_IMAGE_BY_IMAGE_LIST;

Members

bEnd

Whether all return or not? listNum

the number of return data pSearchImageItem

return the searched face information

Client SDK Instructions

NET_SDK_CH_SNAP_FACE_IMG_LIS T

captured face picture data of a camera

typedef struct _net_sdk_ch_snap_face_img_list_ {

unsigned int	bEnd; //1 indicates finishing searching images; 0 means there are still images behind.

unsigned int	listNum;//return NET_SDK_FACE_IMG_INFO_CH num

NET_SDK_FACE_IMG_INFO_CH *pCHFaceImgItem; }NET_SDK_CH_SNAP_FACE_IMG_LIST;

Members

bEnd

Whether all return or not? listNum

the number of return data pSearchImageItem

return the searched face information

Client SDK Instructions

DD_TIME_EX

Struct of time configuration information of the device. Compared to DD_TIME，the month and year value of DD_TIME is different .

typedef struct _dd_time_ex_ {

unsigned char	second; minute (0–59)

unsigned char	minute; hour (0–59)

unsigned char	hour; midnight (0–23)

unsigned char	wday; Sunday = 0)

unsigned char	mday; 31)

unsigned char	month; January = 1)

unsigned short	year; year )

int //total seconds

//Seconds after

//Minutes after

//Hours since

//Day of week (0–6;

//Day of month (1–

//Month (1–12;

//Year (current

nTotalseconds;

int //microsecond

}DD_TIME_EX;//Compared to of DD_TIME is different.

nMicrosecond;

DD_TIME，the month and year value

Members

second

second; it ranges from 0 to 59. minute

minute; it ranges from 0 to 59. hour

Count from 0 0'clock. It ranges from 0 to 23. wday

Day (it ranges from 0 to 6)，A week starts from sunday (the corresponding value is 0).

mday

date of a month. It ranges from 1 to 31. month

month. It ranges from 1 to 12. It starts from January. The coresponding value of January is 1.

year

Year, the current year

Client SDK Instructions

DD_NETWORK_PLATFORM

Reigster the upper-level platform

typedef struct _network_platform {

//N9000 supports two platforms: national standard platform and platform software

unsigned int CurrentPlat; //the current platform. default: 1 (it indicates platform software)，2 (it indicates national standard platform)

//platform software

unsigned int Switcher;	//1 indicates "Enable"， 0 indicates "Disable"

unsigned int Port;	//port unsigned int ReportId;        //device ID

char szAddress[16];	//ip address //national standard platform，ipc unavailable，

N9000 available

unsigned int SwitchGB; "Enable"，0 indicates "Disable"

unsigned int PortGB;

unsigned int uLocalPort; char szRelm[16];

domain

char szAddressGB[16];

char szUserName[16];

char szPassword[16];

//11 indicates

//port

//local port

//sip server

// address

//username

//password

char szDeviceIdGB[32]; char szServerIdGB[32];

}DD_NETWORK_PLATFORM;

//device ID //sip server ID

Members

CurrentPlat

the current platform. default: 1 (it indicates platform software)，2 (it indicates national standard platform)

Switcher

platform software; 1 indicates "Enable"，0 indicates "Disable"

Port

platform software; port ReportId

platform software; device ID szAddress

platform software; ip address SwitchGB

national standard platform; 1 indicates "Enable"，0 indicates "Disable

PortGB

national standard platform; port uLocalPort

national standard platform; local port

szRelm

national standard platform; sip server domain szAddressGB

national standard platform; address szUserName

national standard platform; username szPassword

national standard platform; password szDeviceIdGB

national standard platform; szServerIdGB

national standard platform;

device ID

sip server ID

Client SDK Instructions

DD_SMART_VFD_CONFIG

Face comparison configuration

typedef struct _dd_smart_vfd_config_ {

unsigned int iSize; //struct size

unsigned char enableFaceDetect; //enable/disable face detection

unsigned char enableSaveFacePicture; //enable/disable "Save Face Picture"

unsigned short enableSaveSourcePicture; //enable/disable "Save Source Picture"

unsigned int holdTime;	//hold time

DD_POSITION startPoint;	//coordinate information of the upper left point of the rectangle

DD_POSITION endPoint;	//coordinate information of the bottom right point of the rectangle

unsigned int pushModeType;	//snapshot mode: 0：auto; it will not capture repeatedly. 1：capture pictures according to the fixed time interval.

unsigned int intervalTime;	//inteval period of snapshot (seconds)，only when the snapshot mode is 1, will it take effect. }DD_SMART_VFD_CONFIG;

Members

iSize

struct size enableFaceDetect

enable/disable face detection enableSaveFacePicture

enable/disable "Save Face Picture" enableSaveSourcePicture enable/disable "Save Source Picture" holdTime

hold time startPoint

coordinate information of the upper left point of the rectangle

endPoint

coordinate information of the bottom right point of the rectangle

pushModeType

snapshot mode: 0：auto; it will not capture repeatedly. 1：capture pictures according to the fixed time interval.

intervalTime

inteval period of snapshot (seconds)，only when the snapshot mode is 1, will it take effect.

Client SDK Instructions

NET_SDK_FACE_INFO_GROUP_ITEM

struct of the group of face comarison

typedef struct _net_sdk_face_info_group_item_ {

unsigned char	guid[48]; //GROUP GUID

char	name[DD_MAX_NAME_LEN];//GROUP NAME

unsigned int property; //NET_SDK_FACE_INFO_GROUP_PROPERTY_TYPE

unsigned int	groupId;	// unsigned int	enableAlarmSwitch;

}NET_SDK_FACE_INFO_GROUP_ITEM;

Members

guid

group GUID name

group name property

type of the group; refer to NET_SDK_FACE_INFO_GROUP_PROPERTY_TYPE.

groupId group id

enableAlarmSwitch

Support face match alarm

Client SDK Instructions

NET_SDK_FACE_INFO_GROUP_ADD

Add the group of face comparison

typedef struct _net_sdk_face_info_group_add_ {

char	name[DD_MAX_NAME_LEN];//GROUP NAME

unsigned int property; //NET_SDK_FACE_INFO_GROUP_PROPERTY_TYPE }NET_SDK_FACE_INFO_GROUP_ADD;

Members

name

group name property

type of the group; refer to NET_SDK_FACE_INFO_GROUP_PROPERTY_TYPE

Client SDK Instructions

NET_SDK_FACE_INFO_GROUP_DEL

Delete the group of face comparison

typedef struct _net_sdk_face_info_group_del_ {

unsigned char	guid[48]; //GROUP GUID }NET_SDK_FACE_INFO_GROUP_DEL;

Members

guid

Delet the GUID of the group

Client SDK Instructions

NET_SDK_FACE_INFO_LIST_GET

struct of searching face comparison target

typedef struct _net_sdk_face_info_list_get_ {

unsigned int (compulsory)

unsigned int

pageIndex;

pageSize;

// 1、2、3...

//compulsory

unsigned int	groupId;//1、2、3....(compulsory)

char	name[DD_MAX_NAME_LEN];//name of NET_SDK_FACE_INFO_LIST_ITEM

unsigned int	itemId; // itemID of NET_SDK_FACE_INFO_LIST_ITEM

}NET_SDK_FACE_INFO_LIST_GET;

Members

pageIndex page number

pageSize

the number of items in the page groupId

group Id。 name

target name (non-compulsory) itemId

target ID (non-compulsory)

Client SDK Instructions

NET_SDK_FACE_INFO_LIST_ITEM_G ROUPS

The group the face picture belongs to and the terms of validity the face picture lasts in this group

typedef struct _net_sdk_face_info_list_item_groups_ {

unsigned int	groupId;	//There is no guid when GROUP id gets target list.

unsigned char	guid[48];	//Add GROUP GUID. when target information is edited, guid must be used.

DD_TIME_EX	validStartTime;//when the property state is "limited", the validStartTime and validEndTime are shown.

DD_TIME_EX	validEndTime;//when the property state is "limited", the validStartTime and validEndTime are shown. }NET_SDK_FACE_INFO_LIST_ITEM_GROUPS;

Members

groupId group Id

guid

GUID of the group。 validStartTime

valid start time of the group; when the type of the group is "limited", it takes effect.

validEndTime

valid end time of the group; when the type of the group is "limited", it takes effect.

Client SDK Instructions

NET_SDK_FACE_INFO_LIST_ITEM

target face information of face comparsion

typedef struct _net_sdk_face_info_list_item_ {

unsigned int	itemId;	//id char              name[DD_MAX_NAME_LEN];

//compulsory

unsigned int	sex; //0:male 1:female unsigned int	birthday;//eg:19900707

char	nativePlace[DD_MAX_NAME_LEN]; //

unsigned int	certificateType; //0:idCard char

certificateNum[DD_MAX_CERTIFICATE_NUM]; //

char	mobile[20];	// char	number[20];	// unsigned int     faceImgCount;

NET_SDK_FACE_INFO_LIST_ITEM_GROUPS groups[DD_MAX_FACE_INFO_GROUPS];

}NET_SDK_FACE_INFO_LIST_ITEM;

Members

itemId :target face Id name: name

sex : gender

birthday :date of birth nativePlace: native place

certificateType : certifcate type; 0:idCard

certificateNum : certificate number mobile: phone number

number: nuber

faceImgCount: number of face pictures

groups :group information; one face picture can be added to a maximum of 16 groups.

Client SDK Instructions

NET_SDK_FACE_INFO_LIST

search the returned face comparison target lsit.

typedef struct _net_sdk_face_info_list_ {

unsigned int	totalNum;	// unsigned int	listNum;//return

NET_SDK_FACE_INFO_LIST_ITEM num NET_SDK_FACE_INFO_LIST_ITEM

*pFaceInfoListItem; }NET_SDK_FACE_INFO_LIST;

Members

totalNum

total number of target listNum<

number of target pFaceInfoListItem

target list

Client SDK Instructions

NET_SDK_FACE_IMG_INFO_CH

The face picture captured by the camera can be used as target picture of comparison.

typedef struct _net_sdk_face_img_info_ch_ {

DD_TIME_EX           frameTime; unsigned int	imgId;

unsigned int	chl; //return value of 255 means the deleted channel.

unsigned char resv[8];//reserved

}NET_SDK_FACE_IMG_INFO_CH;

Members

frameTime< snapshot time

imgId image Id

chl

snapshot channel

Client SDK Instructions

NET_SDK_FACE_INFO_ADD

Add the face picture you want to compare.

typedef struct _net_sdk_face_info_add_ {

NET_SDK_FACE_INFO_LIST_ITEM sFaceInfoItem; unsigned int	imgNum;

NET_SDK_FACE_IMG_INFO_CH sFaceImgInfo[DD_MAX_FACE_INFO_IMG];//最大5张

unsigned int unsigned int

==1 Valid

haveImgData;//0、1 imgWidth;//haveImgData

unsigned int	imgHeight;//haveImgData ==1 Valid

unsigned int	imgLen;//haveImgData ==1 Valid

unsigned char	*imgData;//haveImgData ==1 Valid

}NET_SDK_FACE_INFO_ADD;

Members

sFaceInfoItem

Face feature information imgNum

the number of images in sFaceImgInfo sFaceImgInfo

image information haveImgData

Picture data of the external importing picture imgWidth

picture width of the external importing picture. haveImgData==1 valid

imgHeight

picture height of the external importing picture. haveImgData==1valid

>imgLen<

picture size of the external importing picture.haveImgData==1valid

imgData

picture data. haveImgData==1valid

Client SDK Instructions

NET_SDK_FACE_INFO_EDIT

Edit the face picture you want to compare.

typedef struct _net_sdk_face_info_edit_ {

unsigned int delFaceImgs[DD_MAX_FACE_INFO_IMG];

NET_SDK_FACE_INFO_ADD sFaceInfoItem;

}NET_SDK_FACE_INFO_EDIT;

Members

delFaceImgs

Delete face image sFaceInfoItem

modified target face information

Client SDK Instructions

NET_SDK_FACE_INFO_DEL

Delete the face picture you want to compare.

typedef struct _net_sdk_face_info_del_ {

unsigned int faceInfoListItemId;//NET_SDK_FACE_INFO_LIST_ITEM 中的itemId

unsigned int groupsId[DD_MAX_FACE_INFO_GROUPS];//NET_SDK_F ACE_INFO_LIST_ITEM中的itemId-->groups-->groupId

}NET_SDK_FACE_INFO_DEL;

Members

faceInfoListItemId

target face Id。 groupsId

group Id。

Client SDK Instructions

NET_SDK_FACE_MATCH_ALARM_TRI GGER

Alarm linkage information of face match alarm for target groups

typedef struct _net_sdk_face_match_alarm_trigger_ {

unsigned char	guid[48]; //GROUP GUID unsigned int     groupId; //group ID unsigned char     groupSwitch;//enable

unsigned char	alarmOutSwitch;//trigger alarm output

unsigned char	alarmOut[16];//trigger a maximum of 16 alarm outputs. The index starts from 1.

unsigned char	recSwitch;//recording unsigned int     recCH[128];//trigger recording

channels/cameras. The index starts from 1. unsigned char	snapSwitch;//snapshot

unsigned int	snapCH[128];//trigger snapshot channels/camera. The index starts from 1.

unsigned int	popVideo;//pop up window

unsigned char unsigned char unsigned char unsigned char

msgPushSwitch; buzzerSwitch; popMsgSwitch; emailSwitch;

}NET_SDK_FACE_MATCH_ALARM_TRIGGER;

Members

guid

Group GUID

groupId Group ID

groupSwitch

Whether to enable alarm for the group. alarmOutSwitch

trigger alarmOut alarmOut

Trigger the channels of alarm out. The index of channels starts from 1.

recSwitch

Trigger recording recCH

Trigger the recording channels. The index of channels starts from 1.

snapSwitch Trigger snapshot

snapCH

Trigger snapshot channels. The index of channels starts from 1.

popVideo

Trigger pop-up video. 0 menas no video pops up. Other number means the video pops up.

msgPushSwitch

Trigger message push buzzerSwitch

Trigger buzzer popMsgSwitch

Trigger pop-up message emailSwitch

Trigger email

Client SDK Instructions

NET_SDK_FACE_MATCH_ALARM

Face match alarm

typedef struct _net_sdk_face_match_alarm_ {

unsigned int	similarity;// similarity

unsigned int	enableCH[128];//【Enable CH】 starts from 1.

unsigned int	faceFeatureGroupsNum;//number of face group

NET_SDK_FACE_MATCH_ALARM_TRIGGER *pFaceMatchAlarmTrigger;

}NET_SDK_FACE_MATCH_ALARM;

Members

similarity

face picture similarity enableCH

enable channel. It starts from1. faceFeatureGroupsNum

Number of linkage alarms of face target groups sFaceMatchAlarmTrigger

linkage alarm information of face target groups

Client SDK Instructions

NET_SDK_FACE_INFO_IMG_DATA

Image data of face picture

typedef struct _net_sdk_face_info_img_data_ {

unsigned int	imgLen;//length of face picture

unsigned char	*imgData;//face picture data }NET_SDK_FACE_INFO_IMG_DATA;

Members

imgLen

the length of face picture imgData

face picture data

Client SDK Instructions

NET_SDK_FACE_INFO_IMG_GET

area struct

typedef struct _net_sdk_face_info_img_get_ {

unsigned int unsigned int

itemId; //target id

index;//start index 1 of faceImgCount

}NET_SDK_FACE_INFO_IMG_GET;

Members

itemId target Id

index

The index of faceImgCount starts from 1.

Client SDK Instructions

DD_ENCODE_CONFIG_EX

struct of encoding configuration

struct _dd_encode_config_{ unsigned long	iSize; unsigned short       resolution; unsigned short       rate; unsigned short       encodeType; unsigned short       quality; unsigned short       minBitrate; unsigned short       maxBitrate; unsigned short bitrate; //bitrate

unsigned short encodeFormat; //H264or265 coding DD_VIDEO_ENCODE_FORMAT

char recv[14];	//reserved bytes }DD_ENCODE_CONFIG;

Members

iSize

struct size resolution

resolution. Refer to DD_VIDEO_SIZE_N9000： rate

Frame rate encodeType

encoding type. Refer to the following table.

Type	ValueDescription

DD_VIDEO_ENCODE_MODE_VBR0x01 VBR DD_VIDEO_ENCODE_MODE_CBR0x02 BR

quality

Refer to the following table.

Type	ValueDescription

DD_IMAGE_QUALITY_LOWEST	0x01

DD_IMAGE_QUALITY_LOWER	0x02

DD_IMAGE_QUALITY_LOW	0x03

DD_IMAGE_QUALITY_MEDIUM	0x04

DD_IMAGE_QUALITY_HEIGHTER 0x05

lowest quality

lower quality

low quality

middle quality

higher quality

DD_IMAGE_QUALITY_HEIGHTEST0x06

highest quality

minBitrate

lower limit of bitrate; unit:kbps maxBitrate

upper limit of bitrate; unit: kbps bitrate

bitrate; unit: kbps encodeFormat

Encoding type: H264or265. Refer to DD_VIDEO_ENCODE_FORMAT

Client SDK Instructions

NET_SDK_CH_SNAP_FACE_IMG_LIS T_SEARCH

View face pictures.

typedef struct _net_sdk_ch_snap_face_img_list_sreach_ {

DWORD	dwChannel;//camera/channel of snapshot

DD_TIME_EX	startTime; //time

DD_TIME_EX

DWORD

DWORD the page

endTime; //time

pageIndex;//page

pageSize;//the number of items in

unsigned char	resv[8]; }NET_SDK_CH_SNAP_FACE_IMG_LIST_SEARCH;

Members

Client SDK Instructions

NET_SDK_SEARCH_IMAGE_BY_IMAG E

Search image by image

typedef struct _net_sdk_search_image_by_image_ {

unsigned int pageIndex; //compulsive 1、2、3... unsigned int	pageSize; //compulsive

unsigned int	similarity; //similarity

unsigned int	resultCountLimit; //result limit DD_TIME_EX     startTime;

DD_TIME_EX	endTime; unsigned int

searchType;//NET_SDK_SEARCH_IMAGE_BY_IMAGE_TYPE struct

{

unsigned int itemId; //target id

} sfaceFeatures;//SEARCH_IMAGE_BY_FACE_FEATURES NET_SDK_FACE_IMG_INFO_CH

sfaceImgs;//SEARCH_IMAGE_BY_FACE_IMAGES

struct {

unsigned int groupsId; //GROUP Id }sfaceFeatureGroups

;//SEARCH_IMAGE_BY_FACE_FEATURE_GROUPS

struct {

unsigned int unsigned int

isContainRecognized; //0 or 1 isContainNotRecognized; //0 or 1

unsigned int	groupsId; //GROUP Id }srecognizedFilter

;//SEARCH_IMAGE_BY_RECONGNIZED_FILTER struct

{

unsigned int	imgWidth;// unsigned int	imgHeight;// unsigned int	imgLen;// unsigned char               *imgData;//

}sfaceImgData;//SEARCH_IMAGE_BY_FACE_IMAGE_DATA

}NET_SDK_SEARCH_IMAGE_BY_IMAGE;

Members

Client SDK Instructions

DECODE_FRAME_INFO

Decode YUV frame information

struct decode_frameInfo {

int nWidth; int nHeight;

unsigned int time; unsigned int dwLen; unsigned char *pData; }DECODE_FRAME_INFO;

Members

nWidth

frame width nHeight

frame height time

frame time stamp dwLen

decode frame length pData

decode frame data

Client SDK Instructions

NET_SDK_USB_BACKUP_PROCESS_E X

the process and status of the saving record to USB device

typedef struct _usb_backup_process_ex {

DD_TIME_EX	startTime; // the start time of the record

DD_TIME_EX	endTime; // the end time of the record

unsigned int	dataSize;//MB the size of the record

unsigned char	backupPath[64]; // the usb path of the backup

unsigned char	creator[36];// the creator of the backup task

unsigned int	progress;//0-100, the process of the backup 0-100

unsigned int	backupFileFormat;// unsigned int	status;//

unsigned int	eventType;//

unsigned char	chls[64];// the channel of the backup

unsigned int	chlNum;// the actual number of the channels

}NET_SDK_USB_BACKUP_PROCESS_EX;

Members

startTime

the start time of the record endTime

the end time of the record dataSize

MB the size of the record backupPath

the usb path of the backup creator

the creator of the backup task progress

0-100, the process of the backup 0-100 backupFileFormat

0 is avi,1 is private format status

0 is backuping, 1 is complete eventType

refer to DD_RECORD_TYPE chls

the channels of the backup chlNum

the number of the backup channels

Client SDK Instructions

NET_SDK_IVE_VEHICE_ITEM_INFO

Vehicle number information

typedef struct {

unsigned int begin_flag; unsigned int data_type; unsigned int image_type; unsigned int plateId; unsigned int plateCharCount; char plate[32];

char plateCharConfid[32]; NET_SDK_IVE_RECT_T ptPlateCharRect[32]; unsigned in ptWidth;

unsigned int ptHeight; NET_SDK_IVE_POINT_T ptLeftTop; NET_SDK_IVE_POINT_T ptRightTop; NET_SDK_IVE_POINT_T ptLeftBottom; NET_SDK_IVE_POINT_T ptRightBottom; unsigned short plateWidth; unsigned short plateHeight; unsigned int plateConfidence; unsigned int plateIntensity; unsigned char	plateColor; unsigned char plateStyle;

unsigned char PlateColorRate; unsigned char vehicleColor; unsigned int plateAngleH; unsigned int plateAngleV; unsigned in jpeg_len; unsigned int jpeg_vir_len; char owner[32];

int listType;

unsigned long long beginTime; unsigned long long endTime; unsigned char	iVehicleDirect; unsigned char resrv[11];

unsigned int end_flag; }NET_SDK_IVE_VEHICE_ITEM_INFO;

Members

begin_flag

start identification，0x5a5a5a5a data_type

0：JPG,1:YUV image_type

0:source image，1：vehicle number plateId

ID，just for identification plateCharCount

the number of the characters of the vehicle number plate

the vehicle number, utf8 codec plateCharConfid

the confidence of the vehicle number ptPlateCharRect

the coordinate of the upleft of the vehicle number ptWidth

the width of the vehicle number（for drawing rectangle

to following the vehicle number plate） ptHeight

the height of the vehicle number ptLeftTop

coordinate of the upleft of the plate ptRightTop

coordinate of the upright of the plate ptLeftBottom

coordinate of the downleft of the plate ptRightBottom

coordinate of the downright of the plate plateWidth

width of the plate plateHeight

height of the plate plateConfidence

plate confidence plateIntensity

plate intensity plateColor

the color of the vehicle number plate // 0-blue 1-black 2-yellow 3-white 4-green 5-red 6-gray 7-purple(KISE)

plateStyle plate style

PlateColorRate

the similarity of the plate color vehicleColor

color of the vehicle plateAngleH

horizen angle of the plate plateAngleV

vertical angle of the plate jpeg_len

the lenth of the jpeg image data jpeg_vir_len

the total lenth of the jpeg image data owner

the name of the vehicle's owner listType

list type,0-comparision failed，1-strange，2-white

list，3-black list， beginTime

start time endTime

end time iVehicleDirect

vehicle's direction, 1 unknown 2 near 3 far resrv

preserve end_flag

end identification，0xa5a5a5a5

Client SDK Instructions

NET_SDK_IVE_VEHICE_HEAD_INFO

Vehicle number detection alarm call back information header

typedef struct {

unsigned int begin_flag; unsigned int item_cnt; unsigned int plate_cnt; long long relativeTime; long long absoluteTime;

unsigned int softwareVersion; unsigned int softwareBuildDate; unsigned int resver[2]; unsigned int end_flag;

}NET_SDK_IVE_VEHICE_HEAD_INFO;

Members

begin_flag

start identification，0x5a5a5a5a item_cnt

the number of NET_SDK_IVE_VEHICE_ITEM_INFO plate_cnt

the number of vihicle number has detected relativeTime

the relative time of the detected happened absoluteTime

the absolute time of the detected happened softwareVersion

the version of the software, 0xABCDEFGH,AB：Brand

CD：main version EFGH：sub version Brand 1:OMRON version:V5.00

softwareBuildDate

the build time of the software,0xYYYYMMDD

resver Preserve end_flag

end identification，0xa5a5a5a5

Client SDK Instructions

NET_SDK_DEV_SUPPORT

the functions of the IPC

struct {

unsigned int supportThermometry:1; //support mask and temperature detect

unsigned int supportVfd:1; //support face detect

unsigned int supportVfdMatch:1; //support face match

unsigned int supportThermal:1; //thermal

unsigned int supportPassLine:1; //pass line

unsigned int supportresv:27;	// unsigned int resv[15];	// }NET_SDK_DEV_SUPPORT;

Client SDK Instructions

REG_LOGIN_INFO

the information of the auto register device。

typedef struct _reg_login_info {

unsigned int deviceId;//register id

char m_szUserName[36];//user name char m_szPasswd[36];//password

}REG_LOGIN_INFO;

Client SDK Instructions

SEARCHED_DEVICE_INFO

searched device's information

struct _searched_deviceInfo{

char	series[64]; char	devName[64]; char             deviceType[16];

char	szproductModel[16]; char	szVersion[32];

char	szFactoryName[16]; char             szEthName[16];

unsigned short netport; unsigned short nHttpPort;

unsigned int	ipaddr; unsigned int	gateway; unsigned int	netmask; unsigned int	dns1; unsigned int	dns2;

unsigned short nChannelCount; //NVR's channel count

unsigned int	dwSecondIP; unsigned int	dwSecondMask;

}SEARCHED_DEVICE_INFO;

Members

series[64] series

devName[64] device name

deviceType[16] device type

szproductModel[16] product model

szVersion[32] version

szFactoryName[16] factory name

szEthName[16]

ethnet name netport;

net port nHttpPort;

http port ipaddr

ip address gateway

gate way netmask

net mask dns1

dns1 dns2

dns2 nChannelCount

NVR channel count dwSecondIP

second ip dwSecondMask

second mask

Client SDK Instructions

NET_SDK_IVE_FACE_MATCH_ADD_F ACE_REPLY_T

return struct of the adding face to IPC

typedef struct _net_sdk_ive_face_match_add_face_reply_t {

unsigned int	dwResult;

int	iPersonId;	// person ID.

char	szRes[32]; }NET_SDK_IVE_FACE_MATCH_ADD_FACE_REPLY_T;

Members

dwResult; result。

iPersonId person ID。

szRes reserve。

Client SDK Instructions

NET_SDK_IVE_PASSLINECOUNT_T

pass line information

typedef struct {

unsigned int	enterCarCount; //enter car count

unsigned int	enterPersonCount;//enter person count

unsigned int	enterBikeCount;//enter bike count

unsigned int	leaveCarCount; //leave car count, it'll be 0 if single direction

unsigned int	leavePersonCount;//leave person count, it'll be 0 if single direction

unsigned int	leaveBikeCount;//leave bike count, it'll be 0 if single direction

unsigned int	existCarCount;//exist car count, it'll be 0 if single direction

unsigned int	existPersonCount;//exist person count, it'll be 0 if single direction

int

existBikeCount;//exist bike count, it'll be 0 if single direction

unsigned int	count;	//count NET_SDK_IVE_PASSLINECOUNT_INFO_T passLineInfo[32];

// pass line analyse result information }NET_SDK_IVE_PASSLINECOUNT_T;

Client SDK Instructions

NET_SDK_IVE_PASSLINECOUNT_INF O_T

pass line analyse result information

typedef struct {

unsigned int	eventId; id

unsigned char	status; status,0:none 1:start 2:end 3:procedure

unsigned char	reserve[3]; reserve

unsigned int	targetId; ID

NET_SDK_IVE_LINE_T       line; line rule

NET_SDK_IVE_RECT_T       rect; target rectangle }NET_SDK_IVE_PASSLINECOUNT_INFO_T;

// event

// alarm

//

// target

// pass

//

Client SDK Instructions

NET_SDK_IVE_LINE_T

pass line rule

typedef struct {

unsigned int X1; unsigned int Y1; unsigned int X2; unsigned int Y2;

}NET_SDK_IVE_LINE_T;

// start x coodinate // start y coodinate // end x coodinate // end y coodinate

Client SDK Instructions

NET_SDK_IVE_RECT_T

target rectangle

typedef struct {

unsigned int X1; unsigned int Y1; unsigned int X2; unsigned int Y2;

}NET_SDK_IVE_RECT_T;

// top left x coodinate // top left y coodinate // right down x coodinate // right down y coodinate

Client SDK Instructions

NET_SDK_IVE_AVD_T

struct of abnormal video detection

typedef struct {

unsigned int	count; NET_SDK_IVE_AVD_INFO_T avdInfo[32];

}NET_SDK_IVE_AVD_T

Members

count count

avdInfo

avd information struct array

Client SDK Instructions

NET_SDK_IVE_AVD_INFO_T

struct of abnormal video detection detail

typedef struct {

unsigned int	eventId; unsigned int      status; unsigned int      type;

}NET_SDK_IVE_AVD_INFO_T

Members

eventId eventId

status

0:none 1:start 2:end 3:procedure type

0:none 1:Scene 2:Clarity 3:Color

Client SDK Instructions

NET_DVR_IVE_VFD_RESULT_HEAD_ T

struct of video face detection result head

typedef struct {

LONGLONG	time; LONGLONG	relativeTime;

unsigned int	detectDataLen; unsigned int	softwareVersion; unsigned int	softwareBuildDate; unsigned int	faceCnt;

unsigned int	faceDataLen[40]; }NET_DVR_IVE_VFD_RESULT_HEAD_T

Members

time

current time relativeTime

relative time detectDataLen

detect data length softwareVersion

software version 0xABCDEFGH,AB:manufacture,CD:major version EFGH:minor version

softwareBuildDate software build date

faceCnt

face count, max is 40 faceDataLen

face data length

Client SDK Instructions

NET_DVR_IVE_VFD_RESULT_DATA_I NFO_T

struct of video face detection result data information

typedef struct

{	unsigned inttype; unsigned	int status; unsigned int      width; unsigned int      height;

unsigned int	dataLen; }NET_DVR_IVE_VFD_RESULT_DATA_INFO_T

Members

type

0, JPG; 1, YUV status

0, INVALID; 1, VALID; 2, SAVED width

width height

height dataLen

data Length

Client SDK Instructions

NET_DVR_IVE_VFD_RESULT_FACE_D ATA_INFO_T

face data information

typedef struct {

int	faceId;

unsigned int	ptWidth; unsigned int	ptHeight;

NET_SDK_IVE_POINT_T ptLeftTop; NET_SDK_IVE_POINT_T ptRightTop;

NET_SDK_IVE_POINT_T ptLeftBottom; NET_SDK_IVE_POINT_T ptRightBottom; int	nPose;

int	nConfidence; int	age;

int	sex;

int	dtFrames;

int	featureSize; NET_SDK_IVE_POINT_T stPosFaceImg;

float	feature_score;

short	eye_dist; short	blur;

char	pose_est_score; char	detect_score; char	illumination; char	faceliveness;

char	completeness; char	glasses;

char	wearmask; char	reserved1[1];

float	comprehensive_score; int                       temperature;

int	foreheadX;

int	foreheadY;

NET_SDK_IVE_POINT_T       stHotLeftTop; NET_SDK_IVE_POINT_T       stHotRightBottom; char	cTemperatureMode; char	tempUnitsType;

char	cTemperatureStatus; char                       reserved[5];

NET_DVR_IVE_VFD_RESULT_DATA_INFO_T stFaceImgData; }NET_DVR_IVE_VFD_RESULT_FACE_DATA_INFO_T;

Members

faceId

face ID Number ptWidth

width ptHeight

height ptLeftTop

Left-Top Face Coordinates ptRightTop

Right-Top Face Coordinates ptLeftBottom

Left-Bottom Face Coordinates ptRightBottom

Right-Bottom Face Coordinates nPose

Face Pose nConfidence

Confidence Degree age

age sex

sex dtFrames

dtFrames featureSize

feature size stPosFaceImg

the coodinate of the image left top feature_score

feature score 0-100 eye_dist

distance of the eyes blur

blur pose_est_score

pose est_score 0-100 detect_score

detect score 0-100 illumination

illumination faceliveness

faceliveness0~100 completeness

completeness 0~100 glasses

if wear glasses wearmask

if wear mask reserved1

reserved1 comprehensive_score

comprehensive score [90,100)best， [80,90)better，

[70,80)good，[60,70)normal，[50,60)medium， [0,50)bad。

temperature temperature

foreheadX

forehead X coordinate foreheadY

forehead Y coordinate

stHotLeftTop

hot left top Coordinates stHotRightBottom

hot right top Coordinates cTemperatureMode

0:normal 1:validate tempUnitsType

0:Celsius 1: Fahrenheit cTemperatureStatus

0:normal，1:temperature too low，2:temperature too high

reserved reserved

stFaceImgData face image data

Client SDK Instructions

NET_SDK_IVE_FACE_MATCH_T

struct of face match

typedef struct {

DD_TIME_EX frameTime; unsigned int dwRealFaceID; unsigned int dwGrpID; unsigned int dwLibFaceID; unsigned int dwSimilar; unsigned char byName[32]; unsigned int Channel; unsigned int imgLen;

}NET_SDK_IVE_FACE_MATCH_T

Members

frameTime frameTime

dwRealFaceID snap face id

dwGrpID group id

dwLibFaceID library face id

dwSimilar similarity

byName name

Channel Channel

imgLen

image length

Client SDK Instructions

NET_SDK_AVPSTORE_FACE_ABSTRA CT_INFO

struct of face abstract

typedef struct {

char szName[32]; unsigned int dwBirth; char szNativePlace[16]; char szNote[16]; unsigned char byPicNum;

unsigned char byTypeCredential; unsigned char bySex;

unsigned char byGroupCount; unsigned char byGroupID[4]; union

{

struct {

unsigned int dwStartTime; unsigned int dwReserve[3]; unsigned int dwEndTime; unsigned char byReserve[11]; unsigned char byContentType;

}PeriodV1;

struct {

unsigned unsigned unsigned unsigned unsigned unsigned unsigned

}PeriodV2;

int byWeekOrDate; int dwReserve[3]; short wStartTime; short wEndTime; short wReserve[5]; char byMode;

char byContentType;

struct {

unsigned char unsigned char

}PlaceHolder;

dwReserve[31]; byContentType;

}TimeCycle;

char szCredential[32];

unsigned char byPhoneNum[16]; unsigned char byIDParam[16];

}NET_SDK_AVPSTORE_FACE_ABSTRACT_INFO

Members

szName name

dwBirth

birthday like 19991220 szNativePlace

native place szNote

note byPicNum

number of picture byTypeCredential

credential type bySex

0:male 1:female byGroupCount

group count byGroupID

group id ress

reserve szCredential

credential id byPhoneNum

phone number byIDParam

id

Client SDK Instructions

NET_SDK_TLV_BUFFER_DESC

struct of buffer description

public struct NET_SDK_TLV_BUFFER_DESC {

unsigned char ucID; unsigned char ucVersion; unsigned short usNumber; unsigned int dwSize;

}

Members

ucID id

ucVersion version

usNumber number

dwSize

the source image's size

Client SDK Instructions

NET_SDK_IVE_BASE_INFO

struct of IPC face match base information

typedef {

struct NET_SDK_IVE_BASE_INFO_T

long long	i64SnapTime; unsigned int              iSnapPicId;

int	iSimilarity; int	iPersonId; int	iType;

char	szName[128]; int	iMale;

int	iAge;

char	szIdentifyNum[128]; char	szTel[64];

char	szRes[128];

int	iSnapPicQuality; int	iSnapPicAge;

int	iSnapPicSex;

char	livingBody; char	comparisonRes; char	wearmask;

char	tempUnitsType; int	temperature;

char	keyID[36]; char	szReserve[20];

}NET_SDK_IVE_BASE_INFO

Members

i64SnapTime snap time

iSnapPicId

snap picture id iSimilarity

(0-100) similarity iPersonId

the person's id iType

0:stranger 1:white list 2: black list szName

name iMale

1:male 0:female. iAge

age szIdentifyNum

identify number szTel

telphone number szRes

reserve iSnapPicQuality

snap picture's quality iSnapPicAge

snap picture's age iSnapPicSex

snap picture's sex livingBody

1:living body 0:not comparisonRes

comparision result 1:success 0:failed wearmask

if ware mask 0:not detect 1:not wear 2 wear mask tempUnitsType

temperature unit type 0:celsius 1:Fahrenheit temperature

temperature keyID

keyID szReserve

reserve

Client SDK Instructions

NET_SDK_IVE_PICTURE_INFO

struct of IPC snap picture information

typedef struct NET_SDK_IVE_PICTURE_INFO_T {

int	iWidth; int	iHeight;

int	iPicFormat; int	iPicSize;

}NET_SDK_IVE_PICTURE_INFO

Members

iWidth

picture's width iHeight

picture's height iPicFormat

picture's format iPicSize

picture's size

Client SDK Instructions

NET_SDK_IVE_POINT_T

point

typedef struct {

int X; int Y;

}NET_SDK_IVE_POINT_T

Members

X1

x coodinate Y1

y coodinate

Client SDK Instructions

NET_SDK_NVR_DISKREC_DATE_ITE M

the structure of NVR record days information

typedef struct _net_sdk_nvr_diskrec_date_item {

unsigned int	diskCount; unsigned int	diskIndex;

char	szDiskSizeGB[16]; char          szStartDate[32]; char          szEndDate[32];

}NET_SDK_NVR_DISKREC_DATE_ITEM;

Members

diskCount disk count.

diskIndex disk index.

szDiskSizeGB

the size(GB) of the disk. szStartDate

recording start day. szEndDate

recording end day.

Client SDK Instructions

NET_SDK_DiscoverDevice

discover device automatically on LAN

long NET_SDK_DiscoverDevice( NET_SDK_DEVICE_DISCOVERY_INFO

*pDeviceInfo, long bufNum;

long waitSeconds; );

Parameters

*pDeviceInfo

[in] an array witch is needed to asign values,its size is bufNum ,if descovered device num is more than,the returned size is just bufNUm

bufNum

[in] size of the array waitSeconds

[in] time to discover devices, unit is second,this interface will be returned after waitSeconds

Return Values

Returned value is the num of discovered devices,if no deivce is found or discovering device gets error,the value is 0. Get error info refer to NET_SDK_GetLastError

See Also

NET_SDK_GetDeviceInfo

Client SDK Instructions

FAQ

Q1 How to get alarm means & invoking method?

A1

What is called protection is SDK connects device actively,device starts loading alarm,once alarm happens alarm information is uploaded to SDK as soon as possible.So except that alarm inputs information & device invokes callback function,the interface NET_SDK_SetupAlarmChan also should be invoked to setup connection between SDK and device.

Q2 Alarm configuration has succeed and alarm signal can be received locally,but why can't client receive alarm signal?

A2

Reasons as follows:1)whether network connection is normal 2)when alarm type is protection,whether setup protection correctly.

Q3 Why is it that returned value is failure when calling NET_SDK_SetDVRConfig()?

A3

NET_SDK_EnterDVRConfig() must be called to lock config before calling NET_SDK_SetDVRConfig().

Q4 Why is it that the start time of playback and downloading is different from the setting start time?

A4

Playback & download start from the nearby key frame of the setting start time.

Q5 Why need to pass a group of channel numbers to NET_SDK_PlayBackByTime()?

A5

The passed group channels realize autosynchronous play, at the same time divide channels into groups to play but not play by itself, these can save device-side performance.

Q6 Why the time of record data index,playback and download is different from the time of device-side?

A6

If this problem appears,first check that whether device timezone and PC timezone is the same,and then check that the time of the two machines is the same.

Q7 What to notice when using configurated parameters in NET_SDK_SetDVRConfig()?

A7

Because NET_SDK_SetDVRConfig() needs struct with complete assignment,otherwise setting error comes out. So for fear of this error, popularly invoke NET_SDK_GetDVRConfig() to assign initial values to the struct which is needed modification before invoking NET_SDK_SetDVRConfig().

Q8 Why does control command of NET_SDK_PTZControl have no effect on some certain PTZ?

A8

Device sends control code to PTZ according to decoder type and decoder address.If current decoder unmatchable,matching decoder setup is needed; if device doesnot support the decoder, control command from device has no effect on the PTZ.

Q9 Do audio talkback & forward aim at device or channel?

A9

Aim at device ,not channel.

Q10 Whether the callback function of audio talkback can set be null or not.

A10

Yes,if be null,vioce is still normal but user can't access data.

Q11 How to save record data into files?

A11

Get data through callback function LIVE_DATA_CALLBACK of NET_SDK_SetLiveDataCallBack,and then save the data into files,refer to the example in livedlg.cpp of SDKdemo,see L1170.You can play the saved files by Player.

Q12 How to get play progress?

A12

Get start time and end time by NET_SDK_PlayBackByTime,and then get current playing time by NET_SDK_GetPlayBackOsdTime.Play progress=current playing time/(end time-start time).

Q13 How to do when PlayerDemo gets error code 0XC0150002?

A13

Solution:install Microsoft Visual C++ 2005 SP1 Redistributable Package4,download the module in MSDN.

Q14 Why some function are invalid when palyback?

A14

2X and 4X speed is invalid in SDKDemo and SDK when backward,but other speed is OK.When forward all speed is OK except 1X.Before starting playback one frame by one frame,Pause should be enabled,and then one frame can be played by click Next frame button one time.

Q15 What is wday in DD_TIME struct?

A15

Start time is DD_TIME type in NET_SDK_FindFile,but wday is invalid in DD_TIME.wday can be empty but can not be cleared.You can search by mday or write a function to convert time into wday.

Q16 How to do when play file gets E_PLAYER_BAD_FORMAT_FILE error?

A16

Check the following four qustions:

1the first frame is format frame when save record file,

2all structs in SDK are 4 bytes alignment,

3check interface calling order,

4the file in PlayerSDK should be :

*frame info£¨SDK_FRAME_INFO£©valid data in frame

*frame info£¨SDK_FRAME_INFO£©valid data in frame

* ......

*frame info£¨SDK_FRAME_INFO£©valid data in frame

* ......

* video info frame should be before video frame,audio info frame should be before audio frame too

Client SDK Instructions

NET_SDK_GetLastError

return the last error code of operation

DWORD NET_SDK_GetLastError( );

Return Values

return value is pointer to error code information. error message has two main types,error message of network communication library and error message of soft and hard decoding library,list the first type as follows:

error message of network communication library

type of errors NET_SDK_SUCCESS

error value

0	no error

NET_SDK_PASSWORD_ERROR NET_SDK_NOENOUGH_AUTH NET_SDK_NOINIT NET_SDK_CHANNEL_ERROR NET_SDK_OVER_MAXLINK NET_SDK_LOGIN_REFUSED NET_SDK_VERSION_NOMATCH NET_SDK_NETWORK_FAIL_CONNECT NET_SDK_NETWORK_NOT_CONNECT NET_SDK_NETWORK_SEND_ERROR NET_SDK_NETWORK_RECV_ERROR NET_SDK_NETWORK_RECV_TIMEOUT NET_SDK_NETWORK_ERRORDATA NET_SDK_ORDER_ERROR NET_SDK_OPER_BY_OTHER NET_SDK_OPER_NOPERMIT NET_SDK_COMMAND_TIMEOUT NET_SDK_ERROR_SERIALPORT NET_SDK_ERROR_ALARMPORT NET_SDK_PARAMETER_ERROR NET_SDK_CHAN_EXCEPTION NET_SDK_NODISK NET_SDK_ERROR_DISKNUM NET_SDK_DISK_FULL NET_SDK_DISK_ERROR NET_SDK_NOSUPPORT NET_SDK_BUSY NET_SDK_MODIFY_FAIL

NET_SDK_PASSWORD_FORMAT_ERROR NET_SDK_DISK_FORMATING NET_SDK_DVR_NORESOURCE NET_SDK_DVR_OPRATE_FAILED NET_SDK_OPEN_HOSTSOUND_FAIL NET_SDK_DVR_VOICEOPENED NET_SDK_TIME_INPUTERROR NET_SDK_NOSPECFILE NET_SDK_CREATEFILE_ERROR NET_SDK_FILEOPENFAIL NET_SDK_OPERNOTFINISH NET_SDK_GETPLAYTIMEFAIL NET_SDK_PLAYFAIL

1	user's name or pa 2	no right for this o 3	SDK is not initial 4	error of channel n 5	the client connect 6	SDK login is refu 7	version doesn't m 8	failed to connect t 9	network isn't conn 10	failed to send data 11	failed to receive t 12	timeout when rec 13	send illegal data t 14	the called order er 15	operation method 16	the privileged use 17	DVR command ti 18	error of serial por 19	error of alarm por 20	parameter error 21	server's channel is 22	no hard disk

23	hard disk no. erro 24	server hark disk is 25	server hard disk e 26	server does not su 27	server is busy

28	failed to modify i 29	the password inpu 30	hard disk is forma 31	DVR no resource 32	DVR failed to ope 33	failed open PC vo 34	server voice dialo 35	time input is not c 36	there is no appoin 37	failed to create a f 38	faile to open a file 39	the last operation 40	faile to get the cur 41	failed to play

NET_SDK_FILEFORMAT_ERROR NET_SDK_DIR_ERROR NET_SDK_ALLOC_RESOURCE_ERROR NET_SDK_AUDIO_MODE_ERROR NET_SDK_NOENOUGH_BUF NET_SDK_CREATESOCKET_ERROR NET_SDK_SETSOCKET_ERROR NET_SDK_MAX_NUM NET_SDK_USERNOTEXIST NET_SDK_WRITEFLASHERROR NET_SDK_UPGRADEFAIL NET_SDK_CARDHAVEINIT NET_SDK_PLAYERFAILED NET_SDK_MAX_USERNUM

NET_SDK_GETLOCALIPANDMACFAIL NET_SDK_NOENCODEING NET_SDK_IPMISMATCH NET_SDK_MACMISMATCH NET_SDK_UPGRADELANGMISMATCH NET_SDK_MAX_PLAYERPORT NET_SDK_NOSPACEBACKUP NET_SDK_NODEVICEBACKUP NET_SDK_PICTURE_BITS_ERROR NET_SDK_PICTURE_DIMENSION_ERROR NET_SDK_PICTURE_SIZ_ERROR NET_SDK_LOADPLAYERSDKFAILED NET_SDK_LOADPLAYERSDKPROC_ERROR NET_SDK_LOADDSSDKFAILED NET_SDK_LOADDSSDKPROC_ERROR NET_SDK_DSSDK_ERROR NET_SDK_VOICEMONOPOLIZE NET_SDK_JOINMULTICASTFAILED NET_SDK_CREATEDIR_ERROR NET_SDK_BINDSOCKET_ERROR NET_SDK_SOCKETCLOSE_ERROR NET_SDK_USERID_ISUSING NET_SDK_PROGRAM_EXCEPTION NET_SDK_WRITEFILE_FAILED NET_SDK_FORMAT_READONLY NET_SDK_WITHSAMEUSERNAME NET_SDK_DEVICETYPE_ERROR NET_SDK_LANGUAGE_ERROR NET_SDK_PARAVERSION_ERROR NET_SDK_FILE_SUCCESS NET_SDK_FILE_NOFIND

42	the file input form 43	path error

44	resources allotting 45	display card mode 46	buffer is not enou 47	establish SOCKE 48	set SOCKET erro 49	the max number 50	user doest not exi 51	wirte FLASH erro 52	failed to upgrade 53	the decode card is 54	player failed

55	the max user no. failed to get the IP

56	end or physical ad 57	the channel is not 58	IP address not ma 59	MAC address not 60	the language of up 61	reach to the max p 62	no enough space t 63	no backup device 64	the bits of picture 65	the dimension is o 66	the size of picture 67	failed to load play 68	not find some fun 69	failed to load DsS 70	not find some fun 71	failed to call func 72	voice card is mon 73	failed join to mult 74	failed to create lo 75	failed to bind sock 76	socket is closed 77	the user ID is ope 78	sdk program exce 79	write file failed

80	failed to format re 81	there is same user 82	device type no ma 83	language no matc 84	soft version no m 85	file has been crea 86	file isn't found

NET_SDK_NOMOREFILE	87 NET_SDK_FILE_EXCEPTION	88 NET_SDK_TRY_LATER	89 NET_SDK_DEVICE_OFFLINE	90 NET_SDK_CREATEJPEGSTREAM_FAIL	91 NET_SDK_USER_ERROR_NO_USER	92 NET_SDK_USER_ERROR_USER_OR_PASSWORD_IS_NULL 93 NET_SDK_USER_ERROR_ALREDAY_LOGIN	94 NET_SDK_USER_ERROR_SYSTEM_BUSY	95 NET_SDK_DEVICE_NOT_SUPPROT	96 NET_SDK_USER_ERROR_SYSTEM_NO_READY	97 NET_SDK_CHANNEL_OFFLINE	98 NET_SDK_GETREADYINFO_FAIL	99 NET_SDK_NORESOURCE	100 NET_SDK_DEVICE_QUERYSYSTEMCAPS_FAIL	101 NET_SDK_INBUFFER_TOSMALL	102 NET_SDK_NO_PASSWORD_STRENGTH	103

there is no more f file exception Try again later Device offline

Failed to create JP No such user!

No username or p The user has been The device is bus The device don no Do not complete g Camera is offline It fails to get devi SDK resources is The device fails to The input buffer a The password stre

Remarks

Get error number through NET_SDK_GetErrorMsg

See Also

NET_SDK_GetErrorMsg